| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F475 |
| --- | --- |

| Title: | Enhancement of quantization matrix coding for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junichi Tanaka Yoshitaka Morigami Teruhiko Suzuki Gate City Osaki West Tower, 1-11-1, Osaki, Shinagawa-ku , Tokyo, 141-0032 JAPAN | Tel: Email: | +81 3 5435 3308 Tanaka.Junichi@jp.sony.com Yoshitaka.Morigami@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

AVC based simple method to support quantization matrix in HEVC is proposed in JCTVC-F362. In this contribution, enhancements to improve coding efficiency of quantization matrix are proposed. Since data size of quantization matrix, especially for large CU, is large and efficient encoding method to support quantization matrix is desired. The proposal includes the following methods;

- Transmit matrices by new NAL unit

- Residual coding to support lossless coding of matrix coefficients

- Predictive coding of matrix coefficients

- Adaptive variable length coding (use the same VLC table in LCEC)

The performance of proposed coding method of quantization matrix is also evaluated. The software is implemented in HM 3.0 and the source code is attached to this contribution.

# Introduction

The requirements and the current practices of quantization matrix are summarized in JCTVC-E056. In existing coding standard, e.g. MPEG2 and AVC, quantization matrix is introduced to improve subjective quality. Quantization matrix is widely used in consumer and professional video products, e.g. video camera, Blu-Ray disc, etc. Such well establish method in the existing standard should be supported by HEVC. The requirements and the current practices of quantization matrix are reported in JCTVC-E056.

Data size of quantization matrix, especially for large CU, is large and efficient encoding method to support quantization matrix is desired.

If compression mode is DPCM mode only, as in AVC, it needs to enormous bits. The efficient quantization matrix to the JCTVC-E073[1] was proposed in Geneva meeting. In the contribution, the simplified coding method of the quantization matrix is proposed. Compared with AVC, the proposed method improves coding efficiency about 36.7 % in average BD-rate. Coding efficiency is improved in all sequences and increased encoding time is small.

# Proposal

## Transmission of quantization matrices by new NAL unit

New NAL unit, QMPS(Quantization Matrix Parameter Set), to transmit quantization matrices is proposed. The detail is summarized in JCTVC-E073. It is better to define new NAL unit to transmit the parameters which is changed dynamically in a bitstream. It is possible to transmit more than one QMPS NAL unit in a bitstream. Then the value of quantization matrices in QMPS NAL unit can be updated at each slice. To support this, an identifier, QMPS_ID (qmps_present_flag : 1bit + qmps_id ue()) is introduced in slice header. The detail syntax is summarized in Annex A.

## Coding method of quantization matrix

In AVC, quantization matrix is coded by DPCM. However, data size coded by DPCM will be large, because transform size is larger in HEVC. Data size of matrix coefficients was evaluated by experiments. It was found that data size is about 16 times larger than AVC (4x4 + 8x8), if matrix size is extended to 32x32. Therefore efficient coding method of quantization matrix is required in HEVC.

The diagram of decoder of quantization matrix of the proposed algorithm is shown in Figure 1. The block diagram of encoder is shown in Figure 2. At the first step, each matrix coefficient is predicted. Three prediction modes are proposed in this contribution. Then residue is coded. In each mode, lossless and lossy coding are supported. Quantization of matrix coefficient is introduced. If quantization is not applied, then matrix can be coded in lossless. If quantization is applied, matrix coefficients are coded by lossy coding and data size can be reduced.

Figure 1: Block diagram of decoder

Figure 2: Block diagram of encoder

## Prediction

Two prediction modes, copy mode and DPCM mode, are proposed. The detail of each prediction mode is described in this sub-clause.

### Copy mode

In this mode, current matrix coefficients are predicted from previously coded matrix coefficients. Previously coded matrix, which is referred by identifiers (QMPS_ID, SizeID, and MatrixID), is copied to the predictor. Then residue is calculated using this predictor. When the size of reference matrix is different from that of the current matrix, up-conversion or down-conversion of matrix is applied by linear interpolation. When the reference ID (QMPS_ID / SizeID / MatrixID) is equal to current ID, the default matrix is used for predictor. Example of copy mode is shown in Figure 3.

Figure 3: Example of copy mode

### DPCM mode

In this mode, matrix coefficients are coded by DPCM, like AVC. Matrix coefficients are coded by DPCM in zigzag scan order. Quantization can be applied after DPCM.

### Quantization

Quantization can be applied for the residual error signal of matrix coefficients or a DPCM signal. Matrix coefficients are classified into four domains as shown in Figure 4. Figure 4 is an example for 8x8 matrix. Index is used to refer to each domain and different quantization scale can be used. The same quantization scale in each frequency domain is used for all transform size, e.g. 4x4, 8x8, 16x16 and 32x32, within one QMPS.

Figure 4: Quantization domain index (8x8)

### Residual coding

#### Coding of residual signal

In case of copy mode and axis mode, residual error of prediction can be coded. Residual error is quantized for lossy coding. Block diagram of residual coding is shown in Figure 5. Residual error of prediction is coded either DPCM coding or run length coding. In DPCM, residual error of prediction is coded using DPCM in raster scan order by VLC. In run length coding, residual error of prediction is converted to combination of zero run and level (the value of non-zero value) in zigzag scan order. Each zero run and level is coded by VLC respectively.

Figure 5: Block diagram of residual decoding

### VLC

DPCM coding signal and run length coding signal are coded by one of the 11 VLC tables defined in LCEC. The indicator to select VLC table is explicitly signaled in a bitstream.

# Simulation results

## Evaluation of coding method of quantization matrices

In the last meeting, the benefit to support quantization matrix in HEVC was generally agreed. In this contribution, coding performance of proposed method is evaluated compared with AVC method.

The following tests were conducted.

Test 1: Results of compression performance of matrix itself (matrix only)

Test 2: Its impact on overall HM performance

In case of the test 1, the coding efficiency of matrix only is investigated. The proposed method is compared with AVC method. In case of lossy coding, coding error of matrix coefficients is also evaluated. In the test 2, QMs are changed frame by frame, and are encoded at each frame. Then the impact on the overall performance can be studied. In this case, the ratio of the generated bits for matrix coefficients and the other remaining bits and BD rate/PSNR are measured.

The two matrices used for experiments are created. One is symmetry structure, another is asymmetry structure. The matrices are summarized in the attached Excel file. The same QM are used in both.

Different quantization matrices for each 4x4/8x8/16x16/32x32 size can be used for Intra/Inter of Luma/Chroma respectively.

## Test 1: Comparison of matrix coding only

The test condition is summarized as followings.

Five types of test conditions are used in the experiments. The conditions are summarized in Table 1. “Cond0” are AVC method. “Cond1” corresponds with lossless coding for multiplication representation. “Cond2-4” corresponds with lossy coding for multiplication representation. Region quantization 0-3 are shown in Table1. When larger quantizer is used, coding efficiency is improved but coding error is increased.

|  | mode | Region0 | Region1 | Region2 | Region3 |
| --- | --- | --- | --- | --- | --- |
| Cond0 | AVC | none | none | none | none |
| Cond1 | lossless | 1 | 1 | 1 | 1 |
| Cond2 | lossy | 2 | 2 | 2 | 2 |
| Cond3 | lossy | 2 | 2 | 7 | 7 |
| Cond4 | lossy | 6 | 6 | 8 | 8 |

Table 1: Test conditions (Setting of quantization)

The proposal is evaluated as follows.

- Generated bits for luma matrix coefficients for 4x4/8x8/16x16/33x32, and Intra/Inter

- Coding error of matrix coefficients (max error and average error)

- Compression ratio between proposed method and AVC method (Cond0)

  - (The generated bits by proposed method) / (The generated bits by AVC method) * 100

### Results

The generated bits for matrix coefficients per frame are summarized in Figure 6 and Figure 9.

The max error and the average error are summarized in Figure 7 and Figure 10.

The compression ratios are summarized in Figure 8 and Figure 11.

Figure 6: The generated bits for symmetry matrix

Figure 7: The max and the average error of symmetry matrix

Figure 8: Compression ratio for symmetry matrix

Figure 9: The generated bits for asymmetry matrix

Figure 10: The max and the average error for asymmetry matrix

Figure 11: Compression ratio for asymmetry matrix

Figure 6 and Figure 9 show that the generated bits are decreased with larger quantizer. Especially, as shown in Figure 9, the generated bits of intra/ibter 16x16 for cond 4 is much smaller than others because residue is not encoded.

Figure 7 and Figure 10 show coding error is not large even in case of cond 4, which supports highest coding efficiency. The figures show the max error is 7 and the average error is 3.

Figure 8 and Figure 11 show that compression ratio is from 59.2 % to 6.7 %. (The definition of compression ratio is

(The generated bits by proposed method) / (The generated bits by AVC method) * 100

It can be observed that high coding efficiency is achieved with small coding error from those figures.

## Test 2: Impact on overall performance

The impact to support quantization matrix on the overall performance is investigated. The test condition is summarized in Table 2.

| software | HM3.0 |
| --- | --- |
| frame number | 1sec |
| condition | HE random access only |
| Quantization matrix transmit period | each frame |
| matrix | symmetry structure |
| Transmission | for each, frame, different matrices are sent. |
| coding method | AVC method and sony method |
| Region quantization | cond2 |

Table 2: Test conditions

In this experiment, different quantization matrices for each 4x4/8x8/16x16/32x32 size are used for Intra/Inter of Luma/Chroma respectively. And the matrices are changed in each frame. This is a kind of worst case to transmit quantization matrix, however, such operational practices exist in the commercial products as reported in JCTVC-E056.

The ratio of the generated bits for matrix coefficients and the other remaining bits is measured. BD rate calculated by using AVC as anchor is also measured.

### Results

The ratio of the generated bits for matrix coefficients and the other remaining bits are shown in Table 3. In case of AVC method, the ratio is 25 % for class A and 550 % for class D. Significant amount of bits are consumed to transmit quantization matrices in case of AVC method. Therefore efficient coding method of matrix coefficient is required.

In case of proposed method, the ratio is 3.7 % for class A and 79.3 % for class D. In average, the proposed method can reduce the ratio from 194.0 % to 27.9 %.

Figure 12 and Figure 13 show the ratio of the generated bits for each sequence. In case of class A, Nebuta, the bits for matrix coefficients is 40 % of other remaining data in AVC method. In case of class D, Basketball pass, the bits for matrix coefficients is 1718 % of other remaining data in AVC method. In case of proposed method, the ratio for Nebuta is 5.7 % and the ratio for Basketball pass is about 260.2 %. The coding efficiency is significantly improved by proposed method.

|  | AVC Method | Sony Proposal Method |
| --- | --- | --- |
| ClassA | 25.1% | 3.7% |
| ClassB | 59.5% | 8.6% |
| ClassC | 138.5% | 19.9% |
| ClassD | 553.1% | 79.3% |
| Total Average | 194.0% | 27.9% |

Table 3: Impact of the matrix bits on picure bits

Figure 12: Bit ratio (AVC method)

Figure 13: Bit ratio (Proposal)

BD rate of proposed method is calculated compared with AVC method. The results are summarized in Table 4. BD rate is improved 13.6 % in class A and is improved 65.5 % in class D. The average gain is 36.7 %. The impact on the encoding time is small.

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -13.6 | -4.1 | -13.2 |
| Class B | -27.5 | -21.9 | -27.5 |
| Class C | -42.5 | -39.3 | -42.9 |
| Class D | -65.5 | -64.8 | -66.1 |
| Class E |  |  |  |
| All | -36.7 | -31.9 | -36.9 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 100% |  |  |

Table 4: BD rate comparison of proposed method with AVC method

# Conclusion

In this contribution, the followings are proposed.

- Transmit matrices by new NAL unit

- Residual coding to support lossless coding of matrix coefficients

- Predictive coding of matrix coefficients

- Adaptive variable length coding (use the same VLC table in LCEC)

Coding efficiency of AVC method is not sufficient to support large transform, therefore the proposed method should be included in HM to support efficient encoding method of quantization matrix. The software is attached to this contribution.

# Reference

[1] Junichi Tanaka, Yoshitaka Morigami, Teruhiko Suzuki, "Quantization matrix for HEVC", JCTVC-E073, March 2011, Geneva

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).