| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F480 |
| --- | --- |

| Title: | Unified design for motion compensation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chanyul Kim Sunyoung Jeon Kwang Pyo Choi | Email: dionism@samsung.com sunyoung.jeon@samsung.com kp5.choi@samsung.com |  |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

- Abstract

This contribution presents an unification scheme for motion compensation filters. In this proposal, the filter for uni-prediction doesn’t round the prediction to input bit-depth but do the rounding when predictions results are added with residual coefficients, while keeping all the intermediate calculations in 16-bit. According to JCTVC- D321 and JCTVC-E242, bi-prediction filters are already using this scheme. Using this proposal, the filter for bi- and uni-directional prediction can be unified without any coding loss and increasing complexity.

Proposal

Currently, the filters for bi-prediction and uni-prediction are separated in HM3.0. In case of bi-prediction, the rounding offset isn’t signaled but keeps each of the interpolated signals at high precision. The rounding is performed at the last step after both prediction signals are added. The calculations are performed in 16-bit intermediate buffer. In case uni-prediction, each interpolated signals are rounded at the filter stage. So the calculations are performed in input-bit depth. This is illustrated in Figure 1 where the red N and M number represent each bit depth and blue number in parenthesis is the example when input bit depth is equal to eight.

Figure 1. Current MC prediction in HM3.0

In order to unify filters for bi- and uni- prediction, we propose the scheme that the uni-prediction keeps the interpolated signals at high precision. The calculations are performed in 16-bit. The rounding is performed when prediction results are added with residual coefficients. According to this proposal, the uni-prediction filter has high accuracy with 16 bit depth as same with bi-prediction. So finally bi- and uni- prediction can be used with the same filter in high precision. Figure 2 illustrates the proposal.

Figure 2. Proposed unified MC prediction method

Experimental Results

We compare our test results with the HM3.0 anchor which are represented in Table 1 and Table 2. The results show that the unified design leads similar or slightly better results compared with previous non-unified design. The complexity is changeless in both cases.

Table 1 Results on the random access test case

Table 2 Results on the low delay test case

Conclusion

This contribution proposed a minor fix on the uni-prediction. Using this proposal, uni-prediction performs high accuracy calculation using 16 bit-depth same as bi-prediction. So the two directional prediction filters can be unified without any coding loss and increasing complexity. Finally the encoder or decoder need not to use separated prediction filters any more for motion compensation.

Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).