| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F481 |
| --- | --- |

| Title: | Verification result of Sony's Improvement of delta-QP Coding (JCTVC-F422) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-F422, Improvement of delta-QP Coding proposed by Sony.

# Cross Check Procedure

The source code corresponds to JCTVC-F422 was provided by Sony, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The results of CE4 Subtest2 test 2.3.a were used as the reference data.  In contrast, there are two test points defined in JCTVC-F422.  One of them is referred as “To be tested -1” which is the combination of tools 2.1&2.2 while the other one is referred as “To be tested -2” which is the combination of tools 2.1&2.2&2.3.

For the “To be tested -1” results, the tested data were generated by modifying the provided source code as follows:

#define SNY_CE4_SUBTEST2                     1

#define SNY_CE4_SUBTEST2_MEDIAN_DISABLE   1

#define SONY_DQP_CABAC_BIN                   0

#define SONY_QP_CBP                           1

Meanwhile, for the “To be tested -2” results, the tested data were generated by modifying the provided source code as follows:

#define SNY_CE4_SUBTEST2                     1

#define SNY_CE4_SUBTEST2_MEDIAN_DISABLE   1

#define SONY_DQP_CABAC_BIN                   1

#define SONY_QP_CBP                           1

The test condition defined in JCTVC-E704 subtest2 was used for the crosscheck.

# Results

## Simulation Result of Anchor + 2.1&2.2 [To be tested -1]

Cross check results for “To be tested -1” are summarized in Table 1, where proposed “To be tested -1” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | All Intra HE |  |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 0.0 | -0.1 | -0.1 | -1.9% | 0.0 | 0.0 | 0.0 | 1.2% |
| Class B | -0.2 | -0.1 | -0.2 | -4.7% | -0.1 | -0.1 | -0.1 | -2.2% |
| Class C | -0.3 | -0.3 | -0.3 | -8.4% | -0.2 | -0.2 | -0.2 | -6.3% |
| Class D | -0.3 | -0.3 | -0.3 | -8.6% | -0.3 | -0.3 | -0.3 | -6.9% |
| Class E | -0.3 | -0.4 | -0.3 | -6.1% | -0.2 | -0.2 | -0.2 | -4.1% |
| All | -0.2 | -0.2 | -0.2 | -5.9% | -0.1 | -0.1 | -0.1 | -3.6% |
| Enc Time[%] | 102% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  |
|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.2 | -0.1 | -0.6 | -3.5% | -0.1 | 0.0 | -0.1 | -2.0% |
| Class B | -0.2 | -0.3 | -0.2 | -6.6% | -0.2 | -0.2 | -0.2 | -5.2% |
| Class C | -0.3 | -0.2 | -0.3 | -8.8% | -0.3 | -0.3 | -0.2 | -8.3% |
| Class D | -0.2 | -0.4 | -0.1 | -7.8% | -0.3 | -0.2 | -0.3 | -7.6% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.2 | -0.3 | -0.3 | -6.7% | -0.2 | -0.2 | -0.2 | -5.8% |
| Enc Time[%] | 100% |  |  |  | 102% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.3 | -0.3 | -0.4 | -7.2% | -0.3 | -0.3 | -0.5 | -6.4% |
| Class C | -0.4 | -0.2 | -0.2 | -9.1% | -0.5 | -0.2 | -0.5 | -9.1% |
| Class D | -0.3 | -0.1 | -0.4 | -7.7% | -0.4 | -0.1 | -0.6 | -8.0% |
| Class E | -0.4 | 0.0 | -0.7 | -9.3% | -0.5 | -0.8 | -0.5 | -9.2% |
| All | -0.4 | -0.2 | -0.4 | -8.2% | -0.4 | -0.3 | -0.5 | -8.0% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 1: BD-rate results for proposed 2.1&2.2 vs. test 2.3.a anchor

## Simulation Result of Anchor + 2.1&2.2&2.3 [To be tested -2]

Cross check results for “To be tested -2” are summarized in Table 2, where proposed “To be tested -2” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | All Intra HE |  |  |  |
| --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.1 | -0.1 | -2.6% |
| Class B | -0.2 | -0.2 | -0.2 | -5.0% |
| Class C | -0.3 | -0.3 | -0.3 | -8.9% |
| Class D | -0.3 | -0.4 | -0.3 | -9.5% |
| Class E | -0.4 | -0.4 | -0.3 | -6.9% |
| All | -0.2 | -0.3 | -0.2 | -6.5% |
| Enc Time[%] | 101% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Random Access HE |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.1 | -0.4 | -0.2 | -4.7% |
| Class B | -0.3 | -0.2 | -0.2 | -7.4% |
| Class C | -0.4 | -0.3 | -0.4 | -10.1% |
| Class D | -0.3 | -0.2 | -0.3 | -9.3% |
| Class E |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -7.9% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 99% |  |  |  |
|  | Low delay B HE |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |
| Class B | -0.4 | -0.5 | -0.7 | -8.2% |
| Class C | -0.5 | -0.3 | -0.4 | -10.5% |
| Class D | -0.4 | -0.2 | -0.1 | -9.6% |
| Class E | -0.6 | 0.0 | -0.8 | -11.4% |
| All | -0.5 | -0.3 | -0.5 | -9.7% |
| Enc Time[%] | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  |

Table 2: BD-rate results for proposed 2.1&2.2&2.3 vs. test 2.3.a anchor

The BD-rate results obtained in our simulation match the results provided by Sony.  Accordingly, bitrate and PSNR results provided by Sony were verified.  On the other hand, because the results on encoder/decoder time were not provided by Sony, only our simulation results on encoder/decoder time are presented in the above tables.

# Conclusion

It has been verified that the bitrate and PSNR results provided by Sony on JCTVC-F422 are accurate.