| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F482 |
| --- | --- |

| Title: | Verification result of Qualcomm and NEC's Temporal QP Memory Compression (JCTVC-F499) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-F499, Temporal QP Memory Compression proposed by Qualcomm and NEC.

# Cross Check Procedure

The source code corresponds to JCTVC-F499 was provided by Qualcomm, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The results of CE4 Subtest2 test 2.3.a were used as the reference data.  In contrast, there are three levels of tested data were generated.  One of them is “D3_TD2” which controls QP with 8x8 granularity while stores temporal QP with 16x16 granularity.  Another one is “D3_TD1” which controls QP with 8x8 granularity while stores temporal QP with 32x32 granularity.  The last one is “D3_TD0” which controls QP with 8x8 granularity while stores temporal QP with 64x64 granularity.

For the “D3_TD2” results, the tested data were generated by modifying the provided source code as follows:

#define QP_BUFFERCOMPRESS                 1

#define QP_DECIMATION_FACTOR             4

#define QP_AVERAGE                         1

For the “D3_TD1” results, the tested data were generated by modifying the provided source code as follows:

#define QP_BUFFERCOMPRESS                 1

#define QP_DECIMATION_FACTOR             8

#define QP_AVERAGE                         1

For the “D3_TD0” results, the tested data were generated by modifying the provided source code as follows:

#define QP_BUFFERCOMPRESS                 1

#define QP_DECIMATION_FACTOR             16

#define QP_AVERAGE                         1

The test condition defined in JCTVC-E704 subtest2 was used for the crosscheck.

# Results

## Simulation Result of “D3_TD2” (temporal QP compression of 2x2)

Cross check results for “D3_TD2” are summarized in Table 1, where proposed “D3_TD2” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.4 | -0.6 | -0.4 | -14.2% | -0.4 | -0.4 | -0.5 | -14.2% |
| Class B | -0.5 | -0.5 | -0.5 | -15.9% | -0.5 | -0.6 | -0.6 | -15.4% |
| Class C | -0.7 | -0.7 | -0.6 | -17.8% | -0.7 | -0.8 | -0.6 | -18.2% |
| Class D | -0.4 | -0.6 | -0.5 | -16.3% | -0.6 | -0.7 | -0.6 | -17.2% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.5 | -0.6 | -0.5 | -16.0% | -0.5 | -0.6 | -0.6 | -16.2% |
| Enc Time[%] | 100% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.7 | -16.8% | -0.7 | -0.7 | -0.8 | -16.0% |
| Class C | -0.8 | -0.8 | -0.7 | -18.0% | -0.9 | -0.9 | -0.9 | -18.6% |
| Class D | -0.7 | -0.9 | -0.4 | -17.2% | -0.9 | -0.9 | -1.2 | -18.1% |
| Class E | -1.1 | 0.2 | -0.1 | -21.4% | -1.3 | -1.5 | -1.3 | -22.4% |
| All | -0.8 | -0.6 | -0.5 | -18.1% | -0.9 | -1.0 | -1.0 | -18.4% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 101% |  |  |  |

Table 1: BD-rate results for proposed D3_TD2 vs. test 2.3.a anchor

## Simulation Result of “D3_TD1” (temporal QP compression of 4x4)

Cross check results for “D3_TD1” are summarized in Table 2, where proposed “D3_TD1” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.4 | -0.5 | -0.3 | -11.9% | -0.2 | -0.2 | -0.2 | -10.9% |
| Class B | -0.4 | -0.5 | -0.5 | -13.0% | -0.4 | -0.5 | -0.5 | -12.3% |
| Class C | -0.5 | -0.4 | -0.5 | -14.1% | -0.5 | -0.6 | -0.5 | -13.8% |
| Class D | -0.4 | -0.4 | -0.3 | -12.5% | -0.5 | -0.5 | -0.4 | -12.0% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.4 | -12.9% | -0.4 | -0.4 | -0.4 | -12.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 100% |  |  |  | 101% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.6 | -0.9 | -13.7% | -0.5 | -0.5 | -0.7 | -12.3% |
| Class C | -0.6 | -0.6 | -0.6 | -13.7% | -0.7 | -0.5 | -0.7 | -12.9% |
| Class D | -0.5 | -0.9 | -0.1 | -12.8% | -0.6 | -0.7 | -0.7 | -12.1% |
| Class E | -1.1 | 0.0 | -0.2 | -18.4% | -1.2 | -1.0 | -1.3 | -18.5% |
| All | -0.7 | -0.6 | -0.5 | -14.4% | -0.7 | -0.6 | -0.8 | -13.6% |
| Enc Time[%] | 101% |  |  |  | 101% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

Table 2: BD-rate results for proposed D3_TD1 vs. test 2.3.a anchor

## Simulation Result of “D3_TD0” (temporal QP compression of 8x8)

Cross check results for “D3_TD0” are summarized in Table 3, where proposed “D3_TD0” is compared with CE4 subtest2 anchor (test 2.3.a).

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.3 | -0.5 | -0.3 | -7.8% | -0.1 | -0.1 | -0.1 | -5.6% |
| Class B | -0.3 | -0.3 | -0.4 | -9.0% | -0.3 | -0.3 | -0.3 | -7.7% |
| Class C | -0.4 | -0.4 | -0.3 | -10.4% | -0.4 | -0.3 | -0.3 | -9.3% |
| Class D | -0.3 | -0.3 | -0.2 | -8.8% | -0.3 | -0.3 | -0.3 | -7.2% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.3 | -0.4 | -0.3 | -9.0% | -0.3 | -0.3 | -0.3 | -7.4% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 102% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.6 | -8.8% | -0.3 | -0.2 | -0.5 | -6.4% |
| Class C | -0.4 | -0.3 | -0.4 | -9.2% | -0.4 | -0.2 | -0.4 | -6.8% |
| Class D | -0.4 | -0.6 | -0.2 | -9.1% | -0.3 | -0.5 | -0.1 | -6.0% |
| Class E | -0.7 | 0.5 | 0.3 | -14.1% | -0.9 | -1.1 | -0.9 | -13.2% |
| All | -0.5 | -0.3 | -0.3 | -10.0% | -0.4 | -0.5 | -0.4 | -7.7% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 102% |  |  |  | 101% |  |  |  |

Table 3: BD-rate results for proposed D3_TD0 vs. test 2.3.a anchor

The BD-rate results obtained in our simulation match the results provided by Qualcomm.  Accordingly, bitrate and PSNR results provided by Qualcomm were verified.  On the other hand, because some of the results on encoder/decoder time presented in JCTVC-F499 seemed to be measured using different platforms, only our simulation results on encoder/decoder time are presented in the above tables.

# Conclusion

It has been verified that the bitrate and PSNR results provided by Qualcomm/NEC on JCTVC-F499 are accurate.