| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F483 |
| --- | --- |

| Title: | Planar intra prediction improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Tammy Lee | Email: | jianle.chen@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to apply the existing intra smoothing filter to planar reference signal in case of 16x16 or 32x32 prediction units. And when the prediction signal is derived, the right-column is generated by replication of the right-top pixel and the bottom-row is generated by replication of the left-bottom pixel outside the current unit. For intra-only coding structure, the experimental results show -0.1% gain in high efficiency profile and -0.2% gain in low complexity profile with both modifications.

- Introduction

The planar mode is used for both luma and chroma intra prediction in HM3.0. As shown in Figure 1, the prediction is generated with the average of two linear interpolations (horizontal and vertical). And the right-column and the bottom-row are generated by replication process from the right-top pixel and the left-bottom pixel inside the current unit. Currently, the planar mode is coded as if DC mode and so MDIS is turned off on planar mode because DC mode is not affected by reference signal smoothing.

+

>> 1

T

L

T

T

L

L

Figure 1: Planar intra prediction method in HM3.0

contribution targets improves the planar prediction performance.

- Planar intra prediction improvement

## MDIS for planar mode

Mode dependent intra smoothing is one of the main features of HEVC intra prediction. It was proven that smoothing filtering on the reference signal improves performance of certain intra prediction modes especially for large prediction units. In this contribution, we propose to use smoothing filtering on reference signal for planar mode of 16x16 and 32x32 units.

## Shifting of top-right (T) and bottom-left (L) pixels

As shown in Figure 2, this contribution proposes to use the exact top-right pixel (TR) and left bottom (LB) pixel of the current unit as T and L for interpolation. The motivation of shifting T and L outside of current block is to retain the extrapolation feature of planar mode which can improve the prediction performance.

+

TR

LB

TR

LB

TR

LB

>> 1

Figure 2: Proposed planar intra prediction method

.2 Proposed planar intra prediction method

- Experimental result

By enabling smoothing filter on planar mode for 16x16 and 32x32 in MDIS, the result shows -0.06% and -0.10% gain for high efficiency and low complexity profile respectively in intra-only coding structure.

Table 1: MDIS on Planar 16x16 and 32x32

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.08 | -0.18 | -0.10 | -0.09 | -0.08 | -0.01 |
| Class B | -0.09 | -0.13 | -0.18 | -0.08 | -0.07 | -0.05 |
| Class C | -0.03 | -0.03 | 0.00 | -0.03 | 0.00 | -0.05 |
| Class D | -0.01 | -0.04 | -0.04 | -0.03 | 0.00 | 0.00 |
| Class E | -0.06 | -0.15 | -0.05 | -0.06 | 0.01 | -0.03 |
| Overall | -0.06 | -0.10 | -0.08 | -0.06 | -0.03 | -0.03 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

By shifting reference pixels for generation the right-column and the bottom-row out of the current unit, the result shows -0.08% and -0.10% gain for high efficiency and low complexity profile respectively in intra-only coding structure.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.06 | 0.17 | -0.08 | -0.06 | 0.00 | -0.15 |
| Class B | -0.09 | 0.08 | 0.00 | -0.09 | -0.12 | -0.17 |
| Class C | -0.08 | 0.01 | 0.01 | -0.11 | -0.15 | -0.22 |
| Class D | -0.08 | -0.05 | -0.02 | -0.11 | -0.14 | -0.13 |
| Class E | -0.11 | 0.09 | 0.18 | -0.12 | -0.44 | -0.42 |
| Overall | -0.08 | 0.06 | 0.01 | -0.10 | -0.15 | -0.21 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Proposed the left-top and the right-bottom pixels

With both modification described in the above, the result shows -0.13% and -0.15% gain for high efficiency and low complexity profile respectively in intra-only coding structure. Since smoothing reference signal of intra prediction always generates the left-top and the right-bottom pixels, there is no complexity increment by this proposed modification.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.14 | 0.02 | -0.16 | -0.15 | -0.07 | -0.15 |
| Class B | -0.17 | -0.05 | -0.10 | -0.17 | -0.17 | -0.22 |
| Class C | -0.11 | -0.04 | -0.05 | -0.14 | -0.17 | -0.25 |
| Class D | -0.09 | -0.07 | -0.07 | -0.13 | -0.17 | -0.16 |
| Class E | -0.17 | -0.03 | 0.09 | -0.17 | -0.48 | -0.44 |
| Overall | -0.13 | -0.03 | -0.07 | -0.15 | -0.20 | -0.23 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 3: Proposed reference pixel positions and MDIS setting

- Specification

## MDIS setting

In Table 8-6 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes, setting for planar mode should be changed as yellow marked ones.

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 0 |
| 9 | 1 | 1 | 1 | 1 | 0 |
| 10 | 0 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 1 | 1 | 0 |
| 12 | 0 | 0 | 1 | 1 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 |
| 14 | 0 | 0 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 1 | 0 |
| 17 | 0 | 0 | 1 | 1 | 0 |
| 18-33 | 0 | 0 | 0 | 1 | 0 |
| 34 | 0 | 0 | 1 | 1 | 0 |
| 35 | n/a | n/a | n/a | n/a | n/a |

## Shifting of top-right (T) and bottom-left (L) pixels

In 8.3.3.1.7, (8-45) should be changed as follows:

predSamples[ x, y ] = (
		( nS – 1 – x ) * p[ -1, y ] + ( x + 1 ) * p[ nS ,-1 ] + 
		( nS – 1 – y ) * p[ x ,-1 ] + ( y + 1 ) * p[ -1, nS ] + nS ) >> ( k + 1 )	(8-1)
with x, y = 0..nS-1 where k = log2( nS )

- Conclusion

This contribution propose to enable smoothing filter for large units’ planar prediction and shifting the reference pixels’ position to derive the right-column and the bottom-row. -0.1% and -0.2% gain in AI configuration is achieved with no complexity increase.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).