| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F485 |
| --- | --- |

| Title: | Cross check report for Panasonic's improving method for adaptive DCT/DST (F225) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, JAPAN | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp sugitou.y-gy@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

The contribution reports cross-verification results on an improving method for adaptive DST/DCT from Panasonic’s JCTVC-F225 [1] contribution. HM3 adopted additional transform 4x4 DST for intra prediction residue. The transform is decided to use according to intra prediction mode. This proposal is to improve the decision process by considering reference pixel availability. The coding experiments with two difference conditions of three proposal conditions were conducted. These coding results were identical to provided results by Panasonic and the implementation was confirmed.

## Test conditions

Two experiment results of three proposal conditions are reported in this document. Test 1 was conducted under common condition according to JCTVC-E700 [2]. Test 2 was not conducted. Test 3 was conducted with additional setting, ConstrainedIntraPred：1. The all verifications were performed on Linux 64 bit OS with gcc-4.1.2 with the same computer systems.

## Test results

The test 1 results are shown in Table 1.

Table 1. Coding results of Test1 (Common condition)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.00 | -0.05 | 0.03 | 0.00 | -0.03 | 0.00 |
| Class B | -0.01 | 0.03 | 0.02 | 0.00 | -0.01 | -0.01 |
| Class C | -0.02 | 0.00 | 0.02 | -0.01 | 0.00 | -0.03 |
| Class D | -0.02 | -0.03 | -0.04 | -0.02 | -0.02 | -0.01 |
| Class E | 0.02 | 0.04 | 0.01 | -0.02 | -0.03 | -0.02 |
| Overall | -0.01 | 0.00 | 0.01 | -0.01 | -0.02 | -0.01 |
| Enc Time[%] | 99.3% |  |  | 99.4% |  |  |
| Dec Time[%] | 100.2% |  |  | 100.1% |  |  |

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.03 | -0.35 | -0.07 | 0.01 | -0.07 | -0.03 |
| Class B | -0.03 | 0.02 | 0.05 | 0.01 | -0.04 | 0.01 |
| Class C | -0.03 | 0.22 | -0.17 | -0.01 | -0.08 | 0.00 |
| Class D | 0.00 | 0.07 | -0.24 | 0.01 | -0.41 | -0.14 |
| Class E |  |  |  |  |  |  |
| Overall | -0.02 | -0.01 | -0.10 | 0.00 | -0.14 | -0.04 |
| Enc Time[%] | 100.0% |  |  | 99.9% |  |  |
| Dec Time[%] | 98.8% |  |  | 98.9% |  |  |

The test 3 results are shown in Table2

Table 2. Coding results of Test 3 (Common condition with ConstrainedIntraPred：1)

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.10 | -0.04 | -0.09 | -0.03 | -0.06 | -0.15 |
| Class B | -0.04 | 0.01 | -0.04 | -0.03 | -0.05 | -0.04 |
| Class C | -0.10 | 0.02 | -0.17 | -0.08 | -0.20 | -0.13 |
| Class D | -0.04 | -0.06 | -0.06 | -0.05 | -0.33 | -0.12 |
| Class E |  |  |  |  |  |  |
| Overall | -0.07 | -0.02 | -0.09 | -0.05 | -0.15 | -0.11 |
| Enc Time[%] | 99.9% |  |  | 99.9% |  |  |
| Dec Time[%] | 100.8% |  |  | 100.2% |  |  |

# Conclusion

All test results are identical to provided results from Panasonic. The coding gain is slightly better than HM3.0 and the process time is not almost increasing.

# References

[1] Y. Shibahara and T. Nishi, “An improving method for adaptive DST/DCT,” Doc. JCTVC-F225, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Torino, IT, July, 2011

[2] “Common test conditions and software reference configurations.” Doc. JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Geneva, CH, March 2011