| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F486 |
| --- | --- |

| Title: | Cross-check of Orange Lab’s Wavefront Parallel Processing for HEVC Encoding and Decoding JCTVC-F274 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Anastasia Henkel Fraunhofer HHI Einsteinufer 37 D-10587 Berlin | Tel: Email: | anastasia.henkel@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports test results of JCTVC-F274, Wavefront Parallel Processing for HEVC Encoding and Decoding proposed by Orange Labs. One method for R/D performance using Wavefront Parallel Processing (WPP) is tested. The proposed code has been checked, compiled and the results reported by Orange Labs can be confirmed.

# Tested Proposal

The tested proposal has been integrated into HEVC HM 3.0 Error: Reference source not found. One method for R/D performance using WPP is tested.

Compared to HM3.0, the following was changed:

- Initialization the CABAC probabilities of the first LCU of each line with the probabilities obtained after the second LCU of the upper line.

- Inset of a single probability buffer.

- Inset of the following macros :
#define OL_USE_WPP			1
#define OL_NUM_SUBSTREAMS	2
#define OL_SHIFT			1

- Usage of 2 Threads (substreams).

- Necessity to provide substream pointers in the bitstream.

# Simulation Environment and Results

## Software

The coding conditions used for the simulations are specified in Error: Reference source not found. The HM3.0 default settings, both for configuration file and compile-time, are used in this test as anchor.

## Hardware

Several types of computers have been used to run the simulations all running a 64 bit Ubuntu 10.10 Linux operating system. Due to this heterogeneous structure, the encoding- and decoding times are not reliable.

## Results

The coding efficiency results in this section are given in terms of BD-rate. The values are averaged over all sequences from one class. BD-PSNR, PSNR values and bit rates for every test point can be found in the attached Excel spreadsheet JCTVC-F486.xls.

Table 1 	Results

# References

- Gordon Clare, Félix Henry, Stéphane Pateaux, “Wavefront Parallel Processing for HEVC Encoding and Decoding”, JCTVC-F274, Torino, 2011.

- JCT-VC, “HEVC Test Model Software 3.0”,
https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-3.0.

- Frank Bossen, “Common test conditions and software reference configurations”,
JCTVC-E700, Geneva, March, 2011.