| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F487 |
| --- | --- |

| Title: | Study of memory bandwidth of HEVC Motion Compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Teruhiko Suzuki 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032 Japan | Tel: Email: | +81-3-5435-3308 teruhikos@jp.sony.com |
| Source: | Sony Corp. |  |  |

_____________________________

# Abstract

This contribution investigated HW complexity of HM, especially memory bandwidth aspects of MC(Motion Compensation). Since MC is one of the most critical tools in HEVC in terms of memory bandwidth, the complexity of MC is reported. The memory bandwidth of HM is increased about 320 % compared relative to AVC, while logic size is about 130 % relative to AVC. The situation is critical for large screen video, e.g. 4K video. The baseband data rate of 4K@60p (3840x2160@60p) is 8 times higher than HDTV (1920x1080@60i). The memory bandwidth of HEVC should be designed feasible for memories (and semi-conductor processing) around 2014. Considering this target, the current HEVC MC is too complex, especially from memory bandwidth aspects.

Moreover, in case of AVC, number of MVs is constrained by Annex A. 4x4 MC block size is not selected so much by this restriction. However, there is no such constraint in HEVC yet. Even if such restriction is applied, HEVC memory bandwidth is still too high.

This contribution proposes

- To reconfirm the importance to reduce the memory bandwidth

- To start to consider restrictions as defined in Annex A in AVC

# Complexity analysis of HM

The HW complexity of HM was evaluated and summarized in this contribution. Table 1 summarized the worst case of MC memory access bandwidth. The value is required the number of read out pixels to create 1 sub pel position.

|  | The number of read pixels / 1 sub pel |  |  |  |
| --- | --- | --- | --- | --- |
|  | Luma |  | Chroma |  |
|  | AVC 6tap | HEVC 8 tap | AVC 2 tap | HEVC 4 tap |
| No restrictions | 10.1 (bi-pred) | 15.1 (bi-pred) | 4.5 | 12.5 |
| Restrictions of Bi pred PU<8x8 | 5.3 (bi-pred) | 7.6 (uni-pred) | 3.1 | 6.2 |

Table 1: The worst case memory access bandwidth

The value is converted to the comparison with AVC and summarized in Table 2. In case of AVC, the number of MVs and bi-pred are restricted for level 3.2 and higher. This table compared HEVC MC with AVC MC with Annex A restrictions.

|  | The number of read pixels / 1 sub pel |  |  |  |
| --- | --- | --- | --- | --- |
|  | Luma |  | Chroma |  |
|  | AVC 6tap | HEVC 8 tap | AVC 2 tap | HEVC 4 tap |
| No restrictions | 1.9 | 2.8 (x AVC) | 1.5 | 4.0 (x AVC) |
| Restrictions of Bi pred PU<8x8 | 1 | 1.4 | 1 | 2.0 |

Table 2: The worst case memory access bandwidth compared with AVC

The current HM memory bandwidth is 2.8 times larger than AVC for luma and 4.0 times higher than chroma. The impact to adopt 4 tap filter for chroma interpolation is large. Table 3 summarized the comparison of HW complexity of HEVC and AVC. While logic size is 1.3 times higher than that of AVC, the memory bandwidth is 3.2 times higher than that of AVC.

|  | HEVC MC complexity / AVC complexity |
| --- | --- |
| Logic size | 1.3 (x AVC) |
| Memory bandwidth | 3.2 (x AVC) |

Table 3: The estimated complexity relative to AVC high profile

Table 4 summarized the baseband data rate of HDTV and higher resolution video. In case of 4K@60p, baseband data rate is 8 times larger than HDTV (1080@60i). The required memory size and data transfer rate are significantly increased.

|  | 1280x720@60p | 1920x1080@60i | 1920x1080@60p | 3840x2160@30p | 3840x2160@60p |
| --- | --- | --- | --- | --- | --- |
| Baseband dara rate | 0.89 | 1 | 2 | 4 | 8 |

Table 4: The baseband data rate compared with 1920x1080@60i

Semi-conductor process technology is improved and it is expected to use more area in LSI. It means we can use However, it is not expected to increase memory transfer bandwidth. Even in case of 4K video, it is expected to introduce into B&I domain (semi-professional). Therefore cost affordable solution is required, and further reduction of MC, especially memory bandwidth, is crucial.

# Conclusion

This contribution proposes

- To reconfirm the importance to reduce the memory bandwidth

- To start to consider restrictions as defined in Annex A in AVC (just applying the same restriction is not sufficient)

- Target complexity is based on the available technology around 2014

# Patent rights declaration(s)

Sony Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).