| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F488 |
| --- | --- |

| Title: | Requirements for Scalable extension of HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Edouard FRANCOIS Sebastien LASSERRE Fabrice LE LEANNEC Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | edouard.francois@crf.canon.fr sebastien.lasserre@canon.crf fabrice.leleannec@crf.canon.fr |  |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution aims at proposing requirements in the context of the ad-hoc group on requirements for the scalable extension of HEVC, established in March 2011. The intent is to bring inputs related to mobile and video capture devices. In particular, requirements related to complexity are discussed.

# Introduction

One major focus of the HEVC standard under development is high definition video. Ultra-high definition already used in professional capture devices, is also targeted for consumer capture devices [1] and mobile devices [2]. The design of new devices for ultra-high definition content is highly challenging both in terms of memory access bandwidth and power consumption, and consequently of cost. Scalability is an opportunity to better master these complexity issues.

Capture devices embedded video coders are generally constrained by the following areas of operation. Note that unlike many applications, these requirements mainly relate to the encoder side, embedded in the capture device.

- Coding efficiency – This is of course a major feature that would justify the migration toward new scalable coding technologies in the HEVC extension.

- Limited computational complexity – Real-time encoding with limited low-cost processing capacities is required for capture devices, especially for the consumer market.

- Limited memory footprint and access bandwidth – This requirement becomes more and more critical as the video resolutions and frame rates increase. It has a huge impact on the device cost and power consumption.

- Scalability – This is also an important feature in the context of heterogeneous access networks and display devices (from mobile to home TV). When considering capture devices, this feature is also of high interest, especially for ultra-high resolution, as a solution to better control the complexity.

- Region of Interest (ROI) scalability – This feature is relevant when considering very high resolution content. Such a feature gives possibility for devices with lower display resolution to access parts of the picture without heavy computations. User generated video sharing, Interactive video, Video surveillance [3], interactive TVs [4], are some application examples that can benefit from such a feature.

- Low delay – This feature is generally required for interactive applications, such as on line editing or conversional applications. It is also a strong request from the professional market for easy content editing and manipulation.

# Scalability as an opportunity to better control the codec complexity

Among these requirements, reduced computational complexity, limited memory footprint and access bandwidth are of major importance, since they directly drive the device cost and power consumption.

In a multi-layer pyramidal approach such as used in spatial scalability of SVC, both the base layer and enhancement layers have to be encoded and overall complexity can in principle be higher than the single layer case. Nevertheless scalable coding technology can be exploited to reduce the complexity compared to single layer coding. A good tradeoff complexity/coding performance can be reached if efficient inter-layer coding tools are designed. For instance, if only the base layer makes use of temporal prediction and coding loop, while the enhancement layer is only built based on base-layer up-sampling and inter-layer prediction, the overall complexity can be significantly reduced. This can be beneficial both on the computational complexity side and on the memory access bandwidth side.

Regarding memory access bandwidth, one critical issue is the fast random access to reference frames blocks involved by temporal motion compensation. Random access is actually much less efficient than the sequential access involved in pure intra or inter-layer prediction. Limiting significantly the random access for the highest spatial resolution can then significantly reduce the fast access memory needs.

This possibility should be considered when designing the scalable extension of HEVC, when defining test conditions for spatial scalability, and when specifying the profiles applying to low memory capture devices applications.

# Proposed scalability requirements

The following requirements are proposed. They are based on the current status of the ad-hoc group discussions on requirements for the scalable extension of HEVC. All the dimensions are not considered and we only focus on features considered as critical for the capture devices application field.

- Spatial scalability. A limited set of ratio (1.5 and 2) as currently recommended is sufficient. In addition, the number of spatial layers should be limited to 3.

- Region of Interest (ROI) scalability. Spatial random access should be supported. The granularity should be configurable, from the LCU size to larger tile sizes.

- Profiles flexibility. As recommended by the ad-hoc group, flexibility to have different profiles across various layers should be supported. For instance, the base layer coding process involves conventional temporal/spatial prediction loops, while only inter-layer intra prediction is allowed in the enhancement layer.

- Memory complexity. Scalable extension of HEVC should support coding modes enabling to involve temporal coding / decoding loop only at the spatial base layer, so that memory requirements are mostly driven in spatial scalability by the base layer, leading to a memory usage closer to the base layer one than to the high layer one.

- Computational complexity. Similarly scalable extension of HEVC should support coding modes enabling to involve temporal coding / decoding loop only at the spatial base layer, so that computational complexity is mostly driven in spatial scalability by the base layer, leading to a complexity closer to the base layer one than to the high layer one.

- Temporal. This feature is needed but dyadic decomposition should be sufficient.

# Conclusion

A good design of the scalable extension of HEVC could be beneficial for the video capture and mobile devices market. When dealing with ultra-high definition content, usage scenarios based on low power consumption and low memory bandwidth devices must be considered to standardize the HEVC scalable extension coding technology.

# References

[1] - http://www.engadget.com/2010/09/02/canons-4k-multipurpose-camera-concept-weakens-knees-shoots-vid/

[2] - Ukur, Ridge, ‘Requirements for high-performance video standards’, ISO/IEC JTC1/SC29/WG11MPEG2009/M16207, February 2009, Lausanne, Switzerland.

[3] - http://www.dallmeier-electronic.com/en/cctv-ip-video-surveillance/cameras/models/panomera.html

[4] - Mavlankar, Girod, ‘Spatial-Random-Access-Enabled Video Coding for Interactive Virtual pan/tilt/zoom functionality’, in TCSVT, vol.21, N.5, May 2011 - http://www.youtube.com/watch?v=Ko9jcIjBXnk

# Patent rights declaration(s)

Canon Research Centre France may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Even if the standard specifies the decoder, encoding complexity must be considered when designing the technology for mobile and capture devices applications.