| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: | Document: JCTVC-F491 WG11 Number: M20921 |
| --- | --- |

| Title: | High level syntax for scalability support in HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Thomas Rusert, Rickard Sjöberg, Zhuangfei Wu, Per Fröjdh Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 767602660 thomas.rusert@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes high-level syntax changes to HEVC aiming at making inclusion of future scalability extensions straightforward without changing the NAL unit headers. It is proposed to introduce a fixed-length NAL unit header that includes seq_parameter_set_id, such that all VCL and non-VCL NAL units that are associated with a certain scalable layer are assigned the same value of seq_parameter_set_id. Furthermore, it is proposed to introduce syntax for signaling of both dependencies between scalable layers and properties of the respective layers into the SPS. Thus according to the proposed scheme, the seq_parameter_set_id serves as general scalable layer identifier, whereas the associated SPSs indicate both dependencies between layers and respective layer properties. Consequently, it is proposed to move temporal_id (which is carried in the NAL unit header in the current HM design) into the sequence parameter set. The proposal claims these changes make it possible to signal dependencies within scalable video representations in an extensible way. It also claims that new scalability extensions can be defined and used together with old ones.

# Introduction

The AVC standard was initially designed with single-representation video as target input and output. In particular there was no need to support scalable video and multi-view video in the NAL unit header and the Sequence Parameter Set. Later, when the AVC standard was extended to include SVC and MVC, the hierarchical dependencies exhibited in SVC and MVC bitstreams required the introduction of additional information fields in the NAL unit headers, such as decoding dependencies and view_ids. In order to retain compatibility with existing AVC implementations, the basic AVC NAL unit header was not changed. Instead, the extra dependency and view id information was presented by introducing new types of NAL units: the prefix NAL unit and the coded slice extension NAL unit, which will be ignored by legacy AVC decoders. Disadvantages of the design, however, include that it complicates implementation and results in a variable-length NAL unit header that is gateway unfriendly. The design also implies a complicated sub-bitstream extraction process for SVC and MVC since it is necessary to inspect a variety of id labels (temporal_id, view_id etc.) in the NAL unit header in order to decide which NAL units should be discarded.

Another problem that is not solved well in the non-scalable AVC profiles is related to temporal scalability signaling. The AVC SPS has a syntax field level_idc, which specifies the required decoder capability to decode a bitstream. A full representation that contains all temporal layers should have a higher level_idc than a sub-stream that only contains the base temporal layer, and multiple level_idc should be used to describe required decoding capability for each layer. However, there is only one level_idc in the SPS that always indicates the level_idc needed to decode the full representation. The bitstream extraction process specified by AVC does not mandate level_idc be changed accordingly after extraction. Consequently, a decoder supporting a lower level_idc has no way of knowing whether or not it can actually decode a pruned bitstream. Signaling of layer-specific level_idc and other operating point specific syntax has been introduced in different ways in SVC and MVC, such as scalability information SEI in SVC and view scalability information SEI in MVC. The method proposed in this contribution provides a general means for operating point signaling that can be applied for different degrees of scalability.

For HEVC we have a unique opportunity to start over and design a clean and “future-proof” high-level syntax. This will facilitate and simplify the design when scalable (and/or multi-view) representations are included in the scope and standardized on top of HEVC. To this end we propose to:

- Simplify NAL unit header by removing the second NAL unit header byte carried by VCL NAL units, and moving temporal_id into the SPS, and output_flag into the slice header.

- Make sequence_parameter_set_id a fixed-length field and move it to the NAL unit header.

- Remove sequence_parameter_set_id in Sequence Parameter Set and in Picture Parameter Set (since it is indicated in the NAL unit header).

- Extend the syntax of the SPS to indicate layer properties and dependencies between layers.

These proposals will simplify the bitstream extraction process and provide easy “hooks” for scalable codecs to come.

# Cleaner design towards layered representation

## Previous design proposal

In JCT-VC E422, we proposed to include a generic identifier “NID (NAL unit identifier)” in the NAL unit header to serve as “pointer” to a syntax structure (NPS, NAL unit Parameter Set) that specifies properties of the sub-stream labeled by this NID. The properties of the sub-stream can be, i.e., temporal_id for current HEVC WD, view_id for possible HEVC multi-view extension or dependency_id for possible HEVC scalable extension etc. The abstract “NID” pointing mechanism can bundle together all the properties describing layered feature of a sub-stream that previously lie in NAL unit header and they can be stored in a separate packet (NPS). In this way, the NAL unit header can be of fixed length and fixed syntax whereas the actual variation is done in the NPS. The benefits brought by this shift include:

- In principle, no new NAL unit type needs to be introduced for future HEVC based codecs. For instance, there is no need to introduce prefix NAL unit or coded slice extension NAL unit for HEVC scalable extension.

- NAL unit header is fixed length which makes it easy to parse. The packet delivery/discarding method becomes easier for gateways.

Above NID and NPS, a higher layer grouping feature “Representation Parameter Set” (RPS) is also proposed in JCT-VC E422. It can group several types of NAL units which has different NIDs to a meaningful representation that is decodable.

## New design proposal

Based on the philosophy in JCT-VC E422 we now propose an improved and simplified design that keeps the benefits brought by NID/NPS/RPS while requiring minimal changes by reusing existing structures as much as possible.

The main concept lies in taking advantage of Sequence Parameter Set (SPS) where all the functionality provided by NPS and RPS can be included. Originally in AVC and HEVC, SPS is a syntax structure containing syntax elements that apply to zero or more entire coded video sequences. It is determined by the content of a seq_parameter_set_id syntax element found in the Picture Parameter Set (PPS) referred to by pic_parameter_set_id syntax element found in each slice header. Since the SPS already defines syntax elements applied to a certain bitstream, it can naturally be extended with information on bitstream layer properties and can group different layers into a representation.

The proposed concept has two major aspects:

- Definition of a generic “seq_parameter_set_id” (SPS_id) in NAL unit header, such that NAL units with the same SPS_id have common properties. In this way, NAL units with the same SPS_id together define a subset of the bitstream which has the given properties. A set of NAL units with equal SPS_ids is referred to as a sub-stream. This generic SPS_id is conceptually a superset of typical SPS_id defined by AVC. Note that the SPS_id is present in every NAL unit and not only in VCL NAL units. This makes it possible to tie SEI messages to layers and have constant NAL unit header length.

- Extension of the SPS syntax structure to specify properties of sub-streams. Using the extended syntax, a SPS that is associated with an enhancement layer (i.e., a layer that has decoding dependencies with respect to other layers) can be used to signal decoding dependencies as well as sub-stream properties. A SPS that is associated with a base layer (i.e. a layer that does not have any decoding dependencies) is very similar to the SPS design in the current HM. Thus, the SPS has two different “modes”. When associated with an enhancement layer, we refer to the SPS as “eSPS”. Constituting a mode of the SPS, an eSPS is identified through a SPS_id, and can thus be referenced through the SPS_id syntax element in the NAL unit header. An eSPS also contains one or more pointers to the SPS/eSPS it is depending on and provides necessary signaling enhancement s/updates relative to the base SPS/eSPS.

The above concepts are used as follows:

- Use SPS to bundle all the NAL units in the base layer.

- For each type of enhancement, a specific type of eSPS is created to tight together enhancement layer and base layer, and to provide property description of enhancement layer as well as necessary updates to the base layer SPS considering the entire bitstream as a whole entity.

- eSPS performs the link through referring to one or more base SPS_id

- eSPS can have more than one enhancement type, e.g. can be both temporal enhancement and spatial enhancement.

- eSPS can refer to either SPS or another eSPS.

Figure 1: Example of SPS and eSPS (which is a mode of SPS) when two temporal layers are used.

In the example scenario shown in Figure 1, a bitstream that has 2 temporal layers is structurally linked to an SPS and an eSPS. The sequence is IBPBP coded where all the base layer NAL unit headers (I and P slices) have the same SPS_id. The SPS associated with this SPS_id has a low level_idc since it represents a sub-stream that is only half frame rate relative to the full representation. All the enhancement layer NAL unit headers (B slices) share another SPS_id pointing to an eSPS that has temporal updates. The temporal eSPS, as suggested by the name, provides the temporal enhancement layer related information. A typical temporal eSPS does not contain all the fields that exist in SPS, it has a field called base_seq_parameter_set_id which gives a link to base SPS/eSPS from which it can inherit most of the fields. Apart from those inherited fields, a temporal eSPS also contains some fields that describe the layer property as well as some possible updates of the field to the SPS/eSPS it is derived from. In the example, they are temporal_id and level_idc.

When a decoder is only capable of decoding bitstream of level_idc up to i.e. 2.1, it can simply refer to the SPS and decode only the NAL units that are associated to it. The eSPS and its associating NAL units can be discarded safely.

Another type of eSPS could be defined for multiview extensions that indicate view dependencies. It is also possible to use different kinds of eSPS in the same bitstream to indicate multidimensional dependencies, even mixing scalability with multiview.

# Required changes

All required changes to support proposed functionalities are specified in detail in this section.

## NAL unit header change

We propose to put a seq_parameter_set_id syntax element after the first byte of the NAL unit header. The seq_parameter_set_id has fixed length 16 which would be enough to accommodate all different layers. We also propose to remove temporal_id and output_flag to make the NAL unit header length constant. Temporal_id is moved to eSPS and output_flag is suggested to move to slice header. An alternative solution would be to always reserve 1 bit in NAL unit header for output_flag so that its existence is not dependent on nal_unit_type and make seq_parameter_set_id length 15 to keep byte alignment.

The new design makes sure that all NAL unit headers are of the same length. In practice, fields like nal_ref_idc can also be “absorbed” into seq_parameter_set_id but for now we have left the first byte untouched. Note that the value of sps_id can indicate priority and can be used by naïve bitstream extractors, i.e., a lower sps_id is more important.

### Proposed syntax change

| nal_unit( NumBytesInNALunit ) { | Descriptor |
| --- | --- |
| forbidden_zero_bit | f(1) |
| nal_ref_idc | u(2) |
| nal_unit_type | u(5) |
| seq_parameter_set_id | u(16) |
| NumBytesInRBSP = 0 |  |
| nalUnitHeaderBytes = 3 |  |
| if( nal_unit_type = = 1 ¦ ¦ nal_unit_type = = 4 ¦ ¦ nal_unit_type = = 5 ) { |  |
| temporal_id | u(3) |
| output_flag | u(1) |
| reserved_one_4bits | u(4) |
| nalUnitHeaderBytes += 1 |  |
| } |  |
| for( i = nalUnitHeaderBytes; i < NumBytesInNALunit; i++ ) { |  |
| if( i + 2 < NumBytesInNALunit && next_bits( 24 ) = = 0x000003 ) { |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| i += 2 |  |
| emulation_prevention_three_byte /* equal to 0x03 */ | f(8) |
| } else |  |
| rbsp_byte[ NumBytesInRBSP++ ] | b(8) |
| } |  |
| } |  |

seq_parameter_set_id identifies the sequence parameter set that is referred to by the NAL unit.

## Sequence Parameter Set change

Since seq_parameter_set_id is already a syntax element in the NAL unit header, it is not necessary to define it again in sequence parameter set payload. We propose to delete seq_parameter_set_id field in SPS.

The newly designed SPS can contain either a base SPS that describe only the base layer, or an enhancement SPS that performs an update on top of the SPS it is dependent on.

eSPS provides description to every enhancement layer and supports grouping different layers to form a meaningful representation. It can depend on one or more SPS/eSPS and may contain different types of updates relative to the SPS/eSPS it depends on.

eSPS is defined in a flexible manner that allows new updates being added freely. The most frequently updated syntax elements are present in the beginning of eSPS, these include profile_idc and level_idc. The specific types of updates are appended consecutively. Whether those updates shall exist in eSPS are controlled by corresponding specific type of flags. In the current HEVC WD, only temporal scalability is supported, hence we propose to define a temporal_dependency_flag in eSPS. When this flag equals to 1, it means that the current eSPS describes a temporal enhancement layer relative to a temporal base layer (relatively defined). The temporal base layer is identified by syntax element temporal_base_seq_parameter_set_id whereas the property of the temporal enhancement layer is defined by syntax element temporal_id which originated from NAL unit header.

The proposed syntax in the table below does also include examples of how dependencies other than temporal dependencies could be indicated when e.g. multiview coding or spatial scalability are introduced into HEVC. Depending on the profile used, further dependency flags such as multiview_dependency_flag and spatial_scalable_dependency_flag are included in the eSPS in order to indicate respective decoding dependencies. Along with the respective base_seq_parameter_set_ids for the respective dependencies, layer properties such as view_id for multiview coding or dependency_id for spatially scalable coding would be introduced. Since those functionalities are not yet included in the HM, we do not suggest the exact syntax at this time. It should be noted though that a eSPS can indicate dependencies towards more than one other SPS/bSPS, which would reflect that different types of prediction are used in the same scalable layer, such as temporal prediction and multiview prediction.

Scalability extension types other than those mentioned can be likewise accommodated by the proposed SPS concept. In order to extract a certain compressed video representation from a scalable bit stream, an extractor can simply parse an eSPS it wants to extract and collect all the base SPS/eSPS together with their associated NAL units until it reaches the base layer SPS. In this way, the proposed concept greatly facilitates bitstream extraction process. It should also be noted that when a scalable layer is discarded according to its SPS_id, then the respective SPS/eSPS associated with the layer and thus the description of the respective operating point is “automatically” discarded, too. Thus no rewriting of operating point signaling is necessary, as may be required when all scalable layers are described in a single scalability information SEI in SVC or MVC.

### Proposed syntax change

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| base_seq_parameter_set_flag = 1 |  |
| temporal_dependency_flag | u(1) |
| if( temporal_dependency_flag = = 1 ) { |  |
| temporal_base_seq_parameter_set_id | u(16) |
| temporal_id | u(3) |
| base_seq_parameter_set_flag = 0 |  |
| } |  |
| if( profile_idc = = multiview_profile ) { |  |
| multiview_dependency_flag | u(1) |
| if( multiview_dependency_flag = = 1 ) { |  |
| view_base_seq_parameter_set_id | u(16) |
| … |  |
| base_seq_parameter_set_flag = 0 |  |
| } |  |
| } |  |
| if( profile_idc = = spatial_scalability_profile ) { |  |
| spatial_scalable_dependency_flag | u(1) |
| if( spatial_scalable_dependency_flag = = 1 ) { |  |
| spatial_base_seq_parameter_set_id | u(16) |
| … |  |
| base_seq_parameter_set_flag = 0 |  |
| } |  |
| } |  |
| … |  |
| if(base_seq_parameter_set_flag = = 1 ) |  |
| base_seq_parameter_set_rbsp( ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Note that basically the rest of the “old” SPS syntax elements is moved into base_seq_parameter_set_rbsp( ). For simplicity of the presentation, we have deleted those syntax elements from the seq_parameter_set_rbsp table instead of leaving them here stroked out.

base_seq_parameter_set_rbsp( ) is basically the same as the seq_parameter_set_rbsp in the current HM, only that profile_idc and level_idc are in the new seq_parameter_set_rbsp, and seq_parameter_set_id is removed since it is sent in the NAL unit header.

| base_seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| adaptive_loop_filter_enabled_flag | u(1) |
| pcm_loop_filter_disable_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Note that changes in this syntax table are shown relative to seq_parameter_set_rbsp( )

## Picture Parameter Set Change

Since seq_parameter_set_id is already a syntax element in NAL unit header, it is not necessary to associate it in picture parameter set payload. We propose to delete seq_parameter_set_id field in PPS.

### Proposed syntax change

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Slice Header Change

We introduce output_flag that has been removed from NAL unit header.

### Proposed syntax change

| slice_header( ) { | Descriptor |
| --- | --- |
| lightweight_slice_flag | u(1) |
| if( !lightweight_slice_flag ) { |  |
| output_flag | u(1) |
| slice_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| … |  |

# Patent rights declaration(s)

Ericsson may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).