| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F495 |
| --- | --- |

| Title: | Higher granularity of quantization parameter scaling and adaptive delta QP signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianle Chen Tammy Lee | Email: | jianle.chen@samsung.com tammy.lee@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

The current HEVC Test Model employs a quantization parameter (QP) scaling scheme which is the same to the AVC standard. In AVC, the quantization step size increases by approximately 12.25% with each increment of QP which lead to average around19% and up to 44.2% bit increase. For the purpose of rate control, this 12.25% increment may be too coarse for some certain applications. This contribution proposes to use a higher granularity of the QP scaling in HEVC test model. In addition to higher granularity, this contribution also proposes to specify dQP_scale value at the slice or picture level to support the tradeoff between dQP bits overhead and fine rate/quality control.

- Problem Statement

The current HEVC Test Model under Consideration employs a quantization parameter (QP) scaling scheme that is same to the AVC standard. The quantization step size applied to each transform coefficient is determined approximately as follows:

Qstep≈2QP−46, with QP = 0…51                            (1)

According this equation, the quantization step size increases by approximately 12.25% with each increment of QP and the quantization step size doubles when QP is incremented by 6. The scaling solution may be too coarse for fine rate control and not flexible enough for various application scenarios as mentioned in JCTVC-C135. We conducted the experiments with HM3.0 software with common test condition by using QP (22, 27, 32 and 37) and QP-1 (21, 26, 31 and 32). As shown in Table 1, the average bit rate increasing is around 19% and up to 44.2% for all inter configuration when reduce QP by 1. The results shows that the current quantization step size difference between two adjacent QP values is too large to allocate accurate bits in CU/slice/ picture level which can be critical for rate control.

Table 1: Bit rate increase of QP-1 vs QP in HM3.0

| Test configuration | Bit rate increase (Qp-1 vs QP) |  |
| --- | --- | --- |
|  | Average | Max |
| HE_AI | 12.9% | 29.0% |
| LC_AI | 12.8% | 26.1% |
| HE_RA | 18.3% | 36.5% |
| LC_RA | 18.2% | 38.5% |
| HE_LD | 19.5% | 44.2% |
| LC_LD | 19.2% | 44.2% |

There have been prior efforts to address the coarse quantization step size increment of HEVC. In JCTVC-A114, the quantization step size was proposed to double every 16 increments of QP. In JCTVC-C135, a flexible quantization parameter scaling was proposed to satisfy the difference application scenario.

- Proposed quantization scheme

## Fine granularity of quantization step size scaling

In this contribution, we propose to use a higher granularity of QP scaling to support more accurate bit allocation and perceptual quality control. We propose to the straightforward extension of current scheme by doubling the QP scaling accuracy.

Qstep≈2QP−812, with QP = 0…103                         (2)

With the new equation, same quantization step size derived when the QP in proposed scheme is twice of that in original one. This simple extension results in very easy adaptation of other modules in which the operation depends on QP value, such as deblocking filter, CABAC context model initialization. The quantization step size increases by approximately 5.95% with each increment of QP. Table 2 shows the average bit rate increase between QP (44, 54, 64 and 74) and QP-1 (43, 53, 63 and 73) is less than 10% which is appropriate for most application.

Table 2: Bit rate increase of QP-1 vs QP of proposed method

| Test configuration | Bit rate increase (Qp-1 vs QP) |  |
| --- | --- | --- |
|  | Average | Max |
| HE_AI | 6.4% | 14.0% |
| LC_AI | 6.4% | 12.6% |
| HE_RA | 8.8% | 17.3% |
| LC_RA | 8.7% | 18.0% |
| HE_LD | 9.2% | 20.3% |
| LC_LD | 9.1% | 20.1% |

## Adaptive delta QP signaling

The fine granularity of QP scaling improves rate control accuracy for certain applications. However, it introduces disadvantage since more overhead bits are necessary to code delta QP to achieve same level of quantization step change.

We propose to insert one syntax element dQP_scale into PPS or Slice level to support the flexibility of tradeoff between better rate/quality allocation and dQP bits overhead. In the proposed method, QP of current block is calculated with equation (3), where dQP_scale means QP value change scale when dQP is 1.

QP_curr = QP_pred + dQP * dQP_Scale                            (3)

In the proposed method, the slice level QP scaling granularity is always kept as the highest level. With change of dQP_Scale, the CU level QP update speed is adaptively selected by different dQP_Scale according to application requirement. When dQP_Scale is equal to 2, CU level Quantization step adaptation level is same to that of current HEVC quantization scheme, meanwhile, still keeping the higher quantization step size granularity at slice level. Figure 1 shows how to match the current HEVC quantization scheme with the proposed syntax. If the slice QP is even number and dQP_Scale is 2, then current HEVC quantization scheme can be emulated with the same dQP values.

dQP_Scale = 2

dQP_Scale = 1

QP01234567891011…QStep0.6250.68750.81250.87511.1251.251.3751.6251.7522.25…

QP0246810121416182022…QStep0.6250.68750.81250.87511.1251.251.3751.6251.7522.25…

Figure 1: AVC style mapping with dQP_Scale=2

dQP_Scale = 3

dQP_Scale = 2

dQP_Scale = 1

QP prediction

Figure 2: dQP=1 with different dQP_Scale

Error: Reference source not found shows QP value change from QP prediction according to dQP_Scale when dQP is equal to 1. QP prediction can be any value and the range of QP is within 0 to 103. This solution is integrated in CE4 subtest 2 software to interpret how dQP_Scale will control the dQP overhead bits. The outputted quantization step of CE4 subtest2 software is interpreted into QP with floating with higher mapping granularity which value is exactly double of the anchor. Then the floating QP is rounding to integer QP with different accuracy according to dQP_Scale level. In this experiment, same level Qstep update with different rounding accuracy is tested. When dQP_scale is set to 2, almost same results to CE4 subtest2 are attained. Table 3 and Table 4 show the result compared to CE4 subtest 2 anchor when dQP_scale is 1 and 3. To keep same level Qstep update with higher rounding accuracy, dQP_Scale = 1 has BD-rate 1.4% loss (almost same PSNR with 1.4% bit rate increasing). In comparison of dQP bits, about 38% dQP bits are increased comparing to anchor. To keep same level Qstep update with coarse rounding accuracy, dQP_Scale = 3 has BD-rate -0.8% gain (almost same PSNR with 0.8% bit rate decreasing). In comparison of dQP bits, about 19% dQP bits are reduced comparing to anchor.

Table 3: dQP_Scale=1 in comparing to CE4 subtest 2 anchor

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 1.0 | 1.1 | 0.9 | 37.0% | 0.9 | 1.0 | 1.0 | 37.1% |
| Class B | 1.1 | 1.2 | 1.2 | 40.0% | 1.1 | 1.1 | 1.2 | 37.4% |
| Class C | 1.2 | 1.3 | 1.3 | 38.3% | 1.3 | 1.3 | 1.3 | 38.3% |
| Class D | 1.1 | 1.1 | 1.2 | 36.8% | 1.2 | 1.2 | 1.3 | 38.4% |
| Class E | 1.7 | 1.9 | 1.7 | 39.2% | 1.6 | 1.8 | 1.9 | 38.8% |
| All | 1.2 | 1.3 | 1.2 | 38.3% | 1.2 | 1.3 | 1.3 | 37.9% |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | 1.0 | 1.2 | 1.1 | 35.5% | 1.2 | 1.6 | 1.6 | 37.6% |
| Class B | 1.2 | 1.4 | 1.3 | 37.8% | 1.3 | 1.4 | 1.5 | 38.4% |
| Class C | 1.2 | 1.5 | 1.3 | 36.1% | 1.5 | 1.5 | 1.6 | 38.5% |
| Class D | 1.1 | 1.2 | 0.9 | 36.1% | 1.4 | 1.5 | 1.7 | 38.5% |
| Class E |  |  |  |  |  |  |  |  |
| All | 1.2 | 1.3 | 1.2 | 36.5% | 1.3 | 1.5 | 1.6 | 38.3% |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | 1.4 | 1.7 | 1.2 | 36.6% | 1.6 | 1.7 | 1.5 | 38.9% |
| Class C | 1.5 | 1.4 | 1.5 | 35.4% | 1.8 | 1.6 | 1.9 | 38.5% |
| Class D | 1.4 | 2.0 | 1.5 | 36.0% | 1.8 | 2.5 | 1.9 | 38.6% |
| Class E | 1.5 | 2.2 | 2.6 | 35.2% | 2.1 | 1.3 | 2.2 | 37.9% |
| All | 1.5 | 1.8 | 1.6 | 35.9% | 1.8 | 1.8 | 1.8 | 38.5% |

Table 4: dQP_Scale=3 in CE4 CE4 subtest 2 anchor

|  | Intra |  |  |  | Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.7 | -0.6 | -0.7 | -20.3% | -0.6 | -0.4 | -0.4 | -17.8% |
| Class B | -0.7 | -0.7 | -0.7 | -21.0% | -0.6 | -0.5 | -0.4 | -17.0% |
| Class C | -0.8 | -0.7 | -0.8 | -21.0% | -0.7 | -0.6 | -0.6 | -17.9% |
| Class D | -0.6 | -0.6 | -0.6 | -19.8% | -0.6 | -0.6 | -0.6 | -17.8% |
| Class E | -0.9 | -0.8 | -0.8 | -18.4% | -0.8 | -0.5 | -0.3 | -15.8% |
| All | -0.7 | -0.7 | -0.7 | -20.2% | -0.7 | -0.5 | -0.5 | -17.3% |
|  | Random access |  |  |  | Random access LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.8 | -0.3 | -1.1 | -19.5% | -0.6 | -0.5 | -0.3 | -18.1% |
| Class B | -0.8 | -0.6 | -0.7 | -20.3% | -0.7 | -0.7 | -0.6 | -17.9% |
| Class C | -0.8 | -0.6 | -0.8 | -19.8% | -0.8 | -0.9 | -0.8 | -18.3% |
| Class D | -0.6 | -0.7 | -0.6 | -19.6% | -0.7 | -0.9 | -0.6 | -18.5% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.7 | -0.6 | -0.8 | -19.8% | -0.7 | -0.7 | -0.6 | -18.2% |
|  | Low delay |  |  |  | Low delay LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.9 | -0.9 | -1.1 | -20.0% | -0.9 | -1.0 | -1.2 | -18.4% |
| Class C | -1.0 | -1.0 | -1.1 | -19.5% | -0.9 | -1.2 | -0.9 | -18.6% |
| Class D | -0.8 | -0.3 | -1.0 | -19.5% | -0.9 | -0.6 | -1.2 | -18.8% |
| Class E | -1.2 | -2.2 | -1.2 | -19.2% | -1.1 | -1.3 | -1.8 | -18.2% |
| All | -1.0 | -1.0 | -1.1 | -19.6% | -0.9 | -1.0 | -1.2 | -18.5% |

## Inverse quantization process

Equation (4) is the basic operation of proposed method for inverse quantization process, where the constant 12 shall be 6 in current HEVC inverse quantization. Another change of proposed method is to extend the size of QPScale array from 6 to 12.

Yij = (Zij * DeqScale( QP % 12) << (QP/12)) >> qbits                (4)

Where, DeqScale is {40, 42, 45, 48, 51, 54, 57, 60, 64, 68, 72, 76}.

One advantage of proposed method is that forward and inverse quantizer operation is fixed without considering the adaptive value of dQP_Scale.

- Specification

- In 7.3.3 Slice header syntax,

| if( !lightweight_slice_flag ) { |  |
| --- | --- |
| slice_qp_delta | se(v) |
| scale_qp_delta | se(v) |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |

In 7.4.3 Slice header semantics,

scale_qp_delta specifies how much cu_qp_delta is scaled up. scale_qp_delta shall be inferred to be equal to 2 when it is not present. The decoded value of scale_qp_delta shall be the greater than or equal to 1.

In 7.4.9 Transform coefficient semantics,

QPY=(((QPY,PREV+cu_qp_delta*scale_qp_delta +104+ 2*QpBdOffsetY)%(104+QpBdOffsetY))-QpBdOffsetY  (7-1)

In 8.5.3,

dij = ( cij * LevelScale(nS)x(nS)[ qP%12 ][ i ][ j ] ) << ( qP/12 + trafoPrecisionExt ), with i, j = 0..nS-1	(8-2)

- Conclusion

With only one syntax element dQP_scale into PPS or Slice level, user can have flexibility to choose tradeoff between finer rate/quality control and dQP bits overhead reduction. All operations related to quantization are fixed without regarding to dQP_Scale adaption. It is proposed to be adopted into HM with minimum change for the better flexibility.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).