| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F496 |
| --- | --- |

| Title: | 2x2 block-based reference pictures memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | KANG Lijuan MA Yanzhuo Xidian University, Xi’an, Shaanxi, China LIN Sixin Huawei Tech. Ltd., Shenzhen, China | Tel: Email: | +86-29-88202527-828 kangkanglj@126.com yzma@mail.xidian.edu.cn +86-755-28786613 linsixin@huawei.com |
| Source: | Xidian University, Huawei Tech. Ltd. |  |  |

_____________________________

# Abstract

In this contribution, two adaptive reference pictures memory compression scheme from 10-bit/pix to 8-bit/pix based on 2x2 blocks are proposed. This contribution is to solve the redundant memory bandwidth accessing especially when the align/burst parameters are low. In the proposed scheme, both adaptive scaling and offset compensation are used on top of the first scheme. Results show that compared with the fixed rounding method, better coding efficiency is obtained, and compared with the existing 4x4 block based methods, little performance loss is introduced. In terms of memory access bandwidth, the proposed method decreased the memory access bandwidth by 15~17% compared with the 4x4 block based methods when burst parameter is smaller than 64, and about 9% when burst parameter is 128. For large burst parameters the memory bandwidth can be kept the same as the 4x4 block based compression methods by the non-normative trick that merges four 2x2 blocks to one 4x4 block as the basic unit for store and access.

# Introduction

The existing methods to solve the reference pictures memory compression are mostly based on 4x4 pixel units. This is probably due to the flatness of pixel values of such small blocks, and the capability of more flag bits can be used to design a flexible compression scheme. Compared with no-compressed case, JCTVC-E133[1] and JCTVC-E432[2] have achieved remarkable memory bandwidth decrease with the cost of encoding efficiency slightly dropped off, as Table 4-7 shows.

However, compared with the fixed rounding method[3], these methods increase the memory access bandwidth remarkably (about 40%) when the align/burst parameters are low. In Figure 1, the horizontal pink line shows the access bandwidth that is necessarily 4-7 needed (without cache), is the benchmark.

How come there’s so much redundant bandwidth access? Compared with bit-shift of each single pixel, the 4x4 unit method enlarged the access granularity of the memory access while align/burst parameters are with small values. This causes redundant access of memory, and increases the memory access bandwidth unnecessarily, as the example shown in Figure 2, where an 8x8 block area (shown in orange) is to be accessed. The really accessed areas are presented in pink, green and blue for single-pixel-based, 2x2-block-based and 4x4-block-based schemes, respectively.

This contribution proposes to compress the pixel values by 2x2 unit size, which is effective based on the following 2 reasons:

- The flatness of pixel values is much better than 4x4 ones, therefore, the encode efficiency decrease can be controlled just at the same level as 4x4 unit.

- The 2x2 unit brings smaller granularity and can be reorganized as larger ones, such as 4x4, 8x8, etc. Smaller granularity can bring less redundant access bandwidth with low align/burst parameters. Reorganized as larger units can obtain efficiency with high align/burst parameters or with caches.

Figure 1. The memory access bandwidth of multi methods for Class C

Figure 2. Example of accessing an 8x8 block in reference picture which is not aligned either with 4x4 blocks or with 2x2 blocks while Align/Burst parameters are 8bit/8bit and 64bit/256bit

# 2x2 adaptive scaling for reference pictures memory compression

## Compression Algorithm

When the bit-depth is 2-bit increased and the reference pictures are to be compressed from 10-bit/pix to 8-bit/pix, the scaling algorithm is implemented as follows.

- Obtain the minimum (min) and maximum (max) pixel values in the 2x2 block,

- Calculate the range of pixels’ values in the 2x2 block, as R=max-min,

- Decide the scaling value S so that R becomes less than (64 << S) and no less than (0, if S=0; 64 << (S-1), if S=1，2,  3), where the max value of S is 3, which means R>=(64<<2).

- If S is less than 3, then the position of min in the block (minIdx) is stored, and P(=(pixel_value[i]-min)>>S, where i!=minIdx) will be represented by 6 bits. At the same time, min, S are stored, too. Else if S equals to 3, then the P(=(pixel_value[i])>>3) and offset(=（sum(pixel_value[i]&0x7)+2)>>3) are stored.

In this algorithm, the pixel value will be stored with no more than 2-bit lost in most cases(0-bit lost when S=0, 1-bit lost when S=1, 2-bit lost when S=2，3-bit lost but with offset when s=3). For concrete process ,please refer to Table 1 .

Table 1  The process for reference pictures compression

| The value of R | Precision Losses | Number of Bits to be used |
| --- | --- | --- |
| R<64 | Lossless | 2(flag)+10(min)+2(minIdx)+6*3(differences)=32 |
| R>=64 && R<128 | 1 bit * 3 | 2(flag)+10(min)+2(minIdx)+6*3(differences)=32 |
| R>=128&& R<256 | 2 bits * 3 | 2(flag)+10(min)+2(minIdx)+6*3(differences)=32 |
| R>=256 | 3 bits * 4 - compensated by two bits of offset | 2(flag)+ 7*4(Pixel rightshift 3 bits)+2(offset)=32 |

### Compression Format

When the pixels in each 2x2 unit are compressed, they are stored in memory with the following fixed format detailed in Table 2. Here, each unit is presented by 32 bits instead of 40 bits.

- Table 2 Data format while compressed

| adaptive_scaling_block( ) { | Descriptor | Notes |
| --- | --- | --- |
| If(s==3) { |  | s:[0..3] |
| S | u(2) | Flag:11 |
| for(i=0;i<4;i++) { |  |  |
| P[i] | u(7) | Pixel[i]>>3 |
| offset | u(2) | offset |
| } |  | //32bit |
| } |  |  |
| else{ |  |  |
| S | u(2) | Flag:00,01,10; |
| min | u(10) | min |
| minIdx | u(2) | minIdx |
| For(i=0;i<4&&i!=minIdx;i++){ |  |  |
| P[i] | u(6) | (Pixel[i]-min+1<<(S-1) )>>S |
| } |  |  |
| } |  | //32bit |
| } |  |  |

### Decompression Algorithm

The de-scaling process is as follows.

- Read the first 2 bits, got the flag of scaling.

- If the flag denotes the case of S=3, the reconstruct pixel value D[i] (=(P[i]<<3)+(offset<<1),) is obtained.

- Else if the flag denotes the case of S<3, the reconstruct pixel value D[i](=(P[i]<<S)+min) is calculated.

In fact, this compression is lossless when the R=max-min<64. While 64<=R<128 and 128<=R<256, the minimum pixel value is lossless. And the other case is with a loss of 3bits but with 2bits offset. The R<128 cases for the 2x2 blocks are in the more majority than that of the 4x4 blocks. The concrete de-scaling process is shown in Table 3.

- Table 3  Decompress (de-scaling) process

| Steps | Notes |
| --- | --- |
| Read the first 2 bits, obtain the flag, then get the corresponding value of s | u(2) s:[0..3] |
| If（s==3） | Flag:11, s=3 |
| { |  |
| For(i=0;i<4;i++) D[i]=p[i]<<3+（offset<<1）; |  |
| } |  |
| Else | Flag:00 , 01 or 10 , s=0 ,1 or 2 |
| { Get the values in turn: 10-bit min,2-bit minIdx, 3*6-bit p[i] (0<=i<4 and i!=minIdx); | D[minIdx]=min |
| For(i=0;i<4&& i!=minIdx;i++) D[i]=(p[i]<<s)+min; |  |
| } |  |

## Reorganization of reference frame memory

The 2x2 unit brings smaller granularity and can be reorganized as larger ones, such as 4x4, 8x8, etc. Smaller granularity can bring less redundant access bandwidth with low align/burst parameters. Reorganized as larger units can obtain efficiency with high align/burst parameters or with caches.

By analyzing the size of unit to be compressed better for bandwidth’s reduction, we make use of the 2x2 unit’s flexibility and reorganize them when align/burst parameter is high.

# 3 Experimental results

The experiments are all implemented based on the common test conditions and software reference configurations specified in JCTVC-E700[4]. The test source code is modified based on which the one provided by the AHG of memory compression.

Table 4~7 gives the encoding efficiency, bandwidth, and enc/dec time of the proposed compression scheme, , E432 ,E133 and the fixed Rounding method being compared with the RC2 anchor.

The data in Table 4 shows that on average, 9.1%~29.26% with RA, 14.32%~46.65% with LD of bandwidth is decreased by the proposed method.

From the Table 4, 5 and 6, we can see that on average, 9%~17% with RA, 9%~15% with LD of bandwidth is decreased by the proposed method when burst parameter is not higher than 128  bits, while the coding efficiency is close to the 4x4 methods, 0.09%/1.16% BD-Rate vs.(0.07%/0.77%) or (0.12%/1.62%) .

From the Table 4 and 7, we can see that on average, 5%~35% with RA and LD of bandwidth is decreased by the proposed method when burst parameter is higher than 8bits, while the coding efficiency is better than the FixedRounding method, 0.09%/1.16% BD-rate: vs. 0.68%/4.01%..

Table 4-1. Proposed scheme vs. HM3.0 RC2 anchor: IBDI on, no memory compression, RA HE

|  | HE Random access |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A | 0.03 | -0.03 | 0.00 | -3.19% | -10.43% | -10.91% | -11.28% | -20.31% | -32.41% | -25.28% | -28.01% |
| Class B | 0.09 | 0.00 | 0.05 | -6.45% | -9.45% | -11.37% | -10.90% | -11.19% | -23.92% | -12.29% | -13.95% |
| Class C | 0.11 | 0.19 | 0.15 | -14.40% | -24.40% | -30.49% | -30.14% | -29.05% | -44.84% | -34.43% | -38.48% |
| Class D | 0.13 | 0.03 | -0.07 | -12.98% | -24.69% | -31.62% | -32.20% | -29.71% | -46.95% | -35.76% | -40.04% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.09 | 0.05 | 0.03 | -9.10% | -16.80% | -20.57% | -20.59% | -22.04% | -36.40% | -26.16% | -29.26% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 83% |  |  |  |  |  |  |  |  |  |  |

Table 4-2. Proposed scheme vs. HM3.0 RC2 anchor: IBDI on, no memory compression, LD HE

|  | HE Low delay |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.63 | 0.19 | 0.28 | -14.73% | -23.99% | -29.58% | -28.45% | -26.48% | -41.48% | -29.65% | -33.06% |
| Class C | 0.35 | 0.31 | 0.40 | -13.60% | -24.87% | -31.83% | -31.98% | -31.10% | -48.08% | -38.27% | -42.75% |
| Class D | 0.20 | 0.26 | 0.84 | -11.65% | -25.51% | -33.17% | -34.46% | -31.57% | -50.22% | -39.31% | -44.53% |
| Class E | 4.39 | 0.93 | 0.44 | -18.08% | -22.40% | -26.47% | -26.75% | -33.38% | -48.62% | -30.79% | -33.39% |
| All | 1.16 | 0.38 | 0.48 | -14.30% | -24.29% | -30.46% | -30.51% | -30.20% | -46.65% | -34.43% | -38.41% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 87% |  |  |  |  |  |  |  |  |  |  |

Table 5-1. E432 vs. HM3.0 RC2 anchor: IBDI on, 4x4-block-based memory compression, RA HE

|  | HE Random access |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A | 0.03 | 0.01 | -0.01 | 31.04% | 19.81% | 5.89% | 1.14% | -20.33% | -32.43% | -25.28% | -28.01% |
| Class B | 0.07 | -0.01 | 0.06 | 7.86% | 4.05% | 0.56% | -3.99% | -11.17% | -23.91% | -12.27% | -13.93% |
| Class C | 0.06 | 0.04 | 0.07 | 1.82% | -10.70% | -19.82% | -23.58% | -29.05% | -44.83% | -34.44% | -38.48% |
| Class D | 0.11 | 0.00 | 0.01 | 7.99% | -7.25% | -18.49% | -23.27% | -29.71% | -46.97% | -35.72% | -40.00% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.07 | 0.01 | 0.03 | 11.86% | 1.58% | -7.55% | -11.99% | -22.03% | -36.41% | -26.14% | -29.24% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 85% |  |  |  |  |  |  |  |  |  |  |

Table 5-2. E432 vs. HM3.0 RC2 anchor: IBDI on, 4x4-block-based memory compression, LD HE

|  | HE Low delay |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.37 | 0.10 | 0.22 | 0.44% | -11.00% | -18.91% | -22.03% | -26.49% | -41.51% | -29.72% | -33.14% |
| Class C | 0.22 | 0.24 | 0.03 | 4.52% | -9.82% | -20.50% | -24.74% | -31.16% | -48.13% | -38.37% | -42.84% |
| Class D | 0.15 | 0.28 | 0.39 | 12.66% | -5.79% | -18.82% | -23.99% | -31.61% | -50.26% | -39.37% | -44.59% |
| Class E | 3.02 | 0.42 | 0.33 | -12.10% | -16.91% | -21.79% | -23.33% | -33.29% | -48.57% | -30.63% | -33.24% |
| All | 0.77 | 0.24 | 0.23 | 2.16% | -10.51% | -19.83% | -23.44% | -30.21% | -46.68% | -34.47% | -38.44% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 84% |  |  |  |  |  |  |  |  |  |  |

Table 6-1. E133 vs. HM3.0 RC2 anchor: IBDI on, 4x4-block-based memory compression, RA HE

|  | HE Random access |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A | 0.07 | 0.04 | 0.05 | 31.10% | 19.87% | 5.96% | 1.21% | -20.25% | -32.36% | -25.24% | -27.98% |
| Class B | 0.16 | -0.02 | 0.06 | 7.98% | 4.17% | 0.67% | -3.88% | -11.07% | -23.83% | -12.17% | -13.83% |
| Class C | 0.15 | 0.12 | 0.13 | 1.93% | -10.60% | -19.73% | -23.49% | -28.97% | -44.77% | -34.38% | -38.43% |
| Class D | 0.11 | 0.04 | 0.11 | 8.02% | -7.22% | -18.47% | -23.24% | -29.68% | -46.93% | -35.73% | -40.01% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.12 | 0.04 | 0.09 | 11.95% | 1.66% | -7.47% | -11.92% | -21.96% | -36.35% | -26.09% | -29.20% |
| Enc Time[%] | 100% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 79% |  |  |  |  |  |  |  |  |  |  |

Table 6-2. E133 vs. HM3.0 RC2 anchor: IBDI on, 4x4-block-based memory compression, LD HE

|  | HE Low delay |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.87 | 0.43 | 0.79 | 0.54% | -10.91% | -18.83% | -21.96% | -26.41% | -41.44% | -29.43% | -32.85% |
| Class C | 0.49 | 0.40 | 0.35 | 4.64% | -9.73% | -20.42% | -24.66% | -31.08% | -48.06% | -38.07% | -42.56% |
| Class D | 0.21 | 0.24 | 0.91 | 12.85% | -5.62% | -18.68% | -23.85% | -31.49% | -50.17% | -39.33% | -44.55% |
| Class E | 6.27 | 1.33 | 1.16 | -12.13% | -16.94% | -21.82% | -23.35% | -33.32% | -48.58% | -29.82% | -32.46% |
| All | 1.62 | 0.54 | 0.78 | 2.27% | -10.42% | -19.75% | -23.37% | -30.14% | -46.62% | -34.14% | -38.13% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 78% |  |  |  |  |  |  |  |  |  |  |

Table 7-1. FixedRounding vs. HM3.0 RC2 anchor: IBDI on, single-pixel-based memory compression, RA HE

|  | HE Random access |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A | 0.38 | 1.23 | 1.64 | -11.14% | -0.63% | 22.44% | 22.93% | 33.31% | 48.37% | 28.32% | 32.99% |
| Class B | 1.03 | 3.47 | 4.91 | -10.70% | -4.85% | 2.60% | 1.68% | 9.94% | 24.31% | -2.10% | -1.57% |
| Class C | 0.69 | 2.26 | 2.24 | -20.27% | -20.74% | -19.27% | -18.32% | -10.93% | -3.11% | -21.26% | -22.88% |
| Class D | 0.51 | 1.66 | 1.91 | -20.90% | -21.68% | -20.75% | -18.86% | -11.56% | -1.85% | -24.52% | -25.02% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.68 | 2.23 | 2.81 | -15.42% | -11.57% | -3.42% | -2.88% | 5.40% | 17.35% | -4.74% | -3.98% |
| Enc Time[%] | 101% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |  |  |  |  |  |  |

Table 7-2. FixedRounding vs. HM3.0 RC2 anchor: IBDI on, single-pixel-based memory compression, LD HE

|  | HE Low delay |  |  | Memory bandwidth increase % |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | 8bit/8bit | 32bit/ 64bit | 32bit/128bit | 64bit/128bit | 64bit/256bit | 64bit/512bit | 64bit/256bit FIFO | 64bit/512bit FIFO |
| Class A |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 2.35 | 6.10 | 8.60 | -18.63% | -19.06% | -16.69% | -15.84% | -7.45% | -0.93% | -12.48% | -13.40% |
| Class C | 1.31 | 4.24 | 4.42 | -19.97% | -20.90% | -20.09% | -17.79% | -10.94% | -1.97% | -15.56% | -16.28% |
| Class D | 0.79 | 6.47 | 7.43 | -20.72% | -22.80% | -22.14% | -17.58% | -10.04% | -0.44% | -18.01% | -17.52% |
| Class E | 14.69 | 17.84 | 25.43 | -15.76% | -17.09% | -16.43% | -15.99% | -18.30% | -17.29% | -8.28% | -8.16% |
| All | 4.01 | 7.93 | 10.42 | -18.95% | -20.09% | -18.85% | -16.79% | -11.00% | -4.14% | -13.85% | -14.17% |
| Enc Time[%] | 102% |  |  |  |  |  |  |  |  |  |  |
| Dec Time[%] | 112% |  |  |  |  |  |  |  |  |  |  |

Result of ClassC is reported in detail in Figure 3-Figure 6.

From the results, the proposed method

- Achieves similar RD performance as the 4x4 unit-based ones.

- Achieves smaller memory access bandwidth when burst parameter is lower than or equal to128bits, under almost the same RD coding efficiency for both RA-HE and LD-HE cases.

- While the burst parameter is higher than 128bits ,by comparing Figure 3 with Figure 5，we can see that four 2x2 units are reorganized into 4x4 unit for storing, and the same memory access bandwidth can be achieved as the 4x4 ones under almost the same RD coding efficiency.

That is, by analyzing and synthesizing the factors of memory access bandwidth and coding efficiency, our proposal may balance both of them better.

Moreover, this scheme may bring other benefits:

- With smaller granularity, the memory storage format alignment between the RFC and display processing problem is alleviated.

- Smaller compression unit is good for parallel processing

Figure 3  Memory access bandwidth for Random access High Efficiency (No memory reorganization)

Figure 4   Memory access bandwidth for Low Delay High Efficiency (No memory reorganization)

Figure 5  Memory access bandwidth for Random Access High Efficiency (memory reorganization)

Figure 6  Memory access bandwidth for Low Delay High Efficiency (memory reorganization)

# Conclusion

In this contribution, the 2x2 block is used as the unit of reference picture memory compression instead of 4x4 block. The algorithm and standardization are both given. By this scheme, memory access bandwidth is decreased by 15~17% comparing to the 4x4 ones while burst parameter is smaller than 64. When the burst is with a larger value, the 2x2 units can be reorganized into 4x4 or even 8x8 units in decoder memory to avoid memory bandwidth increasing (Refer to Figure 3, 4 and Figure 5, 6 for RA-HE, LD-HE. Since the complexity introduced is negligible, it is suggested this compression scheme should be introduced into the reference picture memory compression.

# 5  Reference

- Takeshi Chujoh, Komukai-Toshiba-cho, Saiwai-ku, Adaptive scaling for reference pictures memory compression, JCT-VC Document, JCTVC-E133, Geneva, March 2011.

- Dzung Hoang, Unified scaling for 10-bit to 8-bit reference frame compression, JCT-VC Document, JCTVC-E432, Geneva, March 2011.

- HM3.0

- Frank Bossen, Common test conditions and software reference configurations, JCT-VC Document, JCTVC-E700, Geneva, March 2011.

- Keiichi Chono, Takeshi Chujoh, ChongSoon Lim, Ali Tabatabai, Minhua Zhou, Reference pictures

memory compression, JCT-VC AHG report, JCTVC-E008, Geneva, March 2011.

# 6  Patent rights declaration(s)

Huawei Tech. Co. Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University does not have any current or pending patent rights relating to the technology described in this contribution.