| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F498 |
| --- | --- |

| Title: | Adaptive Loop Filter Merge in Temporal Domain |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xinfeng Zhang1,2 Ruiqin Xiong1 Siwei Ma1 Wen Gao1 | Email: | xfzhang@jdl.ac.cn rqxiong@ jdl.ac.cn swma@jdl.ac.cn wgao@jdl.ac.cn |
| Source: | Peking University1, Institute of Computing Technology, Chinese Academy of Sciences2. |  |  |

_____________________________

# Abstract

This contribution proposes a temporal adaptive loop filter (ALF) merging method. The proposed method utilizes decoded ALF parameters from prior frames to implement the adaptive loop filtering. The usage of whether prior decoded filter bank or the current filter bank in HM3.0 is signaled in the bitstream. Compared with HM3.0, the proposed method reportedly achieves 0.3%, 0.2%, and 0.2% bit rate reductions for HE-RA, HE-LD, and HE-LD (P), respectively. The encoding time is reportedly increased by 2%, while the decoding time is reportedly increased by 2~5%. Coding results are confirmed by Microsoft with JCTVC-F207.

# Introduction

To save the bits for transmitting the ALF parameters, an ALF parameter merging scheme in spatial domain is employed in HM3.0. For low bit rate cases, many frames are still not able to afford the bits for transmitting many filters parameters and CU adaptive signals, which restraints the performance of ALF. In order to further save the bits signaling the ALF parameters, in this contribution, a temporal ALF parameter merging scheme is proposed. The proposed scheme makes the filter parameters utilized in prior coded picture reused for current picture without transmit these filter parameters.

# Algorithm description

In this contribution, the temporal ALF parameters merging process is implemented. For notations, the adaptive loop filter bank generating from current picture is denoted as F1 and that employed in previous pictures is denoted as F2.

For intra frame coding, the adaptive loop filter does not changed, except for updating F2 with filter parameters of current picture. For inter frame coding, filter bank F1 is first employed. If ALF is disabled based on F1, the same process is conducted as HM 3.0. If ALF for current frame is enabled based on F1, another filtering process with F2 is carried out. The filter bank with minimal rate distortion cost is selected as the final filter bank.

- If F1 is selected, the ALF parameters of F2 are updated with F1.

- If F2 is selected, the filter parameters of F2 do not encode.

Therefore, it is implemented that ALF adaptively merge in temporal domain. The structure of the proposed scheme is illustrated in Fig.1 (b), compared with that in HM 3.0 in Fig.1 (a).

In the proposed scheme, the adaptive loop filter parameters used in prior coded pictures are needed to save and update both in encoder and decoder. The ALF parameters need to be stored in F2 are ALF coefficients, taps and filter pattern. These syntax elements are all in the PPS level when shared_pps_info_enabled_flag is enable according to JCTVC-E045 [1].

The proposed syntax and semantics is shown in dAppendix the modified part is highlighted with yellow background.

| alf_param() { |  |  |
| --- | --- | --- |
| if ( adaptive_loop_filter_flag ) { |  |  |

(a)

(b)

Figure 1 (a) the structure of ALF in HM 3.0, (b) the structure of the proposed ALF scheme

# Simulation results

The high efficiency (HE) test configurations in [2] were tested. Experimental results are reported as in Table 2. The proposed method achieves 0.3%, 0.2%, and 0.2% bit rate reductions for HE-RA, and HE-LD and HE_LD (P) respectively. The encoding time increase by 2%. The decoding time is increased by 3-5%.

Table 2 Results of the proposed method compared with HM3.0

|  | Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | 0.0 |  | 0.0 |  | 0.0 |
| Class B | 0.0 |  | 0.0 |  | 0.0 |
| Class C | 0.0 |  | 0.0 |  | 0.0 |
| Class D | 0.0 |  | 0.0 |  | 0.0 |
| Class E | 0.0 |  | 0.0 |  | 0.0 |
| All | 0.0 |  | 0.0 |  | 0.0 |
| Enc Time[%] | 103% |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |
|  |  |  | Random access |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.2 |  | -0.3 |  | 0.0 |
| Class B | -0.1 |  | -0.2 |  | -0.2 |
| Class C | -0.3 |  | -0.2 |  | -0.2 |
| Class D | -0.5 |  | -0.5 |  | -0.5 |
| Class E |  |  |  |  |  |
| All | -0.3 |  | -0.3 |  | -0.2 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 105% |  |  |  |  |
|  | Low delay |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |
| Class B | -0.1 |  | 0.2 |  | 0.2 |
| Class C | -0.2 |  | -0.4 |  | 0.3 |
| Class D | -0.4 |  | -0.8 |  | 0.0 |
| Class E | 0.0 |  | -0.5 |  | -0.3 |
| All | -0.2 |  | -0.3 |  | -0.2 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |
|  | Low delay(P) |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |
| Class B | -0.1 |  | -0.1 |  | -0.3 |
| Class C | -0.1 |  | -0.1 |  | -0.3 |
| Class D | -0.4 |  | -0.6 |  | -0.9 |
| Class E | -0.1 |  | 0.1 |  | -0.5 |
| All | -0.2 |  | -0.2 |  | -0.5 |
| Enc Time[%] | 102% |  |  |  |  |
| Dec Time[%] | 103% |  |  |  |  |

# Conclusion

In this contribution, a temporal ALF parameter merging scheme is proposed. It is reported that the proposed method achieves 0.3%, 0.2%, and 0.2% bit rate reductions for HE-RA, HE_LD and HE-LD (P), respectively. The encoding time is reportedly increased by 2%, while the decoding time is reportedly increased by 2~5%.

# References

- F. Bossen, “Common Test Conditions and Software Reference Configurations,” Document of Joint Collaborative Team on Video Coding, JCTVC-E700, Mar. 2011.

- C.-Y. Tsai, C.-W. Hsu, Y.-W. Huang and S. Lei, “CE4 Subset3: Slice Common Information Sharing,” Document of Joint Collaborative Team on Video Coding, JCTVC-E045, Mar. 2011.

# Patent rights declaration(s)

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

#### 7.3.3.5			Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_temporal_adaptive_merge_flag | 2 | u(1) |
| alf_region_adaptation_flag | 2 | u(1) |
| if (alf_temporal_adaptive_merge_flag == 0) { |  |  |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| if (alf_no_filters_minus1 == 1) |  |  |
| alf_start_second_filter | 2 | ue(v) |
| else if (alf_no_filters_minus1 > 1) { |  |  |
| for (i=0; i< 16; i++) |  |  |
| alf_filter_pattern[i] | 2 | u(1) |
| } |  |  |
| if (AlfNumFilters > 1) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| } |  |  |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

7.4.3.4	Adaptive loop filter parameter semantics

alf_temporal_adaptive_merge_flag specifies whether temporal merging is applied. If alf_temporal_adaptive_merge_flag is equal to 1, temporal merging is applied filter parameters in buffers are used in decoder. If alf_temporal_adaptive_merge_flag is equal to 0, the filter parameters are acquired from bitstream in decoder.