| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F499 |
| --- | --- |

| Title: | Temporal QP Memory Compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA Hirofumi Aoki Keiichi Chono 1753, Shimonumabe, Nakahara-Ku Kawasaki, Kanagawa, 211-8666 JAPAN | Tel: Email: | +1 858 658 3973 mcoban@qualcomm.com martak@qualcomm.com +81 50 3757 5438 hao@bu.jp.nec.com chono@ct.jp.nec.com |
| Source: | Qualcomm Inc, NEC corporation |  |  |

_____________________________

# Abstract

This contribution presents a temporal QP memory compression scheme that is complementary to the temporal QP prediction method of JCTVC-F103. Temporal QP memory compression reduces the amount of storage requirements for QP values in reference frames. The memory reduction results in less than 0.1%, 0.2% and 0.3% average BD-rate increase for 4x, 16x and 64x memory reductions, respectively for MinCUDQPSize of 8x8. Coding gain relative to storage requirement is more than that of temporal MV prediction.

# Introduction

Perceptual quantization or adaptive quantization targets visual quality improvement [2] [3] [5]. Generally it performs finer quantization on flat areas and coarser quantization on busy areas. Spatial and temporal correlations among prediction blocks’ QPs are reflected to the QP prediction. QP prediction based on intra prediction is proposed in JCTVC-E198 [3] and JCTVC-E215 [4]. Inter prediction based temporal QP prediction is proposed in [4] and JCTVC-F103 [2]. Temporal QP prediction requires storage of QP values in reference frames. By employing temporal QP memory compression scheme in reference frames, storage requirements for QP values could be significantly reduced with minimal impact on coding efficiency.

# Algorithm description

Temporal QP memory compression algorithm reduces the size of the QP buffers for storing reference frames’ QPs. This algorithm replaces the decoded QP buffer with a lower resolution buffer where only sub sampled QP values that are derived by averaging QP values of neighboring group of quantization group of coding units are stored. For sub sampling factor of N (represented by MinCUTQPSizeLX ) for group of  quantization group of coding units , this can be expressed as

.

Effectively lower resolution  values are stored instead of the  values. Sub sampling factor N can be signaled in the PPS header or set to a prefixed value.

# Experiments

## Setup

The proposed QP prediction algorithm has been implemented onto the software for CE4 Subtest 2.4.b [2]. Experiments to evaluate performance of the proposed method were conducted under the CE4 Subtest2 test conditions [1], with all applicable combinations of MinCUDQPSize and MinCUTQPSizeLX listed in Table 1. Note that cases without compression are shown in JCTVC-F103 [2] and omitted here.

Table 1: Test cases.

| MinCUDQP | MinCUTQP |  |  |
| --- | --- | --- | --- |
|  | 16x16 | 32x32 | 64x64 |
| 8x8 | X | X | X |
| 16x16 |  | X | X |
| 32x32 |  |  | X |

Our computing platform used for the experiments is shown below.

- OS: Windows 7 Professional 64-bit, Linux 64-bit cluster

- CPU: Intel Xeon X5680 3.46GHz, 6 physical cores x 2 CPUs (12 cores in total) (Windows 7)

- Memory: 32GiB

It should be noted that individual decoding/encoding time can be significantly varied since the CPU have automated over-clocking functionality and I/O speed varies by 12 processes running in parallel.

## Results

The performance of temporal QP memory compression (TQPMC) is evaluated by introducing it to the inter QP prediction of test 2.4.b and comparing it with the test 2.4.a employing intra QP prediction. BD-Rates and processing time ratios against test 2.4.a, which employs only intra QP prediction, are compared for all cases listed inError: Reference source not found.

Table 2 shows brief summary of experimental results for all valid combinations of MinCUDQPSize and MinCUTQPSizeLX. Table 4 shows the summary of experimental results with MinCUDQPSize = 8x8 and MinCUTQPSizeLX = 16x16 with respect to 2.4.a. In addition, BD-Rates and processing time ratios against CE4 anchor were computed to evaluate total gains by intra and inter QP prediction with temporal QP memory compression. Brief summary of them is shown in Table 3. Table 5 shows the summary of experimental results with MinCUDQPSize = 8x8 and MinCUTQPSizeLX = 16x16 with respect to the CE4 anchor. Note that results for all intra settings are omitted in Table 2 - Table 5 since the algorithm under test has no effect on intra frames. More detailed results are shown in the attached spreadsheets. The results show that temporal QP memory compression brings significant storage reductions with less than 0.1%, 0.2% and 0.3% average BD-rate increase with for 4x, 16x and 64x memory reductions, respectively for MinCUDQPSize of 8x8. Same results hold for 16x16 and 32x32 MinCUDQPSizes.

Table 2: Brief summary results of test 2.4.b + TQPMC vs. 2.4.a.

| MinCUD(T)QP | Y BD-rates |  |  |  | dQP bits |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HE | RA LC | LB HE | LB LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 (16x16) | -0.4 | -0.4 | -0.7 | -0.8 | -12.4% | -12.3% | -16.5% | -16.5% |
| 8x8 (32x32) | -0.3 | -0.2 | -0.6 | -0.6 | -9.1% | -8.1% | -12.7% | -11.6% |
| 8x8 (64x64) | -0.2 | -0.1 | -0.4 | -0.3 | -5.1% | -3.1% | -8.3% | -5.6% |
| 16x16 (32x32) | -0.2 | -0.3 | -0.5 | -0.6 | -13.4% | -12.7% | -17.8% | -16.9% |
| 16x16 (64x64) | -0.1 | -0.1 | -0.4 | -0.3 | -8.0% | -5.9% | -11.4% | -8.8% |
| 32x32 (64x64) | -0.1 | -0.1 | -0.3 | -0.3 | -14.6% | -13.6% | -20.4% | -19.1% |

Table 3: Brief summary results of test 2.4.b + TQPMC vs. CE4 anchor.

| MinCUD(T)QP | Y BD-rates |  |  |  | dQP bits |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HE | RA LC | LB HE | LB LC | RA HE | RA LC | LB HE | LB LC |
| 8x8 (16x16) | -0.5 | -0.5 | -0.8 | -0.9 | -16.0% | -16.2% | -18.1% | -18.4% |
| 8x8 (32x32) | -0.4 | -0.4 | -0.7 | -0.7 | -12.9% | -12.5% | -14.4% | -13.6% |
| 8x8 (64x64) | -0.3 | -0.3 | -0.5 | -0.4 | -9.0% | -7.4% | -10.0% | -7.7% |
| 16x16 (32x32) | -0.3 | -0.3 | -0.6 | -0.6 | -16.1% | -15.6% | -19.0% | -18.3% |
| 16x16 (64x64) | -0.2 | -0.2 | -0.4 | -0.4 | -10.8% | -9.0% | -12.7% | -10.4% |
| 32x32 (64x64) | -0.2 | -0.2 | -0.3 | -0.4 | -16.1% | -15.4% | -21.1% | -20.0% |

Table 4: Summary results of test 2.4.b + TQPMC vs. 2.4.a with MinCUDQPSize = 8x8, MinCUTQPSize = 16x16 (*Reference and tested are run on different platforms)

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.3 | -0.4 | -0.3 | -12.0% | -0.3 | -0.4 | -0.4 | -11.9% |
| Class B | -0.3 | -0.4 | -0.4 | -12.0% | -0.3 | -0.4 | -0.4 | -11.3% |
| Class C | -0.4 | -0.5 | -0.4 | -13.0% | -0.4 | -0.6 | -0.4 | -13.0% |
| Class D | -0.3 | -0.6 | -0.5 | -12.6% | -0.4 | -0.5 | -0.3 | -13.1% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.4 | -12.4% | -0.4 | -0.5 | -0.4 | -12.3% |
| Enc Time[%] | 119%* |  |  |  | 116%* |  |  |  |
| Dec Time[%] | 101%* |  |  |  | 106%* |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.6 | -0.8 | -0.7 | -15.3% | -0.6 | -0.7 | -0.8 | -14.2% |
| Class C | -0.7 | -0.7 | -0.6 | -15.9% | -0.7 | -0.9 | -0.6 | -15.9% |
| Class D | -0.6 | -0.9 | -0.6 | -15.7% | -0.7 | -0.9 | -0.9 | -16.4% |
| Class E | -1.0 | -0.8 | -1.1 | -20.5% | -1.2 | -1.5 | -1.1 | -21.6% |
| All | -0.7 | -0.8 | -0.7 | -16.5% | -0.8 | -1.0 | -0.8 | -16.5% |
| Enc Time[%] | 119%* |  |  |  | 113%* |  |  |  |
| Dec Time[%] | 98%* |  |  |  | 104%* |  |  |  |

Table 5: Summary results of test 2.4.b + TQPMC vs. CE4 anchor with MinCUDQPSize = 8x8, MinCUTQPSize = 16x16

|  | Random Access HE |  |  |  | Random Access LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A | -0.4 | -0.6 | -0.4 | -14.2% | -0.4 | -0.4 | -0.5 | -14.2% |
| Class B | -0.5 | -0.5 | -0.5 | -15.9% | -0.5 | -0.6 | -0.6 | -15.4% |
| Class C | -0.7 | -0.7 | -0.6 | -17.8% | -0.7 | -0.8 | -0.6 | -18.2% |
| Class D | -0.4 | -0.6 | -0.5 | -16.3% | -0.6 | -0.7 | -0.6 | -17.2% |
| Class E |  |  |  |  |  |  |  |  |
| All | -0.5 | -0.6 | -0.5 | -16.0% | -0.5 | -0.6 | -0.6 | -16.2% |
| Enc Time[%] | 100% |  |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  |  | 99% |  |  |  |
|  | Low delay B HE |  |  |  | Low delay B LC |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | dQP incr. | Y BD-rate | U BD-rate | V BD-rate | dQP incr. |
| Class A |  |  |  |  |  |  |  |  |
| Class B | -0.7 | -0.8 | -0.7 | -16.8% | -0.7 | -0.7 | -0.8 | -16.0% |
| Class C | -0.8 | -0.8 | -0.7 | -18.0% | -0.9 | -0.9 | -0.9 | -18.6% |
| Class D | -0.7 | -0.9 | -0.4 | -17.2% | -0.9 | -0.9 | -1.2 | -18.1% |
| Class E | -1.1 | 0.2 | -0.1 | -21.4% | -1.3 | -1.5 | -1.3 | -22.4% |
| All | -0.8 | -0.6 | -0.5 | -18.1% | -0.9 | -1.0 | -1.0 | -18.4% |
| Enc Time[%] | 100% |  |  |  | 99% |  |  |  |
| Dec Time[%] | 97% |  |  |  | 99% |  |  |  |

# Analysis on storage requirement

TQPMC aims to improve the balance between compression performance and memory requirement of temporal QP prediction (TQPP). Here analysis results on the balance are shown and they are compared with that of temporal MV prediction (TMVP). To evaluate it, ratio of coding gain to bits per reference SCU [6][2], which is calculated by dividing the negative of the luma BD-Rate value by bits consumed for each SCU on reference frame, is employed. As for TQPP with TQPMC, the value is calculated as follows:

RTQPMC = B  (6  2log2MinCUTQPSizeLX-Log2MinCUSize),

where B is BD-Rate value and 6 means bits consumed for storing each QP.

Calculated ratio values, namely relative coding gains, are shown in Table 11. Relative coding gains for TMVP are quoted from JCTVC-F099 [6] and those for TQPP without compression are quoted from JCTVC-F103 [2]. Table 12 shows results of TQPP with TQPMC vs. TMVP, evaluated by ratios of relative coding gains. In these tables, TQPP with MinCUDQPSize=NxN is represented by “TQPP NxN” and unit size of MxM for storing data in reference frames is represented by “tMxM”. Results of TQPP with TQPMC are highlighted in boldface type.

As shown here, with TQPMC, relative coding gain of TQPP is always more than that of TMVP. When QPs in reference frames are stored at 64x64 block level, the relative coding gain of TQPP with TQPMC attains up to about 20 times of that of TMVP, for LB configurations. Although these results do not mean that TQPP with TQPMC is up to 20 times better than TMVP, it is indicated that TQPP with TQPMC is worthwhile to be introduced into the next version of HM.

Table 11: Relative coding gains of TMVP and TQPP.

|  | TMVP | TQPP 8x8 |  |  |  | TQPP 16x16 |  |  | TQPP 32x32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | t16x16 | t8x8 | t16x16 | t32x32 | t64x64 | t16x16 | t32x32 | t64x64 | t32x32 | t64x64 |
| RA.HE | 0.17 | 0.07 | 0.23 | 0.75 | 1.77 | 0.24 | 0.65 | 1.59 | 0.44 | 1.46 |
| RA.LC | 0.16 | 0.07 | 0.24 | 0.63 | 1.06 | 0.27 | 0.69 | 1.21 | 0.42 | 1.36 |
| LB.HE | 0.17 | 0.12 | 0.46 | 1.50 | 3.79 | 0.49 | 1.42 | 3.78 | 1.27 | 3.71 |
| LB.LC | 0.17 | 0.15 | 0.51 | 1.57 | 3.40 | 0.56 | 1.51 | 3.40 | 1.14 | 3.65 |

Table 12: Results of TQPP vs. TMVP.

|  | TQPP 8x8 |  |  |  | TQPP 16x16 |  |  | TQPP 32x32 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | t8x8 | t16x16 | t32x32 | t64x64 | t16x16 | t32x32 | t64x64 | t32x32 | t64x64 |
| RA.HE | 0.43 | 1.42 | 4.55 | 10.72 | 1.47 | 3.95 | 9.64 | 2.66 | 8.85 |
| RA.LC | 0.46 | 1.51 | 3.94 | 6.61 | 1.65 | 4.29 | 7.57 | 2.61 | 8.46 |
| LB.HE | 0.74 | 2.73 | 9.01 | 22.71 | 2.94 | 8.50 | 22.62 | 7.60 | 22.21 |
| LB.LC | 0.88 | 3.03 | 9.27 | 20.10 | 3.31 | 8.93 | 20.13 | 6.75 | 21.60 |

# Conclusions

This contribution has presented the detailed description and the results of temporal QP memory compression prediction method that is complementary to temporal QP prediction in intra/inter prediction (test 2.4.b) for CE4 subtest 2. Temporal QP memory compression reduces the amount of storage requirements for QP values in reference frames. The memory reduction results in less than 0.1%, 0.2% and 0.3% average BD-rate increase for 4x, 16x and 64x memory reductions, respectively for MinCUDQPSize of 8x8. Each stage of sub sampling introduces 0.1% BD-rate increase with 4x reduction in QP buffer size. Coding gain relative to storage requirement is more than that of temporal MV prediction. It is therefore recommended that intra/inter QP prediction combined with the presented QP memory compression be incorporated into HM.

# References

- K. Sato, et al., “Description of Core Experiment 4: Quantization,” JCT-VC document JCTVC-E704, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- H. Aoki, et al., “CE4 Subtest 2: QP prediction based on intra/inter prediction (test 2.4.b),” JCT-VC document JCTVC-F103, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

- M. Kobayashi and M. Shima, “Sub-LCU level delta QP signaling,” JCT-VC document JCTVC-E198, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- H. Aoki, et al., “Prediction-based QP derivation,” JCT-VC document JCTVC-E215, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 5th meeting, Geneva, March 2011.

- K. Sato, et al., “On LBS and Quantization,” JCT-VC document JCTVC-D308, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 4th meeting, Daegu, January 2011.

- H. Aoki, et al., “Performance report of temporal motion vector prediction,” JCT-VC document JCTVC-F099, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/ WG11 6th meeting, Torino, July 2011.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NEC corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed modification of HEVC Working Draft 3

The modified or added portions are highlighted with yellow or green colour.

Note: This modification includes semantics modification by intra/inter QP prediction without compression, which is described in Appendix B of JCTVC-F103 [2]. The portions described in [2] are highlighted with yellow colour and the portions introduced in this contribution are highlighted with green colour.

### A.1. Change of subclause 7.3.2.2 “Picture parameter set RBSP syntax”

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| max_cu_qp_delta_depth | u(4) |
| temporal_qp_prediction_enabled_flag | u(1) |
| if( temporal_qp_prediction_enabled_flag) { |  |
| max_cu_temporal_qp_depth | ue(v) |
| } |  |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

### A.2. Change of subclause 7.4.2.2 “Picture parameter set RBSP semantics”

temporal_qp_prediction_enabled_flag equal to 1 specifies that temporal QP prediction is enabled in derivation process for quantisation parameters. temporal_qp_prediction_enabled_flag equal to 0 specifies that temporal QP prediction is disabled in derivation process for quantisation parameters. When temporal_qp_prediction_enabled_flag is not present, temporal_qp_prediction_enabled_flag shall be inferred to be equal to 0.

max_cu_temporal_qp_depth specifies the maximum allowed depth that is used for specifying temporal QPY values for coding unit. The value of max_cu_temporal_qp_depth shall be in the range of 0 to max_cu_qp_delta_depth, inclusive.

The variable Log2MinCUTQPSize specifying the minimum coding unit size that specifies the temporal QPY values for coding unit as follows:

Log2MinCUTQPSize = Log2MaxCUSize – max_cu_temporal_qp_depth

The variable MinCUTQPSize is set equal to (1 << Log2MinCUTQPSize).

### A.3. Change of subclause 7.4.9 “Transform coefficient semantics”

cu_qp_delta can change the value of QPY specifies the difference between a luma quantisation parameter to be used and its prediction, for a quantisation group of coding units where the quantisation group of coding units is specified as follows:

- If a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 0 and the log2CUSize is greater than or equal to log2MinCUDQPSize, the quantisation group includes this coding unit only.

- Otherwise, if a coding unit with the split_coding_unit_flag[ x0 ][ y0 ] equal to 1 and the log2CUSize is equal to log2MinCUDQPSize, the quantisation group includes all coding units split from this coding unit.

The decoded value of cu_qp_delta shall be in the range of –( 26+ QpBdOffsetY / 2 ) to +( 25+ QpBdOffsetY / 2 ), inclusive. cu_qp_delta shall be inferred to be equal to 0 when it is not present for any quantisation group of coding units.

The value of QPY is derived as

QPY = ( ( ( QPY,PREV + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(7-1)

where QPY,PREV is the luma quantization parameter, QPY, of the left neighbor quantization group of coding units in the current slice. If the left neighbor quantization group in the current slice is not available, QPY,PREV is the luma quantization parameter, of the previous quantization group in decoding order in the current slice. For the first quantization group of coding units in the slice QPY,PREV is initially set equal to SliceQPY at the start of each slice.

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(7-2)

### A.4. Insertion of new subclause into clause 8.5

### 8.5.x	Derivation process for luma quantisation parameters

Inputs to this process are

–	a variable QpBdOffsetY,

–	a syntax element cu_qp_delta specifying the delta luma quantisation parameter of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	a variable PredMode specifying the prediction mode of the top-left coding unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Outputs of this process are

–	luma quantisation parameters QPY and QP’Y.

Depending on PredMode, the following applies.

–	If PredMode is equal to MODE_INTRA, spatial quantisation parameter prediction process as specified in subclause 8.5.x.2 is invoked with the output being the reference quantisation parameter QPY, PRED.

–	Otherwise, if temporal_qp_prediction_enabled_flag is equal to 1, temporal quantisation parameter prediction process as specified in subclause 8.5.x.3 is invoked with the output being the predicted quantisation parameter QPY, PRED.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the predicted quantisation parameter QPY, PRED.

With QPY,PRED, the value of QPY is derived as

QPY = ( ( ( QPY,PRED + cu_qp_delta +52+ 2*QpBdOffsetY )%( 52 + QpBdOffsetY ) ) - QpBdOffsetY	(8-3)

The value of QP’Y is derived as

QP’Y = QPY + QpBdOffsetY		(8-4)

### 8.5.x.1		Default quantisation parameter prediction process

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, QPY, PRED is set equal to QPY, PREV, where QPY,PREV is the luma quantisation parameter of the left neighbouring quantisation group of coding units in the current slice. For the first quantisation group of coding units in the slice, QPY,PREV is initially set equal to SliceQPY at the start of each slice.

### 8.5.x.2		Spatial quantisation parameter prediction process

Inputs to this process are

–	a variable intraPCMFlag specifying if top-left coding unit of the current quantisation group of coding units is coded in PCM mode.

–	a variable intraPredMode specifying the intra prediction mode of the top-left prediction unit of the current quantisation group of coding units relative to the top-left sample of the current picture.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

The rearranged intra prediction order intraPredOrder is derived as follows.

–	If one of the following conditions is true, intraPredOrder is set to 0.

- intraPCMFlag is equal to 1.

- intraPredMode is equal to 2 (Intra_DC).

–	Otherwise, if intraPredMode is equal to 0 (Intra_Vertical), intraPredOrder is set to 9.

–	Otherwise, if intraPredMode is equal to 1 (Intra_Horizontal), intraPredOrder is set to 25.

–	Otherwise, intraPredOrder is derived as specified in Table 8-6.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If intraPredOrder is more than 0 and less than 18 and the above neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the above neighbouring quantisation group of coding units in the current slice.

–	Otherwise, if intraPredOrder is equal to or more than 18 and the left neighbouring quantisation group of coding units in the current slice is available, then QPY, PRED is set equal to the luma quantisation parameter QPY of the left neighbouring quantisation group of coding units in the current slice.

–	Otherwise, default quantisation parameter prediction process as specified in subclause 8.5.x.1 is invoked with the output being the reference quantisation parameter QPY, PRED.

### 8.5.x.3		Temporal quantisation parameter prediction process

Inputs to this process are

–	a luma location ( xQ, yQ ) specifying the top-left luma sample of the current quantisation group of coding units relative to the top-left sample of the current picture,

–	variables specifying the width and the height of the top-left prediction unit of the current quantisation group of coding units, nPSW and nPSH,

–	luma motion vectors mvL0 and mvL1 of the top-left prediction unit of the current quantisation group of coding units,

–	reference indices refIdxL0 and refIdxL1 of the top-left prediction unit of the current quantisation group of coding units,

–	prediction list utilization flags predFlagL0 and predFlagL1 of the top-left prediction unit of the current quantisation group of coding units.

Output of this process is

–	the predicted quantisation parameter QPY, PRED.

For X being replaced by 0 or 1, when predFlagLX is equal to 1, the uni-predicted quantisation parameter PredQPLX is derived in the following ordered steps.

- The variables xQRefLX and yQRefLX specifying the center of the reference block used for motion compensation of the top-left prediction unit of the current quantisation group of coding units are derived as follows.

xQRefLX = xQ + nPSW/2 + ( mvLX[ 0 ] >> 2 )					(8-xx3)
yQRefLX = yQ + nPSH/2 + ( mvLX[ 1 ] >> 2 )						(8-xx4)

- Let variables RefQPLX and SliceQPLX be set as follows.

- RefQPLX is derived in the following ordered steps.

- The variable Log2MinCUTQPSizeLX is set equal to Log2MinCUTQPSize of the reference frame specified by refIdxLX, and the variable MinCUTQPSizeLX is set equal to MinCUTQPSize of the reference frame specified by refIdxLX.

- The variables xQRefTlLX and yQRefTlLX are derived as follows.

xQRefTlLX = (xQRefLX >> Log2MinCUTQPSizeLX ) << Log2MinCUTQPSizeLX	(8-xx5)
yQRefTlLX = (yQRefLX >> Log2MinCUTQPSizeLX ) << Log2MinCUTQPSizeLX	(8-xx6)

- The variable RefQPLX is derived as

RefQPLX=1MinCUTQPSizeLX2((j=0MinCUTQPSizeLX−1i=0MinCUTQPSizeLX-1QLX(xQRefTlLX+i,yQRefTlLX+j))+MinCUTQPSizeLX2/2)										(8-xx7)

where QLX(x,y) specifies the luma quantisation parameter of the quantisation group of coding units covering luma location (x, y) in the reference frame specified by refIdxLX.

- SliceQPLX is the initial value of quantisation parameter of the slice covering luma location  (xQRefLX, yQRefLX) in the reference frame specified by refIdxLX.

- The uni-predicted quantisation parameter PredQPLX is derived as follows.

PredQPLX = RefQPLX + SliceQPY – SliceQPLX,					(8-xx8)

where SliceQPY is the initial value of quantisation parameter of the current slice.

The predicted quantisation parameter QPY, PRED is derived as follows.

–	If predFlagL0 is equal to 1 and predFlagL1 is equal to 0,

QPY,PRED = PredQPL0									(8-xx9)

–	Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1,

QPY,PRED = PredQPL1									(8-xx10)

–	Otherwise (both predFlagL0 and predFlagL1 are equal to 1),

QPY,PRED = ( PredQPL0 + PredQPL1 + 1 ) >> 1						(8-xx11)