| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F502 WG11 Number: m20933 |
| --- | --- |

| Title: | Cross-channel techniques to improve intra chroma prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution reports two techniques to improve intra chroma prediction: a 2-dimensional 3x2 filter for LM intra chroma prediction mode, and a new intra chroma prediction mode based on the technique of local adaptive prediction selection (LAPS) process to produce the intra chroma prediction sample from the comparison of the reconstructed luma sample of target location to the local reconstructed luma samples in the neighborhood. The tests of the 2-D filter for LM intra chroma prediction have reported that an BD-bitrate reduction of U & V components by 0.5% & 0.3% for AI_HE case (up to 2.0% & 1.7% for clip PartyScene) with almost no change in encoding time and decoding time. The tests of LAPS mode have reported that an BD-bitrate reduction of U & V components by 0.3% & 0.3% for AI_HE case (up to 0.9% & 0.9% for clip BasketballPass) with 3% increase in encoding time and almost no change in decoding time. The tests of the combined two techniques have reported that an BD-bitrate reduction of U & V components by 0.7% & 0.6% for AI_HE case (up to 4.0% & 2.9% for clip BasketballDrill) with 3% increase in encoding time and almost no change in decoding time.

# Introduction

Intra chroma prediction based on linear mode (LM) relationship to reconstructed luma samples [1] was adopted into HM3.0 in the last Geneva meeting. In LM intra chroma prediction mode for YUV420 format, the reconstructed luma pixels are down-sample filtered in vertical direction to produce the prediction samples for chroma prediction. In this contribution, a 2-dimensional 3x2 downsample filter is studied to produce the prediction samples from the reconstructed luma samples in order to improve the coding efficiency of LM intra prediction. In addition to improve the existing LM mode in HM3.0, a new intra chroma prediction mode based on the local adaptive prediction selection (LAPS) techniques is studied to improve the coding efficiency of intra chroma prediction in HM3.0. LAPS adaptively selects chroma prediction sample based on the search of local neighboring down-sampled luma samples by comparing the reconstructed luma sample of target location to the reconstructed luma samples in the neighborhood of target location. The following of this contribution has Section 2 and Section 3 to provide the introductions of the two proposed intra chroma prediction techniques, and Section 4 to report the performance data of the coding efficiency and coding complexity of tests. Section 5 concludes this contribution.

# 2-dimensional filter for LM intra chroma prediction

In LM intra chroma prediction mode for the YUV 420 format, a down-sampling process is performed on the reconstructed luma samples to produce the PU size to be the same as the chroma PU size. In [1], a two-tap vertical filter was applied in the down-sampling process for the reconstructed luma samples as pictorially shown in the left part of Figure 1. A 2-dimensional six-tap down-sampling filter, as pictorially shown in the right part of Figure 1, is studied to improve the coding efficiency of LM intra chroma prediction mode. The 2-dimensional filter coefficient set is

1211218                             (1)

Figure 1 - 2-dimensional filter for LM intra chroma prediction

# LAPS mode for intra chroma prediction

A local adaptive prediction selection (LAPS) mode is added to the coding modes of intra chroma prediction to improve the coding efficiency of intra chroma prediction for HM 3.0. LAPS adaptively selects chroma prediction sample based on the search of local neighboring down-sampled luma samples. An example of LAPS scheme based on 5 local neighbors of down-sample luma samples is pictorially demonstrated in Figure 2. Consider a target chroma sample positioned at P with its five neighbors N1 ~ N5 where the 3x2 down-sampling filter introduced in Section 2 was applied to produce the samples N1 ~ N5. Let L(x)be the down-sampled reconstructed luma value of position x, and C(x)be the predicted chroma value of position x. LAPS selects the chroma prediction sample C(P) based on the comparison of the reconstructed luma pixel P to the local neighboring reconstructed luma samples N1 ~ N5.

k=argmini=1~5|L(P)−L(Ni)|                         (2)

If L(Ni)is not available, Ni is not considered in (2). That is, as shown in Figure 2, some positions have only 3 or 4 available neighbors. IfNkis outside the current chroma PU, C(Nk) can be the reconstructed chorma value of positionNk.

Figure 2 - Local adaptive prediction selections

# Performance

The encoder is complied in Linux OS and is tested in a compute farm consisting of hundred of CPU cores. The decoder was tested on a single workstation with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM, and Visual Studio 2008.

## 2-dimensional filter for LM intra chroma prediction

Table 1 shows the performance of 2D filter for LM intra chroma prediction compared with the HM3.0 anchors. In AI_HE case, the average BD bitrate reductions for U and V components are about 0.5% and 0.3% (up to 2.0% and 1.7% for clip PartyScene) respectively with almost no change in encoding and decoding time. In AI_LC case, the average BD bitrate reduction for U and V components are about 0.8% and 0.6% respectively with almost no increase in encoding and decoding time.

Table 1 - The performance of 2D filter for LM intra chroma prediction

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | 0.7 | 0.0 | -0.1 | 0.7 |
| Class B | -0.1 | -0.2 | -0.1 | -0.1 | -0.6 | -0.4 |
| Class C | -0.4 | -1.1 | -1.0 | -0.3 | -1.7 | -1.5 |
| Class D | -0.2 | -0.8 | -0.8 | -0.2 | -1.2 | -1.2 |
| Class E | 0.0 | -0.2 | -0.4 | 0.0 | -0.3 | -0.5 |
| All | -0.1 | -0.5 | -0.3 | -0.1 | -0.8 | -0.6 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

## LAPS for intra chroma prediction

Table 2 shows the performance of LAPS for intra chroma prediction compared with the HM3.0 anchors. In AI_HE case, the average BD bitrate reductions for U and V components are about 0.3% and 0.3% (up to 0.9% and 0.9% for clip BasketballPass) respectively with about 3% increase in encoding time and almost no change in decoding time. In AI_LC case, the average BD bitrate reduction for U and V components are about 0.2% and 0.2% respectively with about 3% increase in encoding and 3% decrease in decoding time.

Table 2 - The performance of LAPS for intra chroma prediction

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | -0.2 | -0.2 | 0.0 | -0.2 | -0.2 |
| Class C | 0.0 | -0.5 | -0.4 | -0.1 | -0.6 | -0.5 |
| Class D | -0.1 | -0.5 | -0.6 | -0.1 | -0.5 | -0.5 |
| Class E | 0.0 | -0.1 | -0.1 | 0.0 | 0.2 | 0.1 |
| All | 0.0 | -0.3 | -0.3 | 0.0 | -0.2 | -0.2 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

## Combination of 2-D filter for LM mode and LAPS

Table 3 shows the performance of combination of 2-D filter for LM mode and LAPS compared with the HM3.0 anchors. In AI_HE case, the average BD bitrate reductions for U and V components are about 0.7% and 0.6% (up to 4.0% and 2.9% for clip BasketballDrill) respectively with about 3% increase in encoding time and almost no change in decoding time. In AI_LC case, the average BD bitrate reduction for U and V components are about 1.0% and 0.8% respectively with about 3% increase in encoding and almost no change in decoding time.

Table 3 - The performances of the combination of 2D filter for LM mode and LAPS

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.2 | 0.5 | 0.0 | -0.1 | 0.7 |
| Class B | -0.1 | -0.4 | -0.4 | -0.1 | -0.8 | -0.6 |
| Class C | -0.4 | -1.5 | -1.4 | -0.3 | -2.2 | -2.0 |
| Class D | -0.3 | -1.2 | -1.4 | -0.3 | -1.6 | -1.6 |
| Class E | 0.0 | -0.3 | -0.5 | 0.0 | 0.0 | -0.4 |
| All | -0.2 | -0.7 | -0.6 | -0.1 | -1.0 | -0.8 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

# Conclusions

This contribution reports two techniques to improve intra chroma prediction: a 2-dimensional 3x2 filter for LM intra chroma prediction mode, and a new intra chroma prediction mode based on the technique of local adaptive prediction selection (LAPS) process to produce the intra chroma prediction sample from the comparison of the reconstructed luma sample of target location to the local reconstructed luma samples in the neighborhood. The tests of the 2-D filter for LM intra chroma prediction have reported that an BD-bitrate reduction of U & V components by 0.5% & 0.3% for AI_HE case (up to 2.0% & 1.7% for clip PartyScene) with almost no change in encoding and decoding time. The tests of LAPS mode have reported that an BD-bitrate reduction of U & V components by 0.3% & 0.3% for AI_HE case (up to 0.9% & 0.9% for clip BasketballPass) with 3% increase in encoding time and almost no change in decoding time. The tests of the combined two techniques have reported that an BD-bitrate reduction of U & V components by 0.7% & 0.6% for AI_HE case (up to 4.0% & 2.9% for clip BasketballDrill) with 3% increase in encoding time and almost no change in decoding time. Based on the improvement of coding efficiency demonstrated in the tests, the proponents recommend establishing a CE to further study the techniques.

# References

- J. Chen, V. Seregin, W. Han, J. Kim, B.M. Jeon, “CE6.a.4: Chroma intra prediction by reconstructed luma samples”, JCTVC-E266, Geneva, March, 2011.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).