| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F503 |
| --- | --- |

| Title: | Results on Slice Header Modification |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naël OUEDRAOGO Patrice ONNO Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE |  | nael.ouedraogo@crf.canon.fr patrice.onno@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution presents results of some experiments conducted by implementing the modifications around the slice header introduced in JCTVC-E222 at the Geneva HEVC meeting. This technical proposal introduces two main modifications for HEVC. First modification consists in gathering some coding parameters of slice headers in a dedicated NAL unit. Second modification adds some parameters within this new NAL unit in order to introduce new frontiers for reconstruction. Several coding efficiency results targeting a streaming scenario are reported.

# Introduction

During the presentation of JCTVC-E222 in March, it was remarked that moving coding parameters out of the slice header is not always well adapted from an error resilience perspective in streaming scenario. For example, the frame_num syntax element present in the slice header is an important syntax element to repeat when packet losses occur. In addition, it was also mentioned that moving some elements such as the reference picture list reordering, from slice level to picture level would remove the ability to change these elements from slice to slice within the picture.

In this contribution, we have improved our proposal around this new NAL unit to offer more flexibility and solve the above concerns. The main idea is to replicate or not this NAL unit according to the targeted scenario. The new NAL unit has been implemented in HM3.0 and the results presented in that contribution show that this proposal shows benefits for streaming applications compared to HM3.0 with entropy slices while preserving the coding efficiency.

# Proposed modification

The goal of this section is to summarize the two main changes introduced by the concept presented in JCTVC-E222.

## Change 1: Move common slice parameters in a new NAL unit

First slice modification consists in using a new NAL unit to gather some parameters of the current slice headers.

Let’s call this new NAL unit “Slice Common Parameters (SCP) NAL unit” for the ease of the description in this contribution.

The main goal presented in E222 was to reduce redundancy of slice header parameters when several slices are used per frame. In addition, SCP NAL unit enables to control the redundancy level of these parameters which is useful in error prone environment for streaming applications. Indeed, in function of network characteristics, one or several instances of SCP NAL units are sent. For example, if the network characteristics ensure a reliable transmission of NAL units, a single version of SCP NALU is sent. On the other hand, if the network transmission is unreliable, the encoder replicates SCP NALUs for each slice NAL units. Each slice NAL units is then aggregated with one SCP NALU in each network packet. This ensures that any element previously coded in slice header (such as frame_num syntax element) is available when decoding a slice unit.

Consequently, the following properties are defined for SCP NAL units:

- At least one SCP NAL units must be received before decoding one slice unit

- SCP NAL unit that has parameter’s values different to a prior SCP NAL unit must be sent prior to the first NAL unit that uses these new parameter values

- Coding parameter values of one SCP NALU replace all values decoded in a prior SCP NALU.

Since these 3 properties are quite similar to Parameter Sets we have falsely named SCP NALU “Slice Parameter Sets NAL unit” in E222. Indeed, since SCP NAL unit contains parameter such as POC, at least one SCP NAL must be sent for each access unit which was not mandatory in the initial definition of Slice Parameter Sets. Nevertheless, the property #2 imposes that a new SCP NAL unit needs to be sent for each access unit.

Moreover, SCP NAL unit concepts is also suitable for other applications than streaming, typically for an encoder that has the ability to efficiently use reference picture list reordering from slice to slice within one picture. Indeed, several SCP NAL units with different parameter could be sent for a single frame. Each slice using the last received SCP NAL units. A more flexible solution is to use an unique ID for each SCP NALU which is referred to by each slice of the frame. For this particular usage, we think that method presented in JCTVC-E309 could be used to update portion of SCP NAL units instead of sending new complete SCP NAL unit.

Using several SCP NAL units within one frame is advantageous for several applications. For this reason, we think that it cannot be used has a Picture Header.

## Change 2: Indication of reconstruction/filtering dependencies

Second modification proposed in JCTVC-E222 is to indicate slice dependencies and loop filter dependencies in SCP NAL unit. In following experiments, only reconstruction slice dependencies have been defined in SCP NAL unit. Loop filters slices are not considered yet and Loop filter parameters are stored in the first slice.

# Implementation in HM3.0

# Experiments and Results

Two different scenarios are considered in this contribution. First one is related to streaming environment and aims at showing the benefit of the introduction of the new NAL unit concept from an error resilience point of view.

Second experiment compares our proposal with HM3.0 with different configurations.

## Streaming scenario

In streaming environment to limit impact of packet losses during the transmission on the network, NAL unit maximum size is classically set to MTU size (typically 1500 bytes). This results in restricting slices size in order not to exceed 1500 bytes. This ensures that each received packet is decodable. However, this is not ideal in some cases. For example, the bitrate of one encoded video sequence varies in function of the video content or even depends in the targeted bitrate provided by the network. Consequently the number of slices per frames is depending on these fluctuating parameters. It is why it is preferable in some cases to define a fixed number of LCU per slice to ensure a predetermined number of slices per frame. This fixed number of slices per frame is advantageously used especially in error concealment or other processing algorithm.

## HM3.0 with 1500Ko ES and 2 rows of LCU as reference,

In HM3.0, entropy slices enable to define a predetermined number of bytes per NAL unit (equal to MTU size) while defining a predetermined number of LCU per slices.

In the following experiments, we choose to define a fix number of LCUs per slices such that each slice contains 2 rows of LCU within the frame. According to the width of each sequence class, we can define the slice size in terms of LCU as defined in Table 1.

| Class | Slice size in LCU |
| --- | --- |
| A | 80 |
| B | 60 |
| C | 26 |
| D | 13 |
| E | 40 |

Table 1: Slice in size in function of tested class

To sum up our reference is HM3.0 software with a fixed number of 1500 bytes per entropy slice and predetermined number of LCU per slice in function of the Class as defined in Table 1.

## HM3.0 modifications regarding our proposal

For this contribution, we have implemented the SCP NAL unit concept in HM3.0 as follows: SCP NAL unit gathers all coding parameters defined in slice headers except the QP parameter. Conceptually, QP could also be moved in SCP NAL unit but since it may significantly vary from slice to slice, there is no benefit to define it in SCP and it would be a bit consuming when replicating SCP NAL units.

In our proposal, a new Slice mode syntax element is defined in Picture Parameters Set. It indicates that a fixed number of LCU is defined per slice. The number of LCU per slice is defined in slice_address_offset syntax element of SCP NAL unit. Entropy slice size is limited to 1500 bytes.

Two experiments have been done to show the benefit of SCP NAL units in HM3.0.

- First experiment consists in sending a single SCP NAL unit per access unit. As presented in previous section, this experiment is dedicated to reliable network transmission since it permits to minimize the redundancy of slice header coding parameters.

- The second experiment is dedicated to unreliable network for which slice header coding parameters are redundant to ensure a proper decoding of slice units in case of packet loss. Thus, SCP NAL units are duplicated to match the number of entropy slices encoded per frame.

### Experiment 1: Single SCP NAL unit per frame

The table below represents the coding gain obtained with the proposal against HM3.0 with  a fixed number of 1500 bytes per entropy slice and predetermined number of LCU per slice in function of the Class as defined in Table 1.

Table 2: Exp-1, Coding efficiency for Random Access and Low Delay and Intra

The gain observed is due mostly to the fact that HM3.0 encodes a new slice NAL unit when encoding the first LCU of the corresponding slice. In our proposal, since slices boundaries are indicated in SCP NAL unit, no new NAL unit is created and thus bits (overhead of new slice NAL units signaling) are saved.

Second reason of gain is bits saved by removing redundancy of parameters coded in slice headers.

### Experiment 2: Multiple SCP NAL unit per frame

Table 3: Exp-2, Coding efficiency for Random Access and Low Delay

In comparison to experiment 1, the gain is lower due to the overhead introduced by the multiple SCP NAL units. In particular for Intra configuration, bitrate is high and the number of 1500 bytes entropy slices defined per frame is important. Thus, the number of duplicated SCP NAL units is also high and the bitrate increases. For RA and LD configurations, there is still some gain since the bitrate per frame is lower: only a few entropy slices are needed per frame. For instance, for Class E less than 1500 bytes is needed per frame. A single entropy slice is used and also a single SCP NAL unit is needed.

### Advantages

Advantages described in JCTVC-E222 are still valid. In addition, SCP NAL unit permits to adapt the redundancy level of coding parameters defined in slice headers in function of the network characteristics. For instance, a single SLPS can be used for network with forward error correction (thus redundancy level is adjusted by FEC) and for network without forward error correction capabilities,  same redundancy level as currently used in HM3.0 can be defined by using Multiple SCP NAL units. Intermediate level of redundancy can be used in function of the network error rate.

Slice dependencies are defined in SCP NAL unit, thus it is not necessary to start a new NAL unit at each slice boundary as done in HM3.0. As result, entropy slice NAL units generated with our proposal are closer to MTU size target. For streaming, network packet payload is better utilized with our proposal.

## Comparison with HM3.0 anchors

For this experiment, common conditions are used [1].  Only one slice per frame is used and no entropy slices are used.

Table 4: Coding efficiency for Intra Only Profile

Table 4 shows the performance of our proposal when comparing our proposal to HM3.0 anchors results. The losses reported were predictable and are mainly due to the overhead introduced by the signaling of SCP NAL units. This is not a surprise since the benefit of SCP NAL units is mostly for streaming application or other dedicated application. One solution to avoid this small coding loss is to use one flag that could be defined in Sequence Parameter Sets to disable the usage of SCP NAL units for some cases  where coding efficiency is more important than the advantages  provided by SCP NAL units.

## Remarks

In above experiments, only a fixed number of LCU is enforced per slice NAL units but other slice modes could be defined and specified in bit stream by slice_mode syntax element of Picture Parameter Sets: For instance the following modes can be considered:

- One slice in the frame

- Fix number of LCUs per slice

- Match slice boundaries to entropy slices

- Explicit slice partition

For each mode, entropy slices are defined independently of the slices. The first mode is used when a single reconstruction slice is use per frame. This mode could disable the usage of SCP NAL units.  Second mode is the one used for Experiment 1 and Experiment 2. The number of LCUs per Slice is defined in SCP NAL unit. Third mode is used to match slice boundaries with entropy slices boundaries. This is equivalent to H.264/AVC slice definition. In that case, slice dependencies are inferred from entropy slice start and end address. Last mode is explicit slice partition. In that case, start address of each slice is indicated in SCP NAL unit.

These 4 slice partitioning modes permit to partition video frames for many applications from streaming up to object segmentation within frame etc.

# Syntax changes

We propose the following syntax changes for our contribution: syntax of slices has been modified as following.

| slice_header( ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && frame_type != I) |  |
| cabac_init_idc | ue(v) |
| first_slice_in_pic_flag | u(1) |
| if( first_slice_in_pic_flag == 0 ) |  |
| slice_address | u(v) |
| slice_qp_delta | se(v) |
| } |  |

Syntax of Picture Parameter Set:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| seq_parameter_set_id | ue(v) |
| entropy_coding_mode_flag | u(1) |
| num_temporal_layer_switching_point_flags | ue(v) |
| for( i = 0; i < num_temporal_layer_switching_point_flags; i++ ) |  |
| temporal_layer_switching_point_flag[ i ] | u(1) |
| num_ref_idx_l0_default_active_minus1 | ue(v) |
| num_ref_idx_l1_default_active_minus1 | ue(v) |
| pic_init_qp_minus26 /* relative to 26 */ | se(v) |
| constrained_intra_pred_flag | u(1) |
| slice_granularity | u(2) |
| shared_pps_info_enabled_flag | u(1) |
| if( shared_pps_info_enabled_flag ) |  |
| if( adaptive_loop_filter_enabled_flag ) |  |
| alf_param( ) |  |
| if( cu_qp_delta_enabled_flag ) |  |
| max_cu_qp_delta_depth | u(4) |
| slice_mode | u(2) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics of new syntax elements which are different than in previous definition of PPS syntax is described below:

slice_mode specifies the partitioning method of slice partition. Below are the different possible values:

- One slice per the frame

- Fix number of LCUs per slice

- Match slice boundaries to entropy slices

- Explicit slice partition

Below is the syntax of the proposed Slice Common Parameters NAL unit that defines reconstruction slices s boundaries. It includes several syntax elements previously defined in slice header syntax.

| slice_common_parameter_rbsp( ) { | Descriptor |
| --- | --- |
| frame_type | ue(v) |
| pic_parameter_set_id | ue(v) |
| frame_num | u(v) |
| if( IdrPicFlag ) |  |
| idr_pic_id | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| pic_order_cnt_lsb /* | u(v) |
| if( slice_type = = P ¦ ¦ slice_type = = B ) { |  |
| num_ref_idx_active_override_flag | u(1) |
| if( num_ref_idx_active_override_flag ) { |  |
| num_ref_idx_l0_active_minus1 | ue(v) |
| if( slice_type = = B ) |  |
| num_ref_idx_l1_active_minus1 | ue(v) |
| } |  |
| } |  |
| ref_pic_list_modification( ) |  |
| ref_pic_list_combination( ) |  |
| if( nal_ref_idc != 0 ) |  |
| dec_ref_pic_marking( ) |  |
| if( sample_adaptive_offset_enabled_flag ) |  |
| sao_param() |  |
| if( deblocking_filter_control_present_flag ) { |  |
| disable_deblocking_filter_idc |  |
| if( disable_deblocking_filter_idc != 1 ) { |  |
| slice_alpha_c0_offset_div2 |  |
| slice_beta_offset_div2 |  |
| } |  |
| if( slice_type = = B ) |  |
| collocated_from_l0_flag | u(1) |
| if( adaptive_loop_filter_enabled_flag ) { |  |
| if( !shared_pps_info_enabled_flag ) |  |
| alf_param( ) |  |
| alf_cu_control_param( ) |  |
| } |  |
| if (slice_mode == 1) |  |
| slice_address_offset | u(v) |
| If (slice_mode == 4) { |  |
| num_recons_slices_minus2 | u(v) |
| for ( i = 0; i < num_recons_slices_minus2; i++) |  |
| recons_slice_ index_offset[i] | u(v) |
| } |  |

The semantics of new syntax elements which are different than in previous slice header definition is described below:

frame_type specifies the coding type of the frame. For each slice, the value of slice_type is equal to frame_type. Coding type of slice_type is specified in Table 7.3 of [3].

slice_address_offset specifies the address offset value in slice granularity resolution between each beginning of slice.

num_recons_slices_minus2 specifies the number of reconstruction slices within  a picture for a picture. The num_recons_slices value should be greater than 2.

# Conclusion

Slices modification proposed in this contribution enables a flexible and independent partitioning of HEVC frames. The principle of defining slice dependencies boundaries in the proposed Slice Common Parameters NAL unit presents numerous advantages for streaming application: coding gain is obtained and generated NAL units are better utilizing network packet payload.

Furthermore, the grouping of slice header parameters in one single Slice Parameter Sets improves the coding efficiency and allows adapting robustness of the HEVC streams to the network characteristics.

Considering the numerous advantages of the proposal for streaming, we propose to further study this proposition in Slice or Error Resilience Ad-Hoc Group or in a CE to further evaluate this technology.

# References

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting: Geneva, Switzerland, March, 2011

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).