| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F504 |
| --- | --- |

| Title: | Cross-checking of experiment “3Merge2AMVP” of JCTVC-F402 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Guillaume LAROCHE Tangi POIRIER Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | guillaume.laroche@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This document is a crosscheck report of the experiment “3Merge2AMVP” of the technical proposal JCTVC-F402, version 1 [1]. This contribution proposes to solve the parsing issue robustness with a fixed list of candidates for both AMVP and Merge modes. In this experiment, the AVMP list contains 2 candidates and the Merge list 3 candidates. It is reported in this document that the bitrate and PSNR match those provided by the proponent but the proposed implementation doesn’t solve the parsing issue robustness. The proponent is aware of it and will update its software in order to fully solve the parsing issue.

- Code verification

Samsung has provided a modified source code based on HM3.0. The source code has been analyzed.

The source code was compiled successfully by using windows32, VC2008 and by using Linux 2.6.18-194.el5,  gcc version 4.1.2 20080704 (Red Hat 4.1.2-48) 64 bits.

# Parsing Check

The aim of the experiment “3Merge2AMVP” is to solve the parsing issue robustness of the Motion vector predictor index. So, we checked the robustness of this experiment. In the “3Merge2AMVP” software, we implemented our parsing check macro which was provided to CE9 participants with our software [2]. At the decoder side, for each frame of the sequence, all reference frames are re-initialized. It means that all data for all CUs of the previous frame are initialized (CU partitioning, mode, reference frames indexes, directions, motion vectors, etc.). Please note that the slice header is also lost, except the POC number. Indeed, when the POC of the reference frame is lost, the same parsing issue occurs in HEVC as in H.264/AVC. In an error resilient implementation, this issue can be solved easily.

For all bit streams tested, the decoder crashed. We analyzed the software and we found that it comes from the use of the reordering of Merge candidates based on the motion vector value. Indeed, when an error occurs, the wrong reordering changes the selected predictor and consequently, the direction and the reference frame index of a Merge candidate could be wrong. The direction and the reference index of the Merge candidate are used to determine the contexts in entropy coding.

The proponent agreed on this and will update its software in the following days to fix this. A new crosscheck is foreseen on the new software.

# Results

Tests were done according to the tests description provided in [3] for RAHE, RALC LBHE and LBLC configurations. The corresponding Excel file is attached. The results perfectly match those generated by Samsung.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.5 | 0.4 | 0.0 | 0.4 | 0.3 | 0.3 |
| Class B | 0.5 | 0.1 | -0.1 | 0.5 | 0.3 | 0.4 |
| Class C | 0.4 | 0.2 | 0.2 | 0.3 | 0.2 | 0.3 |
| Class D | 0.5 | 0.1 | 0.0 | 0.3 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.5 | 0.2 | 0.0 | 0.4 | 0.2 | 0.3 |
| Enc Time[%] | 104% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | -0.3 | -0.8 | 0.5 | 0.6 | 0.5 |
| Class C | 0.5 | -0.1 | -0.2 | 0.3 | 0.2 | 0.1 |
| Class D | 0.4 | -0.6 | 0.2 | 0.3 | 0.1 | 0.2 |
| Class E | 1.6 | 0.9 | 1.1 | 2.0 | 1.9 | 2.2 |
| Overall | 0.6 | -0.1 | 0.0 | 0.7 | 0.6 | 0.6 |
| Enc Time[%] | 103% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusion

The results of experiment “3Merge2AMVP” presented by Samsung in JCTVC-F402, version 1 are confirmed but the software doesn’t solve the parsing issue. Samsung agreed on the parsing problem and will update its software in the following days. Canon will crosscheck it again and will update this cross-check document accordingly.

# References

- Jianle Chen, Tammy Lee, “MVP index parsing with fixed number of candidates”, JCTVC-F402, 6th Meeting: Torino, IT, 14-22 July, 2011.

- G. Laroche, P. Onno, T. Poirier, C. Gisquet, “CE9: Description of experiments ROB01 and ROB02”, JCTVC-F474, 6th Meeting: Torino, IT, 14-22 July, 2011.

- F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, 5th Meeting: Geneva, CH, 16-23 March, 2011.