| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F505 |
| --- | --- |

| Title: | A chroma coding scheme for SDIP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jin Song, Haitao Yang, Mingyuan Yang, Haoping Yu Huawei Changcai Lai Hisilicon Jizheng Xu Microsoft | Tel: Email: | +86 10 82882583 andy.songjin@huawei.com haitao.yang@huawei.com yangmingyuan@huawei.com haoping.yu@huawei.com laichangcai@huawei.com jzxu@microsoft.com |
| Source: | Huawei, Hisilicon, Microsoft |  |  |

_____________________________

# Abstract

In this contribution, a chroma coding scheme for SDIP (Short Distance Intra Prediction) mode is proposed. In the proposed scheme a new chroma prediction mode called SDIP direct mode is introduced. When the SDIP direct mode is selected, chroma component shares the same PU partition and prediction modes of luma component. Therefore no overhead is transmitted to signal the chroma PU mode and the prediction mode of each PU. The proposed scheme is reported to provide U/V component BD-rate of -0.9%/-0.9% and -1.5%/-1.3% under AI_HE and AI_LC conditions, while the performance of Y component is not affected.

# Introduction

In SDIP software, SDIP introduces non-square PU for intra luma prediction. To make full use of the prediction modes of four luma partitions, all the different luma partition prediction modes are added into the candidate mode set for chroma prediction in the current SDIP anchor implementation.

However, some less frequently used luma prediction mode may not be a good candidate mode for chroma prediction. For example, if the prediction mode is only used by one luma partition, it is probably not an optimum prediction mode for the chroma component. So we propose a method to get rid of the less frequently used luma prediction modes from the candidate mode set for chroma component.

Furthermore, new chroma prediction mode, SDM (SDIP Direct Mode) is proposed to better exploit the correlations between luma and chroma when SDIP mode is selected.

# Proposed Scheme

The proposed scheme includes three parts:

Part1: Assign LM with the shortest codeword in SDIP case, which is the same as the method in JCTVC-F110.

Part2: Introduce a new prediction mode, SDM (SDIP Direct Mode) as a candidate mode for chroma component in SDIP case.

Part3: Prune candidate mode set.

## SDIP Direct Mode (SDM)

When SDM is selected, the chroma component share the same PU partition of the luma component, and the prediction mode of each chroma partition is the same as the mode of the corresponding luma partition.

Figure 1 Illustration of SDM mode

## Prune candidate mode set

Luma prediction mode used by only one of the four SDIP partitions inside a CU is excluded from the candidate mode set of chroma component.

For clear description, LumaModeNum is used to denote the number of different prediction modes of the four SDIP luma partitions. LumaModeNum equals 1 means that all four SDIP luma partitions use the same prediction mode. LumaModeNum equals 4 means that the prediction modes of the four SDIP luma partitions are different from each other. LumaMode(x) denotes the prediction mode that is selected by x out of the four luma partitions.

With the above definition of LumaModeNum and LumaMode(x), the proposed prune operation can be simply described as: remove all LumaMode(1) from the candidate mode list.

## Codeword assignment for all possible candidate mode set

All possible candidate chroma mode sets and corresponding codeword assignment are shown in Table 1.

Table 1-1 Codeword assignment when LumaModeNum is 1

| Candidiate chroma prediction modes | Codeword |
| --- | --- |
| LM | 0 |
| LumaMode(4) | 10 |
| SDM | 110 |
| DC/planar | 111 |

Table 1-2 Codeword assignment when LumaModeNum is 2

| Candidiate chroma prediction modes | Codeword |
| --- | --- |
| LM | 0 |
| LumaMode(2)-1 | 10 |
| LumaMode(2)-2 | 110 |
| SDM | 1110 |
| DC/planar | 1111 |

or

| Candidiate chroma prediction modes | Codeword |
| --- | --- |
| LM | 0 |
| LumaMode(3) | 10 |
| SDM | 110 |
| DC/planar | 111 |

Table 1-3 Codeword assignment when LumaModeNum is 3

| Candidiate chroma prediction modes | Codeword |
| --- | --- |
| LM | 0 |
| LumaMode(2) | 10 |
| SDM | 110 |
| DC/planar | 111 |

Table 1-4 Codeword assignment when LumaModeNum is 4

| Candidiate chroma prediction modes | Codeword |
| --- | --- |
| LM | 0 |
| SDM | 10 |
| Vertical | 110 |
| Horizontal | 1110 |
| DC/planar | 1111 |

# Simulations & Results

The proposed chroma coding scheme for SDIP is implemented on top of HM3.0-sdip (r866). Simulations are conducted based on the JCT-VC common test condition defined in Error: Reference source not found. Table 2 shows the performance of the SDM technique (only Part2 enabled). Table 3 shows the performance of the proposed scheme (all the three parts).

Table 2. RD performance of the SDM technique

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | -1.4 | -0.8 |
| Class B | 0.0 | -1.1 | -0.9 | 0.1 | -1.7 | -1.7 |
| Class C | 0.0 | -0.2 | -0.3 | 0.1 | -0.8 | -0.8 |
| Class D | 0.0 | -0.3 | -0.3 | 0.1 | -0.8 | -0.6 |
| Class E | 0.0 | -0.8 | -0.7 | 0.1 | -1.6 | -1.4 |
| Overall | 0.0 | -0.5 | -0.5 | 0.1 | -1.3 | -1.1 |

Table 3. RD performance of the proposed whole scheme

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.3 | -0.2 | 0.0 | -1.6 | -1.0 |
| Class B | 0.0 | -1.4 | -1.3 | 0.1 | -2.0 | -1.9 |
| Class C | -0.1 | -0.8 | -0.7 | 0.0 | -1.0 | -1.0 |
| Class D | -0.1 | -0.8 | -0.7 | 0.0 | -0.9 | -0.8 |
| Class E | 0.1 | -1.4 | -1.5 | 0.1 | -2.0 | -1.8 |
| Overall | 0.0 | -0.9 | -0.9 | 0.0 | -1.5 | -1.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | -0.8 | -0.2 | 0.0 | -2.0 | -1.0 |
| Class B | 0.0 | -1.4 | -1.0 | 0.0 | -2.0 | -1.8 |
| Class C | -0.1 | -0.6 | -0.4 | 0.0 | -1.1 | -1.0 |
| Class D | 0.0 | -0.3 | -0.5 | 0.1 | -0.8 | -0.6 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | -0.8 | -0.5 | 0.0 | -1.5 | -1.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.4 | 0.0 | 0.0 | -1.0 | -0.7 |
| Class C | -0.1 | -0.2 | -0.5 | 0.0 | -0.4 | -0.5 |
| Class D | 0.0 | 0.3 | -0.2 | 0.0 | -0.5 | -0.7 |
| Class E | -0.1 | -2.8 | -2.6 | -0.1 | -3.2 | -1.2 |
| Overall | 0.0 | -0.6 | -0.7 | 0.0 | -1.1 | -0.7 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

# Conclusion

In this contribution, a chroma coding scheme for SDIP is proposed. The chroma coding performance of SDIP is improved and no complexity is introduced by the scheme. We recommend adopting this method in next version of SDIP software.

# Reference

- F. Bossen, "Common test conditions and software reference configurations," JCT-VC document JCTVC-E700, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP3 and ISO/IEC JTC1/SC29/WG11 5th meeting, Geneva, CH, 16-23 March, 2011

- Y. Lin, C. Lai, J. Zheng, L. Liu, “CE6 Subset A: Report of Improved Intra prediction for positive directions in UDI”, Doc. JCTVC-E286, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011.

# Patent rights declaration(s)

Huawei, Hisilicon and Microsoft may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed changes to WD text

8.3.2	Derivation process for chroma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–  a variable log2TrafoSize specifying the size of the current coding unit.

Output of this process is the variable IntraPredModeC.

Depending on PartMode, the chroma intra prediction mode IntraPredModeC is derived as follows:

–  If PartMode is equal to PART_2Nx2N or PART_NxN, the chroma intra prediction mode IntraPredModeC is derived as specifed in Table 8-1 or Table 8-2 with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

IntraPredModeC is further modified to be equal to Intra_Planar when IntraPredModeC is equal to Intra_DC andplanar_flag_chroma is equal to 1.

Table 8-1 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 1

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | 35 | 35 | 35 | 35 | 35 |
| 1 | n/a | 0 | 0 | 0 | 0 |
| 2 | 1 | n/a | 1 | 1 | 1 |
| 3 | 2 | 2 | 2 | 2 | 2 |
| 4 | 0 | 1 | 2 | 3 | X |

Table 8-2 – Specification of IntraPredModeC according to the values of intra_chroma_pred_mode and IntraPredMode[ xB ][ yB ] when chroma_pred_from_luma_enabled_flag is equal to 0

| intra_chroma_pred_mode | IntraPredMode[ xB ][ yB ] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | X ( 4 <= X < 35 ) |
| 0 | n/a | 0 | 0 | 0 | 0 |
| 1 | 1 | n/a | 1 | 1 | 1 |
| 2 | 2 | 2 | 2 | 2 | 2 |
| 3 | 0 | 1 | 2 | 3 | X |

–  Otherwise (PartMode is equal to PART_2NxhN or PART_hNx2N), IntraPredModeC is derived by following procedures with intra_chroma_pred_mode and the four intra prediction modes of the current coding unit as inputs.

- The variables xOff, yOff are set as follows:

if(PartMode == PART_2NxhN)

{

xOff = 0;

yOff = (1<< log2TrafoSize)>>2;

}

else if(PartMode == PART_ hNx2N)

{

xOff = (1<< log2TrafoSize)>>2;

yOff = 0;

}

- The mapping table mapIntraPredModeC is derived as follows.

IntraPredModeCnt[35] = {0};

for(blkIdx = 0; blkIdx < 35; blkIdx++)

IntraPredModeCnt[ IntraPredMode[ xB+xOff*blkIdx][yB+yOff*blkIdx] ]++;

j = 0;

sort = 0;

mapIntraPredModeC[j++] = 3;

for( n = 0; n < 35; n++ )

{

if( IntraPredModeCnt[n] >1)

{

mapIntraPredModeC[j++] = n;

sort++;

}

}

mapIntraPredModeC[j++] = 4;

for( n = 0; n < 3; n++ )

{

if(IntraPredModeCnt[n] == 0 )

{

if(sort == 0)

mapIntraPredModeC[j++] = n;

else if(n = 2)

mapIntraPredModeC[j++] = n;

}

}

- IntraPredModeC is then derived by

IntraPredModeC = mapIntraPredModeC[ intra_chroma_pred_mode ].

8.3.3.1.9	Specification of SDM prediction mode

Inputs to this process are:

–	a sample location ( xB, yB ) specifying the top-left sample of the current block relative to the top-left sample of the current picture,

–	one variable nS specifying the size of the current coding unit

Output of this process is:

–	predicted samples predSamples[ x, y ], with x = 0.. nS-1, y = 0.. nS-1.

This intra prediction mode is invoked when intraPredModeC is equal to 4.

Two variables nW and nH are derived as follows:

–	if PartMode is equal to PART_2NxN, nW = nS, nH = (nS>>2)

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB ][ yB ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. nW-1, y = 0.. nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB+nH ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB ][ yB+nH ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. nW-1, y = nH..2* nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB+2*nH ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB ][ yB+2*nH ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. nW-1, y = 2*nH.. 3*nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB+3*nH ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB ][ yB+3*nH ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. nW-1, y = 3*nH.. 4*nH-1.

–	else if PartMode is equal to PART_Nx2N, nW = (nS>>2), nH = nS

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB, yB ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB ][ yB ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. nW-1, y = 0.. nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB+nW, yB ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB+nW ][ yB ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = nW..2* nW-1, y = 0.. nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB+2*nW, yB ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB+2*nW ][ yB ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 2*nW.. 3*nW-1, y = 0.. nH-1.

- The decoding process for intra blocks as specified in subclause Error: Reference source not found is invoked with the luma location ( xB+3*nW, yB ), the variables log2TrafoWidth set equal to log2(nW) and log2TrafoHeight set equal to log2(nH), the variable trafoDepth set equal to 0, the prediction partition mode PartMode, the luma intra prediction mode IntraPredMode[ xB+3*nW ][ yB ] and the variable cIdx set equal to 1 or 2 as the inputs and the output are predicted samples predSamples[ x, y ], with x = 0.. 3*nW.. 4*nW-1, y = nH-1.