| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F506 |
| --- | --- |

| Title: | CE6.b: Harmonization of SDIP and MDDT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Jiantong Zhou Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China Changcai Lai Section B2, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-28788991 haitao.yang@huawei.com zhoujiantong@huawei.com haoping.yu@huawei.com laichangcai@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This is a report of the harmonization of SDIP (Short Distance Intra Prediction) and MDDT (Mode Dependent Directional Transform). MDDT denotes the mode-dependent DCT/DST in HM3.0. Firstly, Interaction between SDIP and MDDT is tested. Then the harmonization of the two tools described in the CE document is tested. Additional results of the harmonization of SDIP and 8x8 MDDT is also provided in this document for reference.

# Harmonization of SDIP and MDDT

As an implementation of MDDT, the mode-dependent DCT/DST [1] is applied to 4x4 square TUs in HM3.0. Since SDIP introduces non-square TUs for Intra prediction [2], a harmonization solution is necessary for using MDDT in SDIP. For the purpose of harmonization, MDDT is applied as either the row transform or the column transform if the dimension of the non-square TU in row or column direction equals 4. The MDDT for non-square TUs share the same mapping table of MDDT for square TUs when choosing the transform mode according to the intra prediction. As the existing MDDT, the harmonization solution applies MDDT to only the luma component.

As an optional test described in the CE document [3], the 8x8 MDDT is also implemented for both square and non-square TUs in the same manner as 4x4 MDDT. The 8-point DST matrix is computed from the same equation as 4-point DST. The RD performance is provided for reference.

# Simulation

The proposed harmonization is implemented in the released SDIP software with revision number 866, which can be downloaded from the SDIP branch in the SVN server. The SDIP anchor data is obtained using software without modification. And the HM anchor data is obtained using the software with SDIP function switched off.

Intra-HE and Intra LC configurations specified in the common test condition [4] is used for the simulation.

## Interaction between SDIP and MDDT

Three cases are tested to observe the interaction between SDIP and 4x4 MDDT in HM3.0.

- Test case 1: SDIP OFF, MDDT_4x4 OFF 
The RD data of this test case is compared with HM3.0 anchor data to see the performance of MDDT in HM3.0.

- Test case 2: SDIP ON, MDDT_4x4 OFF
The RD data of this test case is compared with SDIP anchor data to see the performance of MDDT on top of SDIP.

- Test case 3: SDIP ON, MDDT_4x4 ON
This test case is actually the SDIP anchor. The SDIP anchor data is compared with HM3.0 anchor data to see the performance of SDIP on top of MDDT.

Table-1 RD performance of test case 1

Table-2 RD performance of test case 2

Table-3 RD performance of test case 3

## Harmonization of SDIP and MDDT

One case is tested to observe the performance of the harmonization solution compared with simple combination of the two tools.

- Test case 4: SDIP ON, MDDT_4x4 ON, HARMONIZATION_4x4 ON
The RD data of this test case is compared with SDIP anchor data to see the performance of the harmonization solution.

We can see from Table-4 that the harmonization solution can bring consistent coding gains.

Table-4 RD performance of test case 4

## Additional results with 8x8 MDDT

In the following two test cases, the 4x4 MDDT and its harmonization with SDIP are turned on by default.

- Test case 5: SDIP OFF, MDDT_8x8 ON 
The RD data of this test case is compared with HM anchor data to see the performance of additional 8x8 MDDT for square TUs.

- Test case 6: SDIP ON, MDDT_8x8 ON, HARMONIZATION_8x8 ON
The RD data of this test case is compared with test case 4 to see the performance of additional 8x8 MDDT for both square and non-square TUs.

Table-5 RD performance of test case 5

Table-6 RD performance of test case 6

# Conclusion

A straightforward harmonization solution of SDIP and 4x4 MDDT is described in this contribution. Consistent coding gains are observed for all test sequences under Intra-HE and Intra-LC test condition. We suggest adopting the harmonization solution along with SDIP.

# References

- Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” ITU-T JCTVC-E125, Geneva, CH, March 2011.

- X. Cao, X. Peng, C. Lai (HiSilicon), et.al., “CE6.b1 Report on Short Distance Intra Prediction Method,” ITU-T JCTVC-E278, Geneva, CH, March 2011.

- M. Budagavi, K. Chono, R. Joshi, et.al., “Core Experiment 6: Intra Prediction Improvement,” ITU-T JCTVC-E706, Geneva, CH, March 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T JCTVC-E700, Geneva, CH, March 2011.

# Patent rights declaration(s)

Huawei may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Proposed changes to WD text

Please refer to section 8.5.4 in the modified WD text in JCTVC-F532.