| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F508 |
| --- | --- |

| Title: | CE4: Cross-check of NEC’s proposal JCTVC-F103 by Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-28788991 haitao.yang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for the proposal JCTVC-F103 performed by Huawei in CE4 activity. The software implementation is consistent with the algorithm description. The RD data we obtained match the results from proponent.

# Cross-verification

## Code-level check

The proposed QP prediction method derives the QP predictor from the temporal reference frames based on the motion information of the top-left PU the current quantization unit.

If the top-left PU is uni-directional predicted, the QP of the quantization unit containing the reference block is used to derive the QP predictor. If the top-left PU is bi-directional predicted, the average of the two QPs obtained from the forward and backward reference frames is used to derive the QP predictor. When deriving the QP predictor, the QP difference between the current slice and the reference slice is taken into account.

## Test condition

The test conditions specified in the JCTVC-E704 are used for the simulation. VS2005 x64 compiler is used to generate the executable file from the provided source code. Simulations were done on computer cluster running Windows Server 2008 x64 OS.

## Test results

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by proponents are exactly the same as the results we obtained. The encoding and decoding time we obtained is similar to that from proponents. Detailed RD data can be found in the attached excel sheet.