| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F509 |
| --- | --- |

| Title: | CE6.a: Report of Bidirectional UDI mode for Intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Haitao Yang, Changcai Lai, Jianhua Zheng, Lingzhi Liu | Tel: Email: | +86-10-82836279 yblin@huawei.com haitao.yang@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the results of CE6.a, Bidirectional UDI mode (BUDI) for Intra prediction. Both LUT-based and LUT-free BUDI techniques have been implemented on top of HM3.0 and HM3.0-SDIP branch software. Two test categories have been verified: (1) BUDI with replacement of Intra prediction mode 6; (2) harmonized signaling on Intra prediction modes of DC, BUID and Planar, with modes of 2, 6 and 9.

For the test category (1), it is reported that 0.2% and 0.1% bitrate reduction for all intra configurations on top of HM3.0 can be achieved for LUT-based and LUT-free BUDI techniques, respectively, with approximately the same encoding and decoding time. Very similar performance is observed for BUDI on top of HM3.0-SDIP branch software.

For the test category (2), 4 combinations of the harmonized Intra prediction mode signaling for DC, BUDI and Planar are tested. It is reported that the harmonized Intra mode signaling for BUDI, DC and Planar further improves coding efficiency, while reducing 3-5% encoding time. Among of them, the combination of 2.a, i.e. Intra prediction mode signaling for DC, BUDI and Planar with 2, 6 and 9 respectively, has better trade-off in terms of performance in Luma and Chroma components. The harmonized methods are also able to solve the parsing dependency problem of Planar mode signaling because they do not require the knowledge of decoded intra prediction modes when deciding whether to decode an additional bit or not of mode 2.

Considering the increased performance, the reduced complexity and better harmonized signaling on Intra prediction mode, we recommend the inclusion of the combination 2.a into HM test model.

# Introduction

Bidirectional UDI mode (BUDI) for Intra prediction has been discussed in the 5th JCT-VC meeting, and was suggested to investigate LUT-based and LUT-free BUDI for further consideration [1-2]. This document presents the progress of BUDI integrations on top of HM3.0 and HM3.0-SDIP branch software, and reports the corresponding results according to CE6 document JCTVC-E607.

# Algorithm description

Fig. 1 Intra prediction mode 6 and mode 9 in HM

As shown in Fig.1, it is observed that both Intra prediction mode 6 and mode 9 in current HM are used to predict the same texture zone where the texture direction is 45 degrees. This observation leads to the design of a unified Intra prediction for the specific texture direction of 45 degrees. BUDI mode is proposed to unify Intra prediction mode 6 and mode 9 by using linear interpolation from above and left reference samples. As also shown in Fig 1, the prediction value of P(x, y) is calculated with equation (1):

P’(x, y) = w1*ref1 + w2*ref2					(1)

Where, P’ is the prediction value of sample P, w1 and w2 are weighting coefficients, which depends on the spatial distances from P to ref1 and ref2 samples.

w1 = (x+1) / (y+x+2)

w2 = (y+1) / (y+x+2)

The calculation of the weighting coefficients involve division operation, which can be implemented by using a pre-defined LUT. This implementation is called LUT-based BUDI.

In addition, the calculation of the weighting coefficients can be approximately simplified as follows. This technique further removes LUT, and is marked as LUT-free BUDI.

w1 = ( nS + (x-y) ) / (2*nS)

w2 = ( nS - (x-y) ) / (2*nS)

where nS denotes the size of current block, the division by 2*ns can be converted to shift operation. In the above equations, (x-y) denotes the deviation of the current pixel from the equal-weight pixels along the diagonal line of the current block. (nS - (x-y)) and (nS + (x-y)) approximate the distance from the current pixel to the top and left reference pixel, respectively.

As for the signaling on Intra prediction mode, BUDI can be used to replace either mode 6 or mode 9. Furthermore, since BUDI mode is designed to unify the intra prediction mode 6 and mode 9, either mode6 or mode9 will be remained and can be further used by other Intra prediction, e.g. DC or Planar prediction, to achieve better harmonization on Intra mode signaling since no additional flag is required to identify DC or Planar mode. Another advantage of this harmonization is that the parsing dependency problem of Planar mode signaling can be solved by giving Planar mode an individual number. All the methods do not need to know the decoded intra prediction modes when parsing, while in HM3.to decide whether to decode an additional bit or not, the decoder needs to know if the mode is 2 or not [3].

# Experimental results

Both the proposed LUT-based and LUT-free BUDI techniques have been integrated on the top of HM3.0 and HM3.0-SDIP branch software according to the test plan in CE6 document JCTVC-E607 [4-5].

Two test categories were set up: (1) BUDI with replacement of Intra prediction mode 6; (2) harmonized signaling on Intra prediction modes of DC, BUID and Planar, with modes of 2, 6 and 9.

For the test category (1), the R-D performance of BUDI on top of HM3.0 is shown in Table 1 and 2. It is shown that 0.2% and 0.1% bitrate reduction can be achieved for LUT-based BUDI and LUT-free BUDI on top of HM3.0, respectively, with approximately the same encoding and decoding time. Very similar performance can be seen for BUDI on top of HM3.0-SDIP branch software, as shown in Table 3 and 4.

Table 1 Performance of LUT-based BUDI on top of HM3.0 (refer to 1_CE6a_HM30.xls)

Table 2 Performance of LUT-free BUDI on top of HM3.0 (refer to 2_CE6a_HM30.xls)

Table 3 Performance of LUT-based BUDI on top of HM3.0-SDIP (refer to 1_CE6a_HM30_SDIP.xls)

Table 4 Performance of LUT-based BUDI on top of HM3.0-SDIP (refer to 2_CE6a_HM30_SDIP.xls)

For the test category (2), there are 4 combinations of Intra prediction mode signaling for DC, BUDI and Planar:

- 2.a : (DC, BUDI, Planar) = (2, 6, 9), which means Intra prediction mode signaling for DC, BUDI and Planar with 2, 6 and 9, respectively.

- 2.b : (DC, BUDI, Planar) = (9, 6, 2);

- 2.c : (DC, BUDI, Planar) = (2, 9, 6);

- 2.d : (DC, BUDI, Planar) = (6, 9, 2);

Table 5 and Table 6 summarize the R-D performance of above combinations for LUT-based and LUT-free BUDI on top of HM3.0, respectively. The corresponding excel sheet for these combinations are listed in appendix. These results show that

- All combinations improve coding efficiency while reducing the encoding time by 3-5%

- 2.a variant of LUT-based BUDI achieves 0.17% and 0.23% bit-rate reduction for AI-HE and AI-LC configurations, while 0.12% and 0.19% bit-rate reduction is obtained for that variant of LUT-fee BUDI. The very similar performance for 2.c variant can be also observed

- 2.b variant obtains slightly better coding efficiency than the other 3 variants according to BD-rate  saving for Luma component, however a little performance loss for Chroma components is observed.

Table 7 and 8 show the performance of 2.a variant of LUT-based and LUT-free BUDI on top of HM3.0-SDIP branch software, respectively. It is shown that 0.1-0.2% bit-rate reduction can be achieved with the reduction of encoding time by 3-4%. Refer to the attached excel sheets 3_CE6a_HM30_SDIP.xls and 4_CE6a_HM30_SDIP.xls for detailed information.

Table 5 Performance of the different combinations for LUT-based BUDI

| Method combination | AI - HE |  |  |  |  | AI – LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Run time |  | BD-rate |  |  | Run time |  |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 2.a | -0.17 | 0.0 | 0.0 | 97% | 100% | -0.23 | 0.0 | 0.0 | 96% | 101% |
| 2.b | -0.23 | 0.1 | 0.1 | 97% | 100% | -0.28 | 0.1 | 0.2 | 96% | 101% |
| 2.c | -0.16 | 0.0 | 0.0 | 97% | 100% | -0.21 | 0.0 | 0.0 | 96% | 101% |
| 2.d | -0.23 | 0.1 | 0.1 | 97% | 100% | -0.27 | 0.1 | 0.2 | 96% | 101% |

Table 6 Performance of the different combinations for LUT-free BUDI

| Method combination | AI - HE |  |  |  |  | AI – LC |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate |  |  | Run time |  | BD-rate |  |  | Run time |  |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |
| 2.a | -0.12 | 0.0 | 0.0 | 97% | 100% | -0.19 | 0.0 | 0.0 | 95% | 100% |
| 2.b | -0.19 | 0.1 | 0.2 | 97% | 100% | -0.24 | 0.1 | 0.2 | 96% | 100% |
| 2.c | -0.12 | 0.0 | 0.0 | 97% | 100% | -0.18 | 0.0 | 0.0 | 96% | 100% |
| 2.d | -0.18 | 0.1 | 0.1 | 97% | 100% | -0.23 | 0.2 | 0.2 | 96% | 101% |

Table 7 Performance of 2.a combination for LUT-free BUDI on top of HM3.0-SDIP

Table 8 Performance of 2.a combination for LUT-free BUDI on top of HM3.0-SDIP

# Conclusion

Based on the above test results, we conclude that

- BUDI with replacement of mode 6 improves coding efficiency, with approximately same encoding and decoding time.

- Harmonized Intra mode signaling for BUDI, DC and Planar further improves coding efficiency, while reducing 3-5% encoding time. The proposed methods are also able to solve the parsing dependency problem of Planar mode signaling because they do not require the knowledge of decoded intra prediction modes when deciding whether to decode an additional bit or not of mode 2.

- Different signaling variants for BUDI, DC and Planar mode have minor variations in coding efficiency. The 2.a variant has the better trade-off in terms of performance in Luma and chroma components.

Considering the improved performance, the reduced complexity and better harmonized signaling on Intra prediction mode, we recommend the inclusion of 2.a variant into HM test model.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Yongbing Lin, Changcai Lai, Jianhua Zheng, etc. "Improved Intra prediction for positive directions in UDI", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D300, 4th Meeting, Daegu, KR, Jan., 2011.

- Yongbing Lin, Changcai Lai, Jianhua Zheng, etc. " CE6 Subset A: Report of Improved Intra prediction for positive directions in UDI", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E286, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- Guichun Li, Lingzhi Liu, Nam Ling, etc. " Using MPS to encode extended UDI mode numbers for robustness", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E113, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- Ali Tabatabai, Keiichi Chono, Muhammed Coban, etc. "Core Experiment 6: Intra Prediction Improvement", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E706, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting, Geneva, Switzerland, Mar., 2011.

# Appendix

The excel sheets for the different combinations are as follows:

| Combination | Excel Sheet |
| --- | --- |
| 2.a variant of LUT-based BUDI | 3_CE6a_HM30.xls |
| 2.a variant of LUT-free BUDI | 4_CE6a_HM30.xls |
| 2.b variant of LUT-based BUDI | 5_CE6a_HM30.xls |
| 2.b variant of LUT-free BUDI | 6_CE6a_HM30.xls |
| 2.c variant of LUT-based BUDI | 7_CE6a_HM30.xls |
| 2.c variant of LUT-free BUDI | 8_CE6a_HM30.xls |
| 2.d variant of LUT-based BUDI | 9_CE6a_HM30.xls |
| 2.d variant of LUT-free BUDI | 10_CE6a_HM30.xls |