7.3.7   Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight, PartIdx , InferredMergeFlag ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| if( PartMode == PART_2Nx2N && log2PUWidth >= Log2IPCMCUSize ) |  |
| pcm_flag | u(1) ¦ ae(v) |
| if( pcm_flag ) { |  |
| while ( !byte_aligned( ) ) |  |
| pcm_alignment_zero_bit | u(v) |
| for( i = 0; i < 1 << ( log2CUSize << 1 ); i++ ) |  |
| pcm_sample_luma[ i ] | u(v) |
| for( i = 0; i < ( 1 << ( log2CUSize << 1 ) ) >> 1; i++ ) |  |
| pcm_sample_chroma[ i ] | u(v) |
| } else { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if( NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ce(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } |  |
| } else { /* MODE_INTER */ |  |
| if( !InferredMergeFlag ) |  |

- 8.3.1   Derivation process for luma intra prediction mode

Inputs to this process are:

–	a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture,

–	a variable log2TrafoSize specifying the size of the current prediction unit,

–	variable arrays IntraPredMode (If available) that are previously (in decoding order) derived for adjacent coding units.

Output of this process is the variable IntraPredMode[ xB ][ yB ].

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated names |
| --- | --- |
| 0 | Intra_Vertical |
| 1 | Intra_Horizontal |
| 2 | Intra_DC |
| Otherwise (3..33, excluding 6 & 9) | Intra_Angular |
| 6 | Intra_BUDI |
| 9 | Intra_Planar |
| 35 | Intra_FromLuma (used only for chroma) |

Table 8-2 specifies the number of luma intra prediction modes intraPredModeNum depending on log2TrafoSize.

- Table 8-2 – Specification of intraPredModeNum

| log2TrafoSize | intraPredModeNum |
| --- | --- |
| 2 | 17 |
| 3 | 34 |
| 4 | 34 |
| 5 | 34 |
| 6 | 3 |

Table 8-3 specifies the mapping table used for converting the number of intra prediction modes.

- Table 8-3 – Specification of mapIntraPredMode3 and mapIntraPredMode9

| value | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| mapIntraPredMode3[ value ] | 0 | 1 | 2 | 2 | 2 | 0 | 2 | 1 | 1 | 2 | 2 | 0 | 0 | 2 | 2 | 1 | 1 |
| mapIntraPredMode9[ value ] | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
| value | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |
| mapIntraPredMode3[ value ] | 2 | 2 | 2 | 2 | 0 | 0 | 0 | 2 | 2 | 2 | 2 | 1 | 1 | 1 | 1 | 2 | 2 |
| mapIntraPredMode9[ value ] | 2 | 3 | 4 | 4 | 0 | 0 | 5 | 5 | 6 | 3 | 7 | 7 | 1 | 1 | 8 | 8 | 2 |

IntraPredMode[ xB ][ yB ] labelled 0, 1, 2, .., 33 represents directions of predictions as illustrated in Figure 8-1.

2:  Intra_DC

6:  Intra_BUDI

9:  Intra_Planar

35: Intra_FromLuma

- Figure 8-1 – Intra prediction mode directions (informative)

IntraPredMode[ xB ][ yB ] is derived as the following ordered steps. [Ed. (WJ): proponent suggests to move this part to the syntax since the other syntax elements utilize IntraPredMode. But it seems too complex to move all the following process to the syntax table. Maybe it’s better to move this part to the semantics section or simply avoid the use of IntraPredMode to parse the syntax item]

- The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set equal to ( xB-1,  yB ) and ( xBB,  yBB ) is set equal to ( xB,  yB-1 ).

- For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to Intra_DC,

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 3 then candIntraPredModeN is set equal to mapIntraPredMode3[ intraPredModeN ]

- Otherwise candIntraPredModeN is set equal to mapIntraPredMode9[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set equal to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ] and NumMPMCand is set equal to 1

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ] and NumMPMCand is set equal to 2.

- IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set equal to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

- IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candModeList[cIdx]) IntraPredMode[ xB ][ yB ]++

- 8.3.2  Derivation process for chroma intra prediction mode

[Ed.: (WJ) this subclause may be moved to the semantics of intra_chroma_pred_mode syntax]

Input to this process is a luma location ( xB, yB ) specifying the top-left luma sample of the current block relative to the top-left luma sample of the current picture.

Output of this process is the variable IntraPredModeC.

The chroma intra prediction mode IntraPredModeC is derived as specifed in Error: Reference source not found or Error: Reference source not found with intra_chroma_pred_mode, IntraPredMode[ xB ][ yB ] and chroma_pred_from_luma_enabled_flag as inputs.

- 8.3.3.1	Intra sample prediction

- 8.3.3.1.1	Reference sample substitution process for intra sample prediction

- 8.3.3.1.2	Filtering process of neighbouring samples

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

Output of this process is:

–	filtered samples pF[ x, y ],. with x, y = -1..2*nS-1.

Table 8-4 – Specification of intraFilterType[ nS ][ IntraPredMode ] for various prediction unit sizes

| IntraPredMode | intraFilterType for nS = 4 | intraFilterType for nS = 8 | intraFilterType for nS = 16 | intraFilterType for nS = 32 | intraFilterType for nS = 64 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 |
| 2 | 0 | 0 | 0 | 0 | 0 |
| 3 | 1 | 1 | 1 | 1 | 1 |
| 4 | 0 | 1 | 1 | 1 | 1 |
| 5 | 0 | 1 | 1 | 1 | 0 |
| 6 | 1 | 1 | 1 | 1 | 0 |
| 7 | 0 | 1 | 1 | 1 | 0 |
| 8 | 0 | 1 | 1 | 1 | 0 |
| 9 | 0 | 1 | 1 | 0 | 0 |
| 10 | 0 | 0 | 1 | 1 | 0 |
| 11 | 0 | 0 | 1 | 1 | 0 |
| 12 | 0 | 0 | 1 | 1 | 0 |
| 13 | 0 | 0 | 1 | 1 | 0 |
| 14 | 0 | 0 | 1 | 1 | 0 |
| 15 | 0 | 0 | 1 | 1 | 0 |
| 16 | 0 | 0 | 1 | 1 | 0 |
| 17 | 0 | 0 | 1 | 1 | 0 |
| 18-33 | 0 | 0 | 0 | 1 | 0 |
| 35 | n/a | n/a | n/a | n/a | n/a |

- 8.3.3.1.3	Specification of Intra_Vertical prediction mode

- 8.3.3.1.4	Specification of Intra_Horizontal prediction mode

- 8.3.3.1.5	Specification of Intra_DC prediction mode

- 8.3.3.1.6	Specification of Intra_BUDI prediction mode

- Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size.

- Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

- This intra prediction mode is invoked when intraPredMode is equal to 6.

- The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

[LUT-free implementation]

predSamples[ x, y ] = (w1*p[x+y+1,-1] + w2*p[-1, x+y+1])+(1<< k))>>(k+1), with x, y = 0..nS-1 	(8-1)

where w1 = nS + (x-y), w2 = nS - (x-y), k = log2(nS).

[LUT-based implementation]

predSamples[ x, y ] = (w1*p[x+y+1,-1] + w2*p[-1, x+y + 1]) * wTab[x+y] + (1<<15) ) >>16, with x, y = 0..nS-1 	(8-2)

where w1 = x+1, w2 = y+1, wTab is specifed inTable xxx.

- Table xxx – Specification of wTab

| x+y | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| wTab | 32768 | 21845 | 16384 | 13107 | 10923 | 9362 | 8192 | 7282 | 6554 | 5958 | 5461 | 5041 | 4681 | 4369 | 4096 | 3855 |
| x+y | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| wTab | 3641 | 3449 | 3277 | 3121 | 2979 | 2849 | 2731 | 2621 | 2521 | 2427 | 2341 | 2260 | 2185 | 2114 | 2048 | 1986 |
| x+y | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 |
| wTab | 1928 | 1872 | 1820 | 1771 | 1725 | 1680 | 1638 | 1598 | 1560 | 1524 | 1489 | 1456 | 1425 | 1394 | 1365 | 1337 |
| x+y | 48 | 49 | 50 | 51 | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 |
| wTab | 1311 | 1285 | 1260 | 1237 | 1214 | 1192 | 1170 | 1150 | 1130 | 1111 | 1092 | 1074 | 1057 | 1040 | 1024 | - |

- 8.3.3.1.7	Specification of Intra_Angular prediction mode

- 8.3.3.1.8	Specification of Intra_Planar prediction mode

Inputs to this process are:

–	neighbouring samples p[ x, y ], with x, y = -1..2*nS-1,

–	a variable nS specifying the prediction size,

–	the bit-depth of the chroma component, bitDepth.

Output of this process is:

–	predicted samples predSamples[ x, y ], with x, y =0..nS-1.

This intra prediction mode is invoked when intraPredMode is equal to 9.

The values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

predSamples[ x, y ] = (
		( nS – 1 – x ) * p[ -1, y ] + ( x + 1 ) * p[ nS – 1 ,-1 ] + 
		( nS – 1 – y ) * p[ x ,-1 ] + ( y + 1 ) * p[ -1, nS – 1 ] + nS ) >> ( k + 1 )	(8-3)
with x, y = 0..nS-1 where k = log2( nS )

- 8.3.3.1.9	Specification of Intra_FromLuma prediction mode

8.4	Decoding process for coding units coded in inter prediction mode

8.5     Scaling, transformation and array construction process prior to deblocking filter process

- 8.5.4	Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nS specifying the size of the current transform unit,

–	a (nS)x(nS) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nS)x(nS) array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nS is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

[Ed.: (WJ) wait decision on transform precision extension]

[Ed: (WJ) derivation of trafoPrecisionExt is duplicated to that of the scaling process. Better to remove one of two.]

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, nS is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-5 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

- Table 8-5 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 |  |