| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F512 |
| --- | --- |

| Title: | CE9: Cross-check report for SP14 and ROB05 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | shenqiu@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for TI’s proposal in CE9: SP14 and ROB05 [1].

# Test condition

The proposed changes are implemented into HM3.0 reference software. The changes of the code conform to the description in [1].

The test conditions specified in [1] are used for the simulation. VS2005 x32 compiler is used to generate the executable file from the provided source code. Simulations were done on computers running Windows Server 2008 x64 OS.

However, due to the limited computational resources, the encoding tasks are dispatched to different machines. So encoding and decoding time are not very accurate, and they are just listed for reference.

# Test results

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by TI match the results we obtained.

The results of CE9: SP14 and ROB05 are presented in Table 1 and Table 2 respectively.

Table 1 – Cross-check results of CE9-SP14

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.0 | 0.2 | 0.2 | 0.1 |
| Class B | 0.3 | -0.1 | -0.2 | 0.2 | 0.0 | 0.1 |
| Class C | 0.2 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.4 | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.0 | 0.0 | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 109% |  |  | 93% |  |  |
| Dec Time[%] | 109% |  |  | 93% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | -0.2 | -0.7 | 0.7 | 0.3 | 0.2 |
| Class C | 0.5 | 0.1 | 0.1 | 0.5 | 0.3 | 0.2 |
| Class D | 0.5 | -0.3 | 0.3 | 0.5 | -0.1 | 0.3 |
| Class E | 0.2 | -0.1 | -0.5 | 0.5 | 0.2 | 0.6 |
| All | 0.5 | -0.1 | -0.3 | 0.5 | 0.2 | 0.3 |
| Enc Time[%] | 84% |  |  | 98% |  |  |
| Dec Time[%] | 88% |  |  | 100% |  |  |

Table 2 – Cross-check results of CE9-ROB05

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.4 | 0.3 | 0.2 | 0.1 |
| Class B | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.1 |
| Class C | 0.1 | 0.1 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | 0.2 | 0.2 | 0.1 |
| Enc Time[%] | 104% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 94% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.2 | -0.1 | 0.1 | 0.1 | 0.2 |
| Class C | 0.1 | 0.2 | 0.1 | 0.2 | 0.3 | 0.0 |
| Class D | 0.1 | -0.2 | 0.4 | 0.2 | -0.1 | 0.1 |
| Class E | 0.0 | 0.6 | 0.2 | -0.1 | 0.1 | 0.3 |
| All | 0.1 | 0.2 | 0.1 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 104% |  |  | 106% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  |

# Conclusion

Huawei has crosschecked the two tests proposed by TI for CE9: SP14 and ROB05. The RD performance provided by TI match the results we obtained.

- References

- Yu-Wen Huang, Benjamin Bross, Minhua Zhou, Wei-Jung Chien, Il-Koo Kim, “Description of Core Experiment 9: MV Coding and Skip/Merge Operations”, JCTVC-E709, Geneva, CH, 16-23 March, 2011.