| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F513 WG11 Number: m20944 |
| --- | --- |

| Title: | CE1: Results of experiment A.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Yu Han 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171388 yu.han@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This report provides the results of experiment A.4 defined in JCTVC-E701. In the definition of experiment A.4, the selection of the representative motion data for motion vector storage reduction (MVSR) are the same with HM3.0. But for temporal motion vector predictor (TMVP), the position of C0, as shown in Figure 1, is selected instead of the default selection scheme in HM3.0. The following two macro definitions in Typedef.h were changed from 1 to 0 for experiment A.4.

#define MTK_TMVP_H_AMVP                   0

#define MTK_TMVP_H_MRG                    0

Figure 1: Description of the positions for MDSR

The encoder was compiled in Linux OS and was tested in a parallel computing farm system. The decoder was complied with VS2008 and tested on a single server with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM.

The accompanied spreadsheet contains the detailed data of the results, and the average performance data of BD rate and encoding and decoding time are shown in following table.

Table 1 Results for experiment A.4

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 1.0 | 0.8 | 0.9 | 1.1 | 1.2 | 1.4 |
| Class B | 0.6 | 0.6 | 0.5 | 0.7 | 0.6 | 0.7 |
| Class C | 0.9 | 1.0 | 1.0 | 1.0 | 0.9 | 0.9 |
| Class D | 1.0 | 1.0 | 1.0 | 1.1 | 0.8 | 1.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.9 | 0.8 | 0.8 | 1.0 | 0.9 | 1.0 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 96% |  |  | 96% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 | 1.0 | 0.4 | 1.3 | 0.8 | 0.5 |
| Class C | 1.3 | 1.2 | 1.3 | 1.3 | 1.1 | 0.9 |
| Class D | 1.2 | 1.3 | 1.2 | 1.2 | 0.5 | 0.5 |
| Class E | 1.4 | 1.5 | 0.9 | 1.2 | 0.8 | 0.1 |
| Overall | 1.2 | 1.2 | 0.9 | 1.2 | 0.8 | 0.5 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 95% |  |  | 96% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.9 | 0.8 | 0.4 | 0.9 | 0.7 | 0.7 |
| Class C | 1.2 | 1.1 | 1.1 | 1.2 | 1.0 | 1.0 |
| Class D | 1.0 | 0.8 | 0.7 | 1.1 | 0.5 | 0.7 |
| Class E | 1.1 | 0.5 | 0.3 | 0.8 | 0.3 | 0.6 |
| Overall | 1.0 | 0.8 | 0.6 | 1.0 | 0.7 | 0.7 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  |