| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F514 |
| --- | --- |

| Title: | Crosscheck of Samsung’s proposal JCTVC-F376 by Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Qiu Shen Haitao Yang Haoping Yu Section B1, Huawei Industrial Base, Bantian Longgang Shenzhen, China | Tel: Email: | +86-755-28788991 shenqiu@huawei.com haitao.yang@huawei.com haoping.yu@huawei.com |
| Source: | Huawei Technologies Co. Ltd |  |  |

_____________________________

# Abstract

This document reports the crosschecking procedures and results for Samsung’s proposal JCTVC-F376 performed by Huawei. The software implementation is consistent with the algorithm description. The RD data we obtained match the results from proponent.

# Cross-verification

## Code-level check

The proponent proposes to simplify context modeling for luma and chroma intra modes coding by utilizing CABAC equal probability (bypass) mode. In the modified codec, bypass mode is applied in the following configuration:

Luma PU 4x4, 64x64: all bins are bypassed.

Luma PU 8x8, 16x16 and 32x32: only first bin is coded with context modeling and rest are bypassed since number of modes for these PUs are same.

All chroma PUs: only first bin is coded with context modeling and rest are bypassed since number of modes for all chroma PUs are also same.

## Test condition

The test conditions specified in the JCTVC-E700 are used for the simulation. VS2005 x64 compiler is used to generate the executable file from the provided source code. Simulations were done on computer cluster running Windows Server 2008 x64 OS.

## Test results

The proposal is about context modeling change for CABAC, so only three HE configurations are used for the simulation.

The executables generated from the provided source code can be used for the encoding and decoding test. The decoded sequences match the reconstructed sequences.

The RD data provided by Samsung are exactly the same as the results we obtained. The encoding and decoding time we obtained is similar to that from proponent. Detailed RD data can be found in the attached excel sheet.

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.01 | -0.04 | -0.02 |
| Class B | 0.00 | -0.08 | -0.04 |
| Class C | 0.07 | -0.02 | -0.02 |
| Class D | -0.02 | -0.05 | -0.03 |
| Class E | 0.06 | 0.01 | -0.04 |
| All | 0.02 | -0.04 | -0.03 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | -0.49 | -0.10 |
| Class B | 0.01 | 0.00 | 0.05 |
| Class C | -0.03 | -0.01 | -0.27 |
| Class D | -0.08 | 0.22 | 0.17 |
| Class E |  |  |  |
| All | -0.03 | -0.06 | -0.03 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.00 | 0.02 | 0.54 |
| Class C | 0.00 | -0.10 | 0.02 |
| Class D | -0.08 | 0.37 | 0.10 |
| Class E | -0.08 | 0.72 | 0.22 |
| All | -0.04 | 0.21 | 0.24 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |