| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F518 |
| --- | --- |

| Title: | CE6.b Test6: Report of harmonization on Planar prediction in SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongbing Lin, Changcai Lai, Xiaoran, Cao, Jianhua Zheng, Lingzhi Liu | Tel: Email: | +86-10-82836279 yblin@huawei.com laichangcai@huawei.com |
| Source: | HiSilicon Technologies Co., Ltd Tsinghua Univ. |  |  |

_____________________________

# Abstract

This document reports the results of CE6.b, harmonization on planar prediction in SDIP. A harmonized planar prediction is designed for both square partition and SDIP partition. Test results show 0.1% bit-rate reduction in all Intra configurations can be achieved on top of HM3.0-SDIP branch software.

# Introduction

Planar prediction has been adopted in the 5th JCT-VC meeting, which is only used for square partition. Harmonization on planar prediction in SDIP is one of the subsets of CE6.b. This document describes the unified planar prediction for square and SDIP partitions.

# Algorithm description

In planar prediction for square partition, the values of the prediction samples predSamples[ x, y ], with x, y = 0..nS-1, are derived by

predSamples[ x, y ] = (
		( nS – 1 – x ) * p[ -1, y ] + ( x + 1 ) * p[ nS – 1, -1 ] + 
		( nS – 1 – y ) * p[ x, -1 ] + ( y + 1 ) * p[ -1, nS – 1 ] + nS ) >> ( k + 1 )	
with x, y = 0..nS-1 where k = log2( nS )

Since the sum of width and height of SDIP partition is not a form of power of 2, the above formula can’t be directly applied to SDIP partition. Here a harmonized planar prediction for both square and SDIP partitions is designed as follows. In principle, it averages the two separated values interpolated in horizontal and vertical directions.

predSamples[ x, y ] = (
		(( ( width – 1 – x ) * p[ -1, y ] + ( x + 1 ) * p[ width – 1, -1 ] + width/2 ) >> m ) + 
		(( ( height – 1 – y ) * p[ x, -1 ] + ( y + 1 ) * p[ -1, height – 1] + height/2) >> n ) + 1 ) >> 1	
with x = 0..width-1, y = 0..height-1, where m=log2(width), n=log2(height).

# Experimental results

As described in CE6 documents, the following cases are tested to show the coding gains from SDIP only, Planar only, and harmonization of SDIP and Planar. It is noted that the modified planar prediction is only used in case3.

case1: SDIP=0, PLANAR=0 (anchor: HM3.0)

case2: SDIP=1, PLANAR=0 (anchor: HM3.0-SDIP branch)

case3: SDIP=1, PLANAR=1, SDIP-PLANAR=1 (anchor: HM3.0-SDIP branch)

The following tables summarize the corresponding test results for these 3 cases. The results in Table 1 and Table 2 show that planar mode achieves similar R-D performance in HM3.0 and HM3.0-SDIP platforms. The results in Table 3 show the harmonization in planar and SDIP can further improve coding efficiency with approximately the same encoding and decoding times.

Table 1  R-D performance of test case1 ( SDIP=0, PLANAR=0 )

Table 2  R-D performance of test case2 ( SDIP=1, PLANAR=0 )

Table 3  R-D performance of test case3 ( SDIP=1, PLANAR=1, SDIP-PLANAR=1 )

# Conclusion

Harmonized planar prediction for square and SDIP partitions has been designed. Experimental results show the harmonization of planar and SDIP can further improve coding efficiency with no increased encoding and decoding times. We recommended the inclusion of the harmonization of planar and SDIP into HM test model.

# Patent rights declaration(s)

Huawei Technologies and Tsinghua Univ. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Xiaoran Cao, Xiulian Peng, Changcai Lai, etc. "CE6.b1 Report on Short Distance Intra Prediction Method", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E706, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- Ali Tabatabai, Keiichi Chono, Muhammed Coban, etc. "Core Experiment 6: Intra Prediction Improvement", Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E706, 5th Meeting, Geneva, Switzerland, Mar., 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-E700, 5th Meeting, Geneva, Switzerland, Mar., 2011.