| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F519 |
| --- | --- |

| Title: | Cross-check of reference lists for low delay settings (JCTVC-F433) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu |  |  |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document is a crosscheck report of JCTVC-F433 Error: Reference source not found, reference lists for low delay settings. It is reported that the proposed reference frame setting brought more than 2% bits saving on average under the default test configuration.

# Introduction

This document reports the crosscheck result of JCTVC-F433 Error: Reference source not found, reference lists for low delay settings. The executable file was compiled by Microsoft Visual Studio 2010, 64 bit. The tests were run with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bit | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

# Result

For the low delay case, the default reference frame setting is to use 4 nearest reference frames. It was proposed in Error: Reference source not found that using 2 nearest frames and 2 high quality frames as the reference frame instead of using 4 nearest frames. When setting the number of reference of reference to 2, it was proposed to use 1 nearest frame and 1 high quality frame as the reference frame.

Two tests were performed, 4 reference frames and 2 reference frames. Our results match with the results from the proponent. The summary results are shown in Table 1 and 2.

Table 1 Summary result of 4 reference frames (Y BD-rate)

|  | LD_HE | LD_LC | LD_P_HE | LD_P_LC |
| --- | --- | --- | --- | --- |
| Class B | -1.8 | -2.1 | -1.7 | -1.7 |
| Class C | -3.1 | -3.3 | -2.9 | -3.0 |
| Class D | -3.3 | -3.7 | -3.4 | -3.7 |
| Class E | -0.7 | -1.0 | -0.7 | -0.7 |
| All | -2.3 | -2.6 | -2.2 | -2.3 |
| Enc Time | 102% | 104% | 103% | 105% |
| Dec Time | 98% | 98% | 100% | 100% |

Table 2 Summary result of 2 reference frames (Y BD-rate)

|  | LD_HE | LD_LC | LD_P_HE | LD_P_LC |
| --- | --- | --- | --- | --- |
| Class B | -1.8 | -2.2 | -2.2 | -2.1 |
| Class C | -3.5 | -4.3 | -3.4 | -3.8 |
| Class D | -5.3 | -6.3 | -5.0 | -5.5 |
| Class E | -3.9 | -3.2 | -4.1 | -3.5 |
| All | -3.5 | -3.9 | -3.5 | -3.6 |
| Enc Time | 100% | 101% | 100% | 101% |
| Dec Time | 99% | 98% | 99% | 98% |

It is clear that changing the reference frame setting for low delay case can bring significant bit saving. Similar investigations are also presented in [2], where the improvement capacity by adjusting reference frames for low delay cases is analyzed.

# References

- JCT-VC, “Reference Lists For Low Delay Settings”, JCTVC-F433, JCT-VC Meeting, Torino, IT, July, 2011.

- JCT-VC, “Encoding optimization to improve coding efficiency for low delay cases”, JCTVC-F701, JCT-VC Meeting, Torino, IT, July, 2011.