| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F522 WG11 Number: m20953 |
| --- | --- |

| Title: | Enhancing block/region based Adaptive Loop Filter by MediaTek, Qualcomm, Sharp and Toshiba |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomoo Yamakage1 Takashi Watanabe1 Takeshi Chujoh1 Ching-Yeh Chen2 Chih-Ming Fu2 Chia-Yang Tsai2 Yu-Wen Huang2 Shawmin Lei2 Marta Karczewicz3 In Suk Chong3 Tomonori Ikai4 Andrew Segall4 Tomoyuki Yamamoto4 Yukinobu Yasugi 4 Takanori Yamazaki4 | Email: | tomoo.yamakage@toshiba.co.jp takashi39.watanabe@toshiba.co.jp takeshi.chujoh@toshiba.co.jp chingyeh.chen@mediatek.com chihming.fu@mediatek.com chiayang.tsai@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com martak@qualcomm.com ichong@qualcomm.com ikai.tomohiro@sharp.co.jp asegall@sharplabs.com yamamoto.tomoyuki@sharp.co.jp yasugi.yukinobu@sharp.co.jp yamazaki.takanori@sharp.co.jp |
| Source: | TOSHIBA Corporation1, MediaTek Inc.2, Qualcomm3 and Sharp4 |  |  |

_____________________________

# Abstract

This contribution presents merged version of two CE8 subtest1 proposals (JCTVC-F321 and JCTVC-F384), which aims to improve block/region adaptive loop filter (ALF) adopted in Geneva meeting.

JCTVC-F321 proposes temporal coefficient prediction and BA enhancement, and JCTVC-F384 proposes flexible class representation and additional block-based adaptive mode. It is reported both proposals improve coding gain, and the gain of both proposals are additive. The coding gains of 0.3 %, 0.7 %, 0.6% and 0.7 % in HE-AI, RA, LB, and LP were reported. The decoding time increased by 3% on average with no changes in encoding time. Coding results were confirmed by Mitsubishi.

It is recommended to adopt this proposal to enhance the current ALF in HM30.

# Introduction

In Geneva meeting, block/region-based ALF (JCTVC-E323 [3] / JCTVC-E046 [4]) was adopted in which block-based (BA) and region-based (RA) adaptations were introduced. In Geneva meeting, two additional studies are recommended: reduction of coefficient signaling by temporal prediction and enhancing current block based ALF (i.e., by 2D flexible class representation). Both are studied by JCTVC-F321 [1] and JCTVC-F384 [2], respectively. Additionally JCTVC-F384 studied the more general BA adaptation (i.e., by considering diagonal directions). JCTVC-F321 introduces a gain of 0.0 %, 0.3 %, 0.3% and 0.2 % in HE-AI, RA, LB, LP. And JCTVC-F384 introduces a gain of 0.3 %, 0.3 %, 0.4% and 0.4 % in HE-AI, RA, LB, LP. Conceptually both enhancement approaches are in orthogonal direction. Therefore in this proposal, the merged version of those two proposals is introduced. The details of this proposal consist of a temporal direct prediction of ALF filter coefficients, 2D flexible class representation, and new BA mode switching. It is reported that merged version introduces additive gain.

# Experimental Results against HM3.0

Experiments are conducted based on the test condition JCTVC-E700 [5] and JCTVC-E708 [6] for High Efficiency cases.  Table 1 shows experimental results.  Coding efficiency gain for luma is 0.3 %, 0.7 %, 0.6% and 0.7 % in HE-AI, RA, LB, and LP.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.1 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.5 | 0.0 | 0.0 |
| Class D | -0.4 | 0.2 | 0.2 |
| Class E | -0.3 | 0.0 | 0.0 |
| Overall | -0.3 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.6 | -0.4 | -0.2 |
| Class B | -0.5 | -0.3 | -0.2 |
| Class C | -0.7 | -0.1 | -0.2 |
| Class D | -1.0 | -0.4 | -0.4 |
| Class E |  |  |  |
| Overall | -0.7 | -0.3 | -0.3 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 107% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.4 | 0.1 | 0.0 |
| Class C | -0.6 | -0.3 | -0.3 |
| Class D | -0.8 | -0.3 | 0.2 |
| Class E | -0.8 | -0.3 | -0.2 |
| Overall | -0.6 | -0.2 | -0.1 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.5 | -0.2 | -0.4 |
| Class C | -0.6 | -0.2 | -0.4 |
| Class D | -0.9 | -0.4 | -0.6 |
| Class E | -0.9 | 0.0 | -0.6 |
| Overall | -0.7 | -0.2 | -0.5 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 104% |  |  |

Table 1 Experimental results in HM3.0

# The Same Shape Results against HM3.1dev-adcs

We also implemented the same algorithms on top of HM3.1dev-adcs. Experiments are conducted based on the test condition JCTVC-E700 [5] and JCTVC-E708 [6] for High Efficiency cases.  Table 2 shows experimental results. Coding efficiency gain for luma is 0.2 %, 0.5 %, 0.6% and 0.6 % in HE-AI, RA, LB, and LP.

|  | All Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 |
| Class C | -0.3 | 0.0 | 0.0 |
| Class D | -0.3 | 0.1 | 0.1 |
| Class E | -0.3 | 0.0 | 0.0 |
| Overall | -0.2 | 0.0 | 0.0 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Random Access HE |  |  |
|  | Y | U | V |
| Class A | -0.4 | -0.1 | -0.4 |
| Class B | -0.3 | -0.1 | -0.1 |
| Class C | -0.5 | -0.3 | -0.2 |
| Class D | -0.7 | -0.3 | -0.2 |
| Class E |  |  |  |
| Overall | -0.5 | -0.2 | -0.2 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay B HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3 | -0.3 | -0.1 |
| Class C | -0.5 | -0.2 | -0.1 |
| Class D | -0.7 | -0.6 | -0.8 |
| Class E | -0.9 | -0.5 | -0.4 |
| Overall | -0.6 | -0.4 | -0.3 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Low delay P HE |  |  |
|  | Y | U | V |
| Class A |  |  |  |
| Class B | -0.3 | -0.1 | -0.4 |
| Class C | -0.6 | -0.1 | -0.3 |
| Class D | -0.9 | 0.2 | -0.3 |
| Class E | -0.9 | -0.5 | -0.4 |
| Overall | -0.6 | -0.1 | -0.4 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 104% |  |  |

Table 2 Experimental results in HM3.1dev-adcs

# Conclusion

In this contribution, the coding efficiency is improved by adopting a temporal direct prediction of ALF filter coefficients, 2D flexible class representation, and new BA mode switching. Coding efficiency gain for luma is 0.3 %, 0.7 %, 0.6% and 0.7 % in HE-AI, RA, LB, and LP. The decoding time increased by 3% on average with no changes in encoding time. Similar gains are observed on top of HM31dev-adcs software.

References

- I. S. Chong, M. Karczewicz, Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, Tomoo Yamakage, Takeshi Chujoh, Takashi Watanabe, “CE8 Subtest 1: Block based adaptive loop filter by MediaTek, Qualcomm, and Toshiba ”, JCTVC-F321, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- I. S. Chong, M. Karczewicz, T. Ikai, Y. Yasugi, T. Yamazaki, “CE8.1:Block based Adaptive Loop Filter with flexible syntax and additional BA mode by Sharp and Qualcomm”, JCTVC-F384, 6th JCT-VC Meeting, Torino, Italy, July 2011.

- I. S. Chong, M. Karczewicz, Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, Tomoo Yamakage, Takeshi Chujoh, Takashi Watanabe, “CE8 Subtest 2: Block based adaptive loop filter (ALF)”, JCTVC-E323, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- Ching-Yeh Chen, Chih-Ming Fu, Chia-Yang Tsai, Yu-Wen Huang, Shawmin Lei, “CE8 Subtest 2: Adaptation between Pixel-based and Region-based Filter Selection”, JCTVC-E046, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

[5]	F. Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, March 2011.

[6]	T. Yamakage, Y.-W. Huang, I. S. Chong, T. Ikai, M. Narroschke, W. L. Lai, A. Tabatabai, M. Budagavi, F. Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-E708_r3, March 2011.

# Patent rights declaration(s)

TOSHIBA Corporation, MediaTek Inc., Sharp Corporation, and Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

7.3.3.5	Adaptive loop filter parameter syntax

| alf_param() { | C | Descriptor |
| --- | --- | --- |
| adaptive_loop_filter_flag | 2 | u(1) |
| if ( adaptive_loop_filter_flag ) { |  |  |
| alf_direct_flag | 2 | u(1) |
| if(alf_direct_flag) { |  |  |
| ref_idx_direct | 2 | ue(v) |
| for (i=0; i< 16; i++) |  |  |
| direct_filter_flag[i] | 2 | u(1) |
| } |  |  |
| else { |  |  |
| alf_region_adaptation_flag | 2 | u(1) |
| alf_length_luma_minus_5_div2 | 2 | ue(v) |
| alf_no_filters_minus1 | 2 | ue(v) |
| for (i=1; i < AlfNumRegions; i++) { |  |  |
| if (alf_no_filters_minus1 == 1) { |  |  |
| alf_region_idx[ i ] |  | u(1) |
| } else if (alf_no_filters_minus1 > 1) { |  |  |
| if( remain_fixed_regions > 0 ) { |  |  |
| prev_alf_region_pred_flag[ i ] |  | u(1) |
| if( prev_alf_region_pred_flag[ i ] ) |  |  |
| rem_alf_region_pred_idx[ i ] |  | ue(v) |
| } |  |  |
| } |  |  |
| } |  |  |
| alf_temporal_pred_flag | 2 | u(1) |
| if(alf_pred_method_bit1) |  |  |
| ref_idx_pred | 2 | ue(v) |
| if (AlfNumFilters > 1&& !alf_temporal_pred_flag) |  |  |
| alf_pred_method | 2 | u(1) |
| alf_min_kstart_minus1 | 2 | ue(v) |
| for (i=0; i < AlfMaxDepth; i++) |  |  |
| alf_golomb_index_bit[i] | 2 | u(1) |
| for (i=0; i< AlfNumFilters; i++) |  |  |
| for (j=0; j< AlfCodedLengthLuma; j++) |  |  |
| alf_coeff_luma[i][j] |  | ge(v) |
| } |  |  |
| alf_chroma_idc | 2 | ue(v) |
| if ( alf_chroma_idc ) { |  |  |
| alf_length_chroma_minus_5_div2 | 2 | ue(v) |
| for( i = 0; i< AlfCodedLengthChroma; i++ ) |  |  |
| alf_coeff_chroma[i] |  | se(v) |
| } |  |  |
| } |  |  |
| } |  |  |

7.4.3.5	Adaptive loop filter parameter semantics

alf_direct_flag specifies whether temporal direct prediction of the filter coefficients is applied. If alf_direct_flag is equal to 1, temporal direct prediction is applied and filter information of the reference picture such as filter adaptation method, filter shape, number of filter sets, filter assignment for each variance index is used.

ref_idx_direct specifies the index of the reference picture for adaptive loop filter in the case of P slice or B slice with combined reference list.  The filter applied to the reference picture is used for temporal direct prediction.

direct_filter_flag[ i ] specifies whether luma samples classified to the i-th variance index are filtered.

alf_region_idx[ i ] specifies the filter index for adaptive loop filter.

prev_alf_region_pred_flag[ i ] and rem_alf_region_pred_idx [ i ] specify the filter index for adaptive loop filter . When prev_alf_region_pred_flag[i ] is equal to 0, the alf_region_idx[ i ] will be inferred from neighboring region (i.e., upper or left).

alf_temporal_pred_flag specifies whether temporal prediction of the filter coefficients is applied.

ref_idx_pred specifies the index of the reference picture for adaptive loop filter in the case of P slice or B slice with combined reference list.  The filter applied to the reference picture is used as the predictor for temporal prediction of the filter coefficients.

alf_pred_method specifies whether the filter coefficients are coded in a spatially predictive way.