| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F523 |
| --- | --- |

| Title: | Improvements of SPS syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766-240 Virginie.Drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

The current high-level syntax in the working draft (JCTVC-E607) is different from the one in the software (HM3.2). In a first step, this contribution analyzes discrepancies between working draft and software as well as shortcomings in the SPS (Sequence Parameter Set) syntax. In a second step, an implementation and text is provided that synchronises the SPS syntax in the software and the working draft and that solves problems related to the signalisation of the pictures size.

# Introduction

The current high-level syntax in the working draft [2] is very different from the one in the software [1]. More specifically the SPS (Sequence Parameter Set) syntax related to picture and Coding Units size is addressed in this contribution. Differences between the working draft and the software are solved and the problem of the signalisation of the number of padding pixels is addressed.

# SPS syntax problems

There have been discussions on the reflector related to the many differences between the working draft and the software regarding the high-level syntax. Our own evaluation of the differences related to the SPS syntax is given below (working draft on the left and software on the right):

| seq_parameter_set_rbsp( ) { |  | Software HM3.2 (from function TDecCavlc::parseSPS) |
| --- | --- | --- |
| profile_idc | u(8) |  |
| reserved_zero_8bits /* equal to 0 */ | u(8) |  |
| level_idc | u(8) |  |
| seq_parameter_set_id | ue(v) |  |
| max_temporal_layers_minus1 | u(3) | xReadCode ( 3, uiCode ); pcSPS->setMaxTLayers( uiCode+1 ); |
| pic_width_in_luma_samples | u(16) | xReadUvlc ( uiCode ); pcSPS->setWidth ( uiCode ); |
| pic_height_in_luma_samples | u(16) | xReadUvlc ( uiCode ); pcSPS->setHeight ( uiCode ); |
|  |  | xReadUvlc ( uiCode ); pcSPS->setPadX ( uiCode ); |
|  |  | xReadUvlc ( uiCode ); pcSPS->setPadY ( uiCode ); |
| bit_depth_luma_minus8 | ue(v) |  |
| bit_depth_chroma_minus8 | ue(v) |  |
| pcm_bit_depth_luma_minus1 | u(4) |  |
| pcm_bit_depth_chroma_minus1 | u(4) |  |
| log2_max_frame_num_minus4 | ue(v) |  |
| pic_order_cnt_type | ue(v) |  |
| if( pic_order_cnt_type = = 0 ) |  |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |  |
| else if( pic_order_cnt_type = = 1 ) { |  |  |
| delta_pic_order_always_zero_flag | u(1) |  |
| offset_for_non_ref_pic | se(v) |  |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |  |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |  |
| offset_for_ref_frame[ i ] | se(v) |  |
| } |  |  |
| max_num_ref_frames | ue(v) |  |
| gaps_in_frame_num_value_allowed_flag | u(1) |  |
| log2_min_coding_block_size_minus3 | ue(v) | xReadUvlc ( uiCode ); pcSPS->setMaxCUWidth ( uiCode ); pcSPS->setMaxCUHeight ( uiCode ); |
| log2_diff_max_min_coding_block_size | ue(v) | xReadUvlc ( uiCode ); pcSPS->setMaxCUDepth ( uiCode+1 ); |
| log2_min_transform_block_size_minus2 | ue(v) | xReadUvlc( uiCode ); pcSPS->setQuadtreeTULog2MinSize( uiCode + 2 ); |
| log2_diff_max_min_transform_block_size | ue(v) | xReadUvlc( uiCode ); pcSPS->setQuadtreeTULog2MaxSize( uiCode + pcSPS->getQuadtreeTULog2MinSize() ); |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |  |
| max_transform_hierarchy_depth_inter | ue(v) | xReadUvlc ( uiCode ); pcSPS->setQuadtreeTUMaxDepthInter( uiCode+1 ); |
| max_transform_hierarchy_depth_intra | ue(v) | xReadUvlc ( uiCode ); pcSPS->setQuadtreeTUMaxDepthIntra( uiCode+1 ); |
|  |  | xReadUvlc ( uiCode ); pcSPS->setPCMLog2MinSize (uiCode+3); |
|  |  | xReadFlag( uiCode ); pcSPS->setUseALF ( uiCode ? true : false ); |
|  |  | xReadFlag( uiCode ); pcSPS->setUseDQP ( uiCode ? true : false ); |
|  |  | xReadFlag( uiCode ); pcSPS->setUseLDC ( uiCode ? true : false ); |
|  |  | xReadFlag( uiCode ); pcSPS->setUseMRG ( uiCode ? true : false ); |
| chroma_pred_from_luma_enabled_flag | u(1) | xReadFlag( uiCode ); pcSPS->setUseLMChroma ( uiCode ? true : false ); |
|  |  | xReadFlag( uiCode ); pcSPS->setUseRMP( uiCode ? true : false ); |
|  |  | for (Int i = 0; i < pcSPS->getMaxCUDepth(); i++) |
|  |  | { |
|  |  | xReadFlag( uiCode ); pcSPS->setAMVPMode( i, (AMVP_MODE)uiCode ); |
|  |  | } |
|  |  | xReadUvlc( uiCode ); g_uiBitDepth = 8; g_uiBitIncrement = uiCode; pcSPS->setBitDepth(g_uiBitDepth); pcSPS->setBitIncrement(g_uiBitIncrement); |
| loop_filter_across_slice_flag | u(1) | xReadFlag( uiCode ); pcSPS->setLFCrossSliceBoundaryFlag( uiCode ? true : false); |
| sample_adaptive_offset_enabled_flag | u(1) | xReadFlag( uiCode ); pcSPS->setUseSAO ( uiCode ? true : false ); |
| adaptive_loop_filter_enabled_flag | u(1) |  |
| pcm_loop_filter_disable_flag | u(1) |  |
| cu_qp_delta_enabled_flag | u(1) |  |
| temporal_id_nesting_flag | u(1) | xReadFlag( uiCode ); pcSPS->setTemporalIdNestingFlag ( uiCode > 0 ? true : false ); |
|  |  | xReadCode ( 4, uiCode ); pcSPS->setPCMBitDepthLuma ( 1 + uiCode ); |
|  |  | xReadCode ( 4, uiCode ); pcSPS->setPCMBitDepthChroma ( 1 + uiCode ); |
|  |  | xReadFlag( uiCode ); pcSPS->setPCMFilterDisableFlag ( uiCode ? true : false ); |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

The following problems appear:

- Some syntax elements (blue in the table) are present in the working draft, but not in the software (related to profile, level, sequence parameter set index, frame numbers and picture order counts)

- Some syntax elements (orange in the table) are present in the software, but not in the working draft (related to padding and flags for enabling/disabling coding tools)

- Some syntax elements (gray in the table) are present in one form in the working draft and in another form in the software

- Some syntax elements (green in the table) are present in the same form in both the working draft and the software, but in another order

This contribution mainly solves the second, third and fourth types of problems. More particularly, software modifications were performed for the following syntax elements in order to code them as in the working draft:

- pic_width_in_luma_samples and pic_height_in_luma_samples: coded using 16 bits instead of Exp-Golomb codes

- bit_depth_luma_minus8: moved after the picture size

- log2_min_coding_block_size_minus3: coded by using the logarithm in base 2 of smallest CU size minus 3 instead of coding the size in number of luma samples of the LCU, which is a waste of bits because most values are not allowed

# Padding of the source sequence

In the current working draft, the size of the pictures is signalled in number of luma samples. However, the size of the coded pictures has to be a multiple of the size of the smallest coding unit (SCU), i.e. the width of the picture has to be a multiple of the horizontal size of the smallest coding unit and the height of the picture has to be a multiple of the vertical size of the smallest coding unit. Therefore, although most sizes are not allowed, all sizes between 0 and 216-1 may be signalled with current coding, which is a waste of bits.

Furthermore, it is not described in the working draft how the size of the coded pictures (variables PicWidthInSamplesL and PicHeightInSamplesL in the syntax tables of the working draft) is derived from the syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples.

The HEVC standard should be able to compress sequences with a size that is not a multiple of the size of the smallest coding unit, especially as the size of the smallest coding unit is not fixed in HEVC, but is rather transmitted in the SPS. The software solves that problem at encoder side by padding the right border of the images and the bottom border of the images in order to have an appropriate size for coding. The size of the padded images is sent in the SPS in number of luma samples, along with the number of padding pixels in number of luma samples. This signalisation wastes bits and may be confusing for users because most of the picture sizes and number of padding pixels are not allowed. Only the values that result in picture sizes that are multiples of the size of the smallest coding unit are allowed.

The following solution is proposed to fix this problem and strengthens the SPS syntax:

- pic_width_in_luma_samples and pic_height_in_luma_samples specify the size of each non-padded coded picture, i.e. the size of the pictures of the input signal at encoder side and of the output signal at decoder side.

- The actual number of padding samples is calculated in the same way at the encoder and at the decoder in order to have a size for the coded pictures that is a multiple of the signalled size of the smallest coding units. The size of the coded pictures is derived as follows:

SCUWidth = SCUHeight = 2log2_min_coding_block_size_minus3+3

PicWidthInSamplesL = pic_width_in_luma_samples + SCUWidth – (pic_width_in_luma_samples % SCUWidth)

PicHeightInSamplesL = pic_height_in_luma_samples + SCUHeight – (pic_height_in_luma_samples % SCUHeight)

- The current HM3.2 software allows adding more padding samples. To have the same functionality, the two syntax elements horizontal_padding_in_number_of_SCU and vertical_padding_in_number_of_SCU are introduced. They specify the padding information at the right and bottom borders of the pictures as a number of additional padding smallest coding units. They are transmitted after the size of the smallest coding unit.

The size of the coded pictures is therefore derived as follows:

SCUWidth = SCUHeight = 2log2_min_coding_block_size_minus3+3

PicWidthInSamplesL = pic_width_in_luma_samples + SCUWidth – (pic_width_in_luma_samples % SCUWidth) + horizontal_padding_in_number_of_SCU * SCUWidth

PicHeightInSamplesL = pic_height_in_luma_samples + SCUHeight – (pic_height_in_luma_samples % SCUHeight) + vertical_padding_in_number_of_SCU * SCUHeight

These new syntax elements corresponds to the syntax elements PadX and PadY of the current software, but the number transmitted corresponds to additional smallest coding units, instead of additional luma samples. The reason is to avoid confusion and to avoid wasting bits by sending a number of luma samples although most values are not allowed.

For example, to code a sequence of size 1908x1076 with a smallest coding unit of size 8x8 and using 12 padding columns on the right border and 4 padding rows on the bottom border:

- In the current software HM3.2, the following syntax elements are transmitted in the SPS:

  - pic_width_in_luma_samples = 1920  21 bits

  - pic_height_in_luma_samples = 1080  21 bits

  - padX = 12  7 bits

  - padY = 4  5 bits

- With the proposed solution, the following syntax elements are transmitted in the SPS:

  - pic_width_in_luma_samples = 1908  16 bits

  - pic_height_in_luma_samples = 1076  16 bits

  - horizontal_padding_in_number_of_SCU = 1  3 bits

  - vertical_padding_in_number_of_SCU = 0  1 bit

The number of bits of the two syntax elements pic_width_in_luma_samples and pic_height_in_luma_samples is modified, because the software does not encode these syntax elements correctly (see explanations from section 2).

For the new syntax elements, it is more efficient and less confusing to code the values 0 and 1 instead of the values 4 and 12. Indeed, all the values between 0 and 3 and between 5 and 11 included are not allowed.

# Results

The different changes and improvements listed above have been implemented in HM3.2. The software was run for several sequences of different sizes and the number of bytes in the SPS NAL units of the generated bit streams was checked using the tool from BBC (annexBbytecount-linux-amd64-static). It has been observed that between 2 and 4 bytes can be saved in the SPS, depending on the size of the sequences and the amount of padding defined by the user at encoder side.

| Sequence | HM3.2: number SPS bytes | Modified HM3.2: number SPS bytes | SPS bytes saved |
| --- | --- | --- | --- |
| Nebuta | 15 | 12 | 3 |
| Kimono | 15 | 12 | 3 |
| BasketballDrill | 14 | 12 | 2 |
| Kimono with 8 pixels padded below (coded size 1920x1088) | 16 | 12 | 4 |

# New SPS syntax

A new SPS syntax is given below. This new SPS syntax contains all the syntax elements included in the software, in the order they appear in the software, as well as the additional syntax elements proposed in this contribution. The new syntax elements proposed in this contribution are marked in yellow, the rest of the changes are marked in blue.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_idc | u(8) |
| reserved_zero_8bits /* equal to 0 */ | u(8) |
| level_idc | u(8) |
| seq_parameter_set_id | ue(v) |
| max_temporal_layers_minus1 | u(3) |
| pic_width_in_luma_samples | u(16) |
| pic_height_in_luma_samples | u(16) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_frame_num_minus4 | ue(v) |
| pic_order_cnt_type | ue(v) |
| if( pic_order_cnt_type = = 0 ) |  |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| else if( pic_order_cnt_type = = 1 ) { |  |
| delta_pic_order_always_zero_flag | u(1) |
| offset_for_non_ref_pic | se(v) |
| num_ref_frames_in_pic_order_cnt_cycle | ue(v) |
| for( i = 0; i < num_ref_frames_in_pic_order_cnt_cycle; i++ ) |  |
| offset_for_ref_frame[ i ] | se(v) |
| } |  |
| max_num_ref_frames | ue(v) |
| gaps_in_frame_num_value_allowed_flag | u(1) |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| horizontal_padding_in_number_of_SCU | ue(v) |
| vertical_padding_in_number_of_SCU | ue(v) |
| log2_min_pcm_coding_block_size_minus3 | ue(v) |
| adaptive_loop_filter_enabled_flag | u(1) |
| cu_qp_delta_enabled_flag | u(1) |
| low_delay_coding_enabled_flag | u(1) |
| merge_enabled_flag | u(1) |
| chroma_pred_from_luma_enabled_flag | u(1) |
| for( i = 0; i < log2_diff_max_min_coding_block_size+1; i++ ) |  |
| amvp_mode_enabled_flag[ i ] | u(1) |
| loop_filter_across_slice_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| temporal_id_nesting_flag | u(1) |
| pcm_bit_depth_luma_minus1 | u(4) |
| pcm_bit_depth_chroma_minus1 | u(4) |
| pcm_loop_filter_disable_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The semantics for the new syntax elements are:

pic_width_in_luma_samples specifies the width of each picture in luma samples. The value of pic_width_in_luma_samples shall be in the range of 0 to 216-1, inclusive.

pic_height_in_luma_samples specifies the height of each picture in luma samples. The value of pic_height_in_luma_samples shall be in the range of 0 to 216-1, inclusive.

horizontal_padding_in_number_of_SCU specifies the number of additional horizontal padding coding units of minimum CU size.

The variable SCUWidth is set equal to 2log2_min_coding_block_size_minus3+3.

The variable PicWidthInSamplesL is set equal to pic_width_in_luma_samples + SCUWidth – (pic_width_in_luma_samples % SCUWidth) + horizontal_padding_in_number_of_SCU * SCUWidth.

vertical_padding_in_number_of_SCU specifies the number of additional vertical padding coding units of minimum SCU size.

The variable SCUHeight is set equal to 2log2_min_coding_block_size_minus3+3.

The variable PicHeightInSamplesL is set equal to pic_height_in_luma_samples + SCUHeight – (pic_height_in_luma_samples % SCUHeight) + vertical_padding_in_number_of_SCU * SCUHeight.

# Conclusion

This contribution proposes improvements of the SPS syntax in the software and the working draft. The purpose is to synchronise the SPS syntax in the software and the working draft, as well as to solve problems related to the signalisation of the pictures size. Indeed, most values that can be currently signalled - although they are not allowed - are removed. Therefore, the proposed design is less confusing and avoids waste of bits in the SPS NAL unit.

# References

- HM3.2 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.2

- T. Wiegand, W.-J. Han, J.-R. Ohm, G. J. Sullivan, WD3: Working Draft 3 of High-Efficiency Video Coding, JCTVC-E603, Geneva, CH, March 2011.

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).