| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F524 |
| --- | --- |

| Title: | CE5: Results on modified inter mode coding and joint coding of split flags for CAVLC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Kotra Virginie Drugeon Thomas Wedi Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766-154 Anand.Kotra@eu.panasonic.com +49 6103 766-240 Virginie.Drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution is a part of CE5 and gives the results in HM3.0 of the technique presented in JCTVC-E258. The technique proposed consists of modifications for inter mode and split flags coding in CAVLC. In particular, the number of tables used for inter mode coding in CAVLC is modified. The partitioning information of neighboring CUs is exploited. Furthermore, a joint coding of the split flags for I frames is proposed. These modifications are tested in HM3.0: 0.1% gain for the intra, random access and low delay settings are reported.

# Introduction

This contribution is a part of CE5 [1] and gives the results in HM3.0 [2] of the technique presented in JCTVC-E258 [3]. It consists of small modifications to the current CAVLC coding of partitioning and high-level prediction information.

In the current HM3.0, the split flag, skip flag, merge flag and prediction mode and size of one CU are grouped and coded together if CAVLC is used (see JCTVC-E603 [4]). Seven modes and seven fixed code words are defined for that purpose. A table contains the mapping between the modes and the code words. This table is adapted at each CU using a counter based adaptation (see JCTVC-E143 [5]).

Currently one table is defined for each depth. Therefore, it is assumed that the statistics are similar for all the CUs at one given depth. However, the statistics of the modes may be different depending on the content of the CUs.

For intra, a split flag is encoded for each CU. If the CU is further partitioned in four subCUs, the bit 1 is signaled; otherwise the bit 0 is signaled. However high quality I frames contain mainly 8x8 and 16x16 CUs. Therefore, the bit 1 indicating further partitioning is often signaled.

For example, to signal that a whole LCU (64x64) is partitioned in CUs of size 16x16, it is necessary to send once the bit 1 to signal that the LCU is further partitioned in four 32x32 subCUs and to send four bits 1 to signal that each one of the four 32x32 subCUs are further partitioned. Instead of signaling the bit sequence 1111, it can be more efficient to signal only 1 if it is likely that four CUs in Zscan order are further partitioned. Therefore, depending on the partitioning flags of the CUs above and left, the codeword to send a sequence of four split flags is modified.

# Additional tables

As mentioned above, the frequency of occurrences of the different inter modes may be different depending on the content of the CU: CUs around object edges are often smaller than in homogeneously moving areas. Therefore, several tables for each depth are introduced. The appropriate code word table for a CU is selected depending on the partitioning and prediction information of the CUs above and left.

For each depth, the contexts for the split flags and skip flags as used in CABAC are exploited to define three or five tables per depth.

# Joint coding of split flags for I frames

In I frames, the whole CU tree is signaled using one split flag at each depth. An example of a CU tree and corresponding split flags can be seen on Figure 1.

Figure 1: example of CU tree

One LCU contains at least one split flag and at most 21 split flags.

The technique presented in this contribution consists in coding jointly a group of four split flags corresponding to four CUs in Zscan order (for example, coding the group 1101 from depth 1 in Figure 1 using a predefined codeword). Each group of four split flags can take 16 different values. The statistics of these 16 values were gathered over the whole test set and used to design adapted codewords based on the Huffman algorithm.

Three context dependant tables per depth are used. The contexts correspond to the split flag contexts as defined for CABAC. It enables content adaptive codewords.

In addition, the tables design was made QP dependant. A set of table was designed for QP smaller than 30 and another set was designed for QP higher than 30.

# Results

The modifications described in sections 2 (with three tables per depth and five tables per depth) and 3 are implemented in the version HM3.0 of the software. The tests are performed using the common conditions as described in JCTVC-E700 [6]. The executables are compiled using gcc 4.4.1 and executed on Linux 64 bits. The encoding and decoding runtimes were measured on the same computer for 3 tables per depth case and are therefore comparable.

Table 1: 3 tables per depth

Table 2: 5 tables per depth

Unfortunately, the results are lower than the results reported in JCTVC-E258 [3]. One reason is the inclusion of the counter based adaptation in HM3.0 compared to HM2.0, which was a part of JCTVC-E258. Furthermore, the current idea consists in small modifications for the coding of the syntax elements cu_split_pred_part_mode and split_coding_unit_flag. Therefore, the gain achieved can not be expected to be very high, but the runtime measurements show that no significant additional complexity is added.

The technique performs better for higher QPs. Indeed, for higher QPs, the number of bits needed to code the split flags in I frames gets more important compared to the total number of bits. Therefore, the gain due to the proposed technique gets more important.

|  | All intra LC |  |  |
| --- | --- | --- | --- |
|  | Y | U | V |
| Class A | -0.6 | -0.2 | -0.6 |
| Class B | -0.3 | 0.0 | 0.1 |
| Class C | -0.5 | -0.5 | -0.4 |
| Class D | -0.1 | 0.1 | -0.1 |
| Class E | -0.3 | 0.2 | 0.0 |
| Overall | -0.4 | -0.1 | -0.2 |

Table 3: Joint coding of split flags for QPs 45, 46, 48, 49

# Conclusions

The presented small modifications to the coding of inter mode and split flags in CAVLC bring small gains (0,1% in average) in HM3.0, without any increase in runtime.

# References

- JCTVC-E705 CAVLC Entropy Coding Improvements, Xianglin Wang, Geneva, Switzerland, 16-23 March, 2011

- HM3.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-3.0

# JCTVC-E258 Improvement of inter mode coding and split flags coding for LCEC, Virginie Drugeon, Thomas Wedi, Matthias Narroschke, Geneva, Switzerland, 16-23 March, 2011

# JCTVC-E603 WD3: Working Draft 3 of High-Efficiency Video Coding, Thomas Wiegand, Woo-Jin Han, Benjamin Bross, Jens-Rainer Ohm, Gary J. Sullivan, Geneva, Switzerland, 16-23 March, 2011

# JCTVC-E143 CE5: Counter based adaption for LCEC, Liwei Guo, Bin Li, Xianglin Wang, Marta Karczewicz, Jizheng Xu, Geneva, Switzerland, 16-23 March, 2011

# JCTVC-E700 Common test conditions and software reference configurations, Frank Bossen, Geneva, Switzerland, 16-23 March, 2011

# Patent rights declaration(s)

Panasonic Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Appendix A: Proposed syntax and semantics

The modified portions of JCTVC-E603 are written in blue in the present appendix.

# Changes in the coding tree syntax

| coding_tree( x0, y0, log2CUSize, isFirstCUInDepth ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && cuAddress( x0, y0 ) >= SliceAddress ) { |  |
| if( !entropy_coding_mode_flag && slice_type != I ) |  |
| cu_split_pred_part_mode[ x0 ][ y0 ] | ce(v) |
| else if( !entropy_coding_mode_flag && log2CUSize > Log2MinCUSize && isFirstCUInDepth ) |  |
| split_four_coding_unit[ x0 ][ y0 ] | ce(v) |
| else if( log2CUSize > Log2MinCUSize ) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( adaptive_loop_filter_flag && alf_cu_control_flag ) { |  |
| cuDepth = Log2MaxCUSize – log2CUSize |  |
| if( cuDepth <= alf_cu_control_max_depth ) |  |
| if( cuDepth == alf_cu_control_max_depth ¦¦ |  |
| split_coding_unit_flag[ x0 ][ y0 ] == 0 ) |  |
| AlfCuFlagIdx++ |  |
| } |  |
| if( split_coding_unit_flag[ x0 ][ y0 ] ) { |  |
| if( cu_qp_depta_enabled_flag && log2CUSize == log2MinCUDQPSize ) |  |
| IsCuQpDeltaCoded = 0 |  |
| x1 = x0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2CUSize ) >> 1 ) |  |
| if( cuAddress( x1, y0 ) > SliceAddress ) |  |
| moreDataFlag = coding_tree( x0, y0, log2CUSize – 1, 1 ) |  |
| if( cuAddress( x0, y1 ) > SliceAddress && moreDataFlag && x1 < PicWidthInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y0, log2CUSize − 1, 0 ) |  |
| if( cuAddress( x1, y1 ) > SliceAddress && moreDataFlag && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x0, y1, log2CUSize − 1, 0 ) |  |
| if( moreDataFlag && x1 < PicWidthInSamplesL && y1 < PicHeightInSamplesL ) |  |
| moreDataFlag = coding_tree( x1, y1, log2CUSize − 1, 0 ) |  |
| } else { |  |
| … (rest of syntax is not modified) |  |
| } |  |

split_four_coding_unit[ x0 ][ y0 ] specifies whether each coding unit in a group of four coding units in Zscan order is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the first considered coding block relative to the top-left luma sample of the picture.

The variables split_coding_unit_flag[ x0 ][ y0 ], split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 ], split_coding_unit_flag[ x0 ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] and split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] are derived as specified in subclause 9.2.4.8.

When split_four_coding_unit[ x0 ][ y0 ] is not present and isFirstCUInDepth is equal to true, the following applies:

–	If log2CUSize is greater than Log2MinCUSize, the values of split_coding_unit_flag[ x0 ][ y0 ], split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 ], split_coding_unit_flag[ x0 ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] and split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ]  are inferred to be equal to 1.

–	Otherwise (log2CUSize is equal to Log2MinCUSize), the values of split_coding_unit_flag[ x0 ][ y0 ], split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 ], split_coding_unit_flag[ x0 ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] and split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] are inferred to be equal to 0.

# Changes in "9.2.1 Parsing process for VLC codes"

Inputs to this process are bits from the RBSP and the parameter vlcNum specifying the VLC code. If vlcNum is equal to 14, inputs to this process additionally include a parameter cMax. If vlcNum is equal to 20, inputs to this process additionally include the depth of the current coding unit relative to the LCU size cuDepth and the QP value for coding of the current coding unit.

… (text not modified)

- Otherwise, if vlcNum is equal to 20:

  - If QP is smaller than 30, codeNum is determined according to Table 1.

  - If QP is higher than 30, codeNum is determined according to Table 2.

| Split flag codeword | cuDepth=1 | cuDepth=1 | cuDepth =1 | cuDepth >=2 | cuDepth >=2 | cuDepth >=2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ctx = 0 | Ctx = 1 | Ctx = 2 | Ctx = 0 | Ctx = 1 | Ctx = 2 |
| 0 | 100 | 01011 | 000011 | 01 | 1111 | 0010 |
| 1 | 0010 | 011000 | 0101100 | 1100 | 11010 | 100010 |
| 2 | 10110 | 011111 | 000001 | 0001 | 11100 | 01001 |
| 3 | 0101 | 01010 | 010101 | 0010 | 11011 | 01011 |
| 4 | 10101 | 011001 | 000000 | 1101 | 0010 | 10000 |
| 5 | 0110 | 00100 | 010100 | 1011 | 11101 | 01010 |
| 6 | 101001 | 001011 | 0101101 | 111111 | 00010 | 100011 |
| 7 | 000 | 0011 | 0001 | 0011 | 1001 | 0011 |
| 8 | 01111 | 011110 | 010111 | 0000 | 1000 | 0111 |
| 9 | 101000 | 001010 | 000010 | 111110 | 00011 | 01000 |
| 10 | 01110 | 0100 | 0111 | 10100 | 0000 | 0110 |
| 11 | 0011 | 0001 | 0100 | 11110 | 1011 | 1010 |
| 12 | 01001 | 0000 | 0110 | 11100 | 1100 | 000 |
| 13 | 10111 | 01110 | 0010 | 11101 | 1010 | 1001 |
| 14 | 01000 | 01101 | 0011 | 10101 | 0011 | 1011 |
| 15 | 11 | 1 | 1 | 100 | 01 | 11 |

Table 1: QP smaller than 30

| Split flag codeword | cuDepth =1 | cuDepth =1 | cuDepth =1 | cuDepth >=2 | cuDepth >=2 | cuDepth >=2 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Ctx = 0 | Ctx = 1 | Ctx = 2 | Ctx = 0 | Ctx = 1 | Ctx = 2 |
| 0 | 00 | 000 | 10110 | 11 | 110 | 1110 |
| 1 | 1001 | 01110 | 1111110 | 000 | 0001 | 00110 |
| 2 | 11110 | 10101 | 101110 | 1001 | 0111 | 10111 |
| 3 | 1101 | 0010 | 111110 | 1010 | 0010 | 00111 |
| 4 | 11101 | 10100 | 110111 | 1011 | 1011 | 11011 |
| 5 | 11111 | 01111 | 110110 | 0100 | 11110 | 00101 |
| 6 | 110011 | 101101 | 1111111 | 00110 | 01010 | 00100 |
| 7 | 1010 | 1000 | 11110 | 0010 | 11111 | 11010 |
| 8 | 11100 | 10111 | 11010 | 0110 | 1110 | 1111 |
| 9 | 110010 | 101100 | 101111 | 100010 | 01011 | 10110 |
| 10 | 10001 | 0101 | 1100 | 01011 | 0110 | 1010 |
| 11 | 11000 | 0110 | 1010 | 01010 | 0100 | 1000 |
| 12 | 10111 | 1001 | 1110 | 10000 | 1010 | 000 |
| 13 | 10110 | 0011 | 1000 | 00111 | 0011 | 1001 |
| 14 | 10000 | 0100 | 1001 | 100011 | 0000 | 1100 |
| 15 | 01 | 11 | 0 | 0111 | 100 | 01 |

Table 2: QP higher than 30

# Changes "9.2.2.4 Initialisation process for splitPredPartModeTable"

Outputs of this process are initial values of the variable array splitPredPartModeTable and associated counters.

The variable array splitPredPartModeTable is initialized as follows:

–	For each cuDepth with a value from 0 to (Log2MaxCUSize – Log2MinCUSize) and each context with a value from 0 to 4, set splitPredPartModeTable[cuDepth][ctx][codeNum] equal to codeNum, with codeNum =  0…6.

For each cuDepth with a value from 0 to (Log2MaxCUSize – Log2MinCUSize) and each context with a value from 0 to 4, there are seven counters used for codeword index adaptive mapping. These counters are initialized according to subclause Error: Reference source not found.

# Changes in "9.2.4.5 Parsing process for cu_split_pred_part_mode "

This process is invoked when entropy_coding_mode_flag is equal to 0 for parsing syntax element cu_split_pred_part_mode in subclause Error: Reference source not found.

Inputs to this process are bits from slice data, splitPredPartModeTable.

Output of this process is the syntax element cu_split_pred_part_mode.

The value of cu_split_pred_part_mode is derived as follows.

- If log2CUSize is greater than Log2MinCUSize, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 6 as inputs and the variable codeNum as output.

- Otherwise, the parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 14 and cMax of 5 as inputs and the variable codeNum as output.

- Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit.

    - If the prediction unit covering location ( xL, yL ) is available and its depth is higher than the depth of the current coding unit, condSplitL is set equal to 1.

    - Otherwise, condSplitL is set equal to 0.

    - If the prediction unit covering location ( xL, yL ) is available and skip_flag[ xL ][ yL ] is equal to true, condSkipL is set equal to 1.

    - Otherwise, condSkipL is set equal to 0.

- Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit.

    - If the prediction unit covering location ( xA, yA ) is available and its depth is higher than the depth of the current coding unit, condSplitA is set equal to 1.

    - Otherwise, condSplitA is set equal to 0.

    - If the prediction unit covering location ( xA, yA ) is available and skip_flag[ xA ][ yA ] is equal to true, condSkipA is set equal to 1.

    - Otherwise, condSkipA is set equal to 0.

- The variable ctx is derived as:

    - If condSplitL and condSplitA are both equal to 0, one of condSkipL or condSkipA is equal to 1 and one of condSkipL or condSkipA is equal to 0, the variable ctx is set equal to 1.

    - Otherwise if condSplitL and condSplitA are both equal to 0 and condSkipL and condSkipA are both equal to 1, the variable ctx is set equal to 2.

    - Otherwise if condSplitL and condSplitA are both equal to 1 and condSkipL and condSkipA are both equal to 0, the variable ctx is set equal to 3.

    - Otherwise if condSplitL and condSplitA are both equal to 1, one of condSkipL or condSkipA is equal to 1 and one of condSkipL or condSkipA is equal to 0, the variable ctx is set equal to 4.

    - Otherwise, the variable ctx is set equal to 0.

- The syntax element cu_split_pred_part_mode is derived as

cu_split_pred_part_mode = splitPredPartModeTable[Log2MaxCUSize – log2CUSize][ctx][codeNum] 	(9-1)

- If log2CUSize is equal to Log2MinCUSize and cu_split_part_mode has a value of 5, PredMode and PartMode are determined as follows

- Read one bit and assign its value to a variable b

- If b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_2Nx2N

- Otherwise, read the next one bit and assign its value to the variable b.

    - if b is equal to 1, set PredMode to MODE_INTRA and PartMode to PART_NxN

    - Otherwise, set PredMode to MODE_INTER and PartMode to PART_NxN

- The variable array splitPredPartModeTable[Log2MaxCUSize – log2CUSize][ctx] is updated by invoking process Error: Reference source not found with splitPredPartModeTable[Log2MaxCUSize – log2CUSize][ctx], codeNum and a value of 7 as inputs.

# Addition of new subsection "9.2.4.8 Parsing process for split_four_coding_unit "

Inputs to this process are bits from slice data, the size of the current coding unit log2CUSize, the location ( x0, y0 ) of the top-left luma sample of the considered coding unit relative to the top-left luma sample of the picture and the QP for coding of the current coding unit.

Outputs of this process are the syntax elements split_four_coding_unit[ x0 ][ y0 ], as well as the variables split_coding_unit_flag[ x0 ][ y0 ], split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 ], split_coding_unit_flag[ x0 ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ] and split_coding_unit_flag[ x0 + ( ( 1 << log2CUSize ) >> 1 ) ][ y0 + ( ( 1 << log2CUSize ) >> 1 ) ].

The value of split_four_coding_unit[ x0 ][ y0 ]is derived as follows.

- Let the luma location ( xL, yL ) specify a location covered by the prediction unit to the left of the top-left luma sample of the current prediction unit.

    - If the prediction unit covering location ( xL, yL ) is available and its depth is higher than the depth of the current coding unit, condSplitL is set equal to 1.

    - Otherwise, condSplitL is set equal to 0.

- Let the luma location ( xA, yA ) specify a location covered by the prediction unit above the top-left luma sample of the current prediction unit.

    - If the prediction unit covering location ( xA, yA ) is available and its depth is higher than the depth of the current coding unit, condSplitA is set equal to 1.

    - Otherwise, condSplitA is set equal to 0.

- The variable ctx is derived as:

    - If condSplitL and condSplitA are both equal to 0, the variable ctx is set equal to 0.

    - Otherwise if one of condSplitL or condSplitA is equal to 0 and one of condSplitL or condSplitA is equal to 1, the variable ctx is set equal to 1.

    - Otherwise, the variable ctx is set equal to 2.

- The parsing process described in subclause Error: Reference source not found is invoked with vlcNum of 20, cuDepth of (Log2MaxCUSize - log2CUSize) and QP as inputs and the variable codeNum as output.

- The syntax element split_four_coding_unit[ x0 ][ y0 ] is set equal to codeNum.

The variable x1 is set equal to x0 + ( ( 1 << log2CUSize ) >> 1 ).

The variable y1 is set equal to y0 + ( ( 1 << log2CUSize ) >> 1 ).

The values of split_coding_unit_flag[ x0 ][ y0 ], split_coding_unit_flag[ x1 ][ y0 ], split_coding_unit_flag[ x0 ][ y1 ] and split_coding_unit_flag[ x1 ][ y1 ] are derived as follows:

split_coding_unit_flag[ x0 ][ y0 ] = split_four_coding_unit[ x0 ][ y0 ] & 8

split_coding_unit_flag[ x1 ][ y0 ] = split_four_coding_unit[ x0 ][ y0 ] & 4

split_coding_unit_flag[ x0 ][ y1 ] = split_four_coding_unit[ x0 ][ y0 ] & 2

split_coding_unit_flag[ x1 ][ y1 ] = split_four_coding_unit[ x0 ][ y0 ] & 1