| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F527 |
| --- | --- |

| Title: | Cross-check of JCTVC-F274 from Orange Labs |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Virginie Drugeon Panasonic R&D Center Germany Monzastr. 4c, 63225 Langen, Germany | Tel: Email: | +49 6103 766240 Virginie.Drugeon@eu.panasonic.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution reports test results and software analysis for the combination of wavefront parallel processing and tiles proposed by Orange Labs in JCTVC-F274 (section 8 experiment 3). The software provided by Orange Labs has been analyzed and compiled. The results obtained are the same as the ones reported by Orange Labs.

# Introduction

Wavefront parallel processing was proposed by Orange Labs and combined with tiles (see JCTVC-F274 [1]). This is a verification (tests and software) of experiment 3 (section 8) of JCTVC-F274.

# Results

Tested configuration

The tests were performed using the software provided by Orange Labs and the software with tiles provided by eBrisk. Both softwares are based on HM3.0.

The configuration files provided with HM3.0 were modified to add the following parameters (for tiles):

#============ Tiles ================

# Changed from reference to have two side by side views.

ColumnRowInfoPresent          : 0

UniformSpacingIdr             : 1

TileBoundaryIndependenceIdr   : 0 or 1

ColumnWidthMinus1             : from table below.

RowHeightMinus1               : from table below.

For the different classes, following values of ColumnWidthMinus1 and RowHeightMinus1 were used:

|  | classA | classB | classC | classD | classE |
| --- | --- | --- | --- | --- | --- |
| ColumnWidthMinus1 | 19 | 14 | 6 | 3 | 9 |
| RowHeightMinus1 | 24 | 16 | 7 | 3 | 11 |

For both experiments described below, the software provided by Orange Labs was compared to the software with tiles provided by eBrisk. In both the software from Orange Labs and the reference, two tiles per frame were used (see parameters above for the position and size of each tile).

The executables were compiled on 64-bit Linux wit gcc 4.5.0.

The reference and the software from Orange Labs were run on machines with the same hardware characteristics. Therefore the encoder and decoder runtimes are comparable.

Results for first experiment (scenario B in JCTVC-F274)

For the first experiment, the parameter TileBoundaryIndependenceIdr was set to 0 in both the reference and the software from Orange Labs. Therefore, both softwares use two dependant tiles per frame. Furthermore, the software from Orange Labs was configured to use two substreams per frame.

A summary of the results is provided below. For full results, please refer to the attached excel sheet JCTVC-F527_2substreamsPerFrame.xls.

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,1 | 0,0 | -0,3 | 1,0 | 0,9 | 0,3 |
| Class B | 0,2 | 0,1 | -0,1 | 0,7 | 1,0 | 0,8 |
| Class C | 0,1 | 0,0 | 0,1 | 0,5 | 0,8 | 0,6 |
| Class D | 0,0 | -0,1 | -0,1 | 0,4 | 0,7 | 0,6 |
| Class E | 0,5 | -0,2 | -0,1 |  |  |  |
| Overall | 0,2 | 0,0 | -0,1 | 0,7 | 0,8 | 0,6 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1,1 | 1,8 | 2,1 | 1,0 | 1,0 | 1,2 |
| Class C | 0,7 | 1,1 | 1,0 | 0,5 | 0,8 | 0,4 |
| Class D | 0,6 | 1,0 | 1,2 | 0,5 | 0,3 | 0,5 |
| Class E | 2,1 | 2,6 | 2,5 | 2,0 | 1,6 | 1,3 |
| Overall | 1,1 | 1,6 | 1,7 | 0,9 | 0,9 | 0,8 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The results obtained are identical to the results provided by Orange Labs.

Results for second experiment (scenario A in JCTVC-F274)

For the second experiment, the parameter TileBoundaryIndependenceIdr was set to 1 in both the reference and the software from Orange Labs. Therefore, both softwares use two independent tiles per frame. Furthermore, the software from Orange Labs was configured to use two substreams per tile, i.e. four substreams per frame.

A summary of the results is provided below. For full results, please refer to the attached excel sheet JCTVC-F527_2substreamsPerTile.xls.

|  | All Intra HE |  |  | Random Access HE |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0,2 | 0,0 | -0,4 | 1,3 | 1,1 | 0,5 |
| Class B | 0,2 | 0,1 | -0,2 | 0,8 | 1,3 | 1,0 |
| Class C | 0,1 | 0,1 | 0,1 | 0,7 | 1,1 | 1,1 |
| Class D | 0,1 | -0,1 | 0,0 | 0,8 | 0,7 | 0,9 |
| Class E | 0,6 | -0,2 | 0,0 |  |  |  |
| Overall | 0,2 | 0,0 | -0,1 | 0,9 | 1,1 | 0,9 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Low delay B HE |  |  | Low delay P HE |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 1,3 | 2,4 | 1,9 | 1,0 | 1,2 | 1,4 |
| Class C | 0,9 | 1,6 | 1,3 | 0,7 | 0,8 | 0,7 |
| Class D | 0,9 | 1,0 | 1,0 | 0,8 | 0,7 | 0,5 |
| Class E | 2,7 | 2,8 | 1,4 | 2,5 | 1,9 | 1,3 |
| Overall | 1,4 | 1,9 | 1,4 | 1,2 | 1,1 | 1,0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

The results obtained are identical to the results provided by Orange Labs.

# Software analysis

The software provided by Orange Labs was analyzed. The implemented algorithm corresponds to the technique described in JCTVC-F274.

The following observations have been made:

- The WPP technique for tiles has been implemented on top of the software HM3.0 with tiles provided by eBrisk.

- In order to limit memory allocations, the current software is limited to a maximum number of four tiles.

- The current implementation causes the software to crash when CAVLC is used.

- In the current implementation, each LCU is processed in the same order as in the code with tiles. The WPP technique only changes the writing of the bits into the bitstream, which enables parallel processing. The current implementation does not use multithreading.

- The source code is clean and readable.

# References

- JCTVC-F274 , “Wavefront Parallel Processing for HEVC Encoding and Decoding”, Gordon Clare, Félix Henry, Stéphane Pateux, 6th JCT-VC Meeting, Torino, Juli 2011