| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F528 |
| --- | --- |

| Title: | Simplified Bilateral Intra Smoothing Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.), Nam Ling (Santa Clara University), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), | Tel: Email: | +1-408-554-4794 g1li@scu.edu lingzhi.liu@huawei.com nling@scu.edu zhengjianhua@huawei.com pzhang@huawei.com |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd. |  |  |

_____________________________

# Abstract

This proposal provides a method to improve the performance of Intra smoothing filtering in High Efficiency Video Coding (HEVC).  The proposed technology employs a simplified bilateral filter to adaptively adjust filter coefficients to preserve edges while smoothing.

# Introduction

The Mode Dependent Intra Smoothing (MDIS) is used in current HEVC Test Model (HM). In the current design of HM, a 3-tap filter with coefficients of [1 2 1] is used for intra smoothing.  The problem of this design is that the same filter is applied for reference pixels regardless of edges.  In this proposal, a simplified bilateral filter is used instead of the original 3-tap filter.  It improves the coding efficiency, especially on pictures with strong edges.

# Simplified Bilateral Filter

The original design of bilateral filters has high computational complexity. A simplified design is proposed for efficiently performing intra smoothing in HEVC while reducing loss caused by smoothing on strong edges.

In the proposed filtering process, a range filter and a domain filter are combined and simplified, and then applied to the pixels.

Figure 1 – Diagram of the filtering process

## Range Filter

The original Bilateral Filter consists of a range filter and a domain filter. Both are nonlinear Gaussian filters, which require complex floating-point computations. This proposed method uses linear approximation of the range filter and a simple fix coefficient filter as the domain filter.

The proposed range filter is a linear approximation of the range filter in the original bilateral filter design. The range filter’s coefficients are related to the photometric similarity of the current pixel and the weighted nearby pixels. The coefficient can be linearly calculated with the difference of the neighboring pixel value and the maximum value of the pixel, in the range between 1 and k*(Max - Difference)/Max. k is a constant which determines the degree of filtering, Max is the Maximum value of a pixel. Depending on the value of k, the filter could be either a smoothing or a sharpening filter.

Let P(n) denote the current pixel, Max is the maximum value of a pixel value,

The difference between the current pixel and the left neighboring pixel value:

DL = |P(n) – P(n-1)|;

The difference between the current pixel and the right neighboring pixel value:

DR = |P(n) – P(n+1)|;

Thus, the range filter’s coefficients can be denoted as:

## Domain Filter

The domain filter is a smoothing filter applied according to the geometric closeness of the weighted pixels. The coefficients may vary according to the window size and the distance between the weighted pixel and current pixel.

In the current implementation, window size is fixed as 1, a 3-tap smoothing filter of [1 2 1] is used.

## Combination and Simplification of the filters

Combining the Range filter and the domain filter, the coefficients can be simplified as:

Therefore, the output value can be denoted as:

## Simplification of the Computation

The calculation of the filtered value involves division operation, which has much higher complexity. Since all the variables in the calculation of F(n) are integers, the calculation can be further simplified.

;

;

For integer division, a rounding factor should be added to the dividend,

In the implementation, k is set as a fixed value of 2. The integer division can be further simplified when implemented with only shifting and additions.

# Simulation Result

In the simulation, HM-3.1 Anchor and the proposed Intra Smoothing tool implemented based on HM-3.1 were used. Comparisons were made for 2 cases:

- HM-3.1 with QC_MDIS = 0 compared with Proposed Intra Smoothing

- HM-3.1 Anchor compared with Proposed Intra Smoothing

The R-D performance comparisons are shown in the following tables:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.35 | -0.39 | -0.48 | -0.50 | -0.40 | -0.61 |
| Class B | -0.23 | -0.28 | -0.35 | -0.31 | -0.23 | -0.25 |
| Class C | -0.01 | -0.10 | -0.04 | 0.20 | -0.01 | 0.01 |
| Class D | -0.16 | -0.25 | -0.22 | -0.17 | -0.16 | -0.23 |
| Class E | -0.34 | -0.54 | -0.53 | -0.52 | -0.49 | -0.46 |
| Overall | -0.21 | -0.30 | -0.32 | -0.25 | -0.24 | -0.30 |
| Enc Time[%] | 103% |  |  | 105% |  |  |
| Dec Time[%] | 103% |  |  | 103% |  |  |

Table-1 R-D performance of HM-3.1 with MDIS off vs. Proposed Intra Smoothing

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.04 | 0.05 | 0.14 | 0.14 | 0.03 | 0.09 |
| Class B | -0.04 | -0.07 | -0.15 | 0.02 | -0.03 | -0.08 |
| Class C | -0.14 | -0.15 | -0.10 | -0.15 | -0.17 | -0.16 |
| Class D | -0.12 | -0.12 | -0.07 | -0.07 | -0.05 | -0.12 |
| Class E | -0.03 | -0.01 | -0.01 | 0.07 | -0.01 | -0.01 |
| Overall | -0.06 | -0.06 | -0.04 | 0.00 | -0.05 | -0.06 |
| Enc Time[%] | 102% |  |  | 102% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |

Table-2 R-D performance of HM-3.1 Anchor vs. Proposed Intra Smoothing

To check the performance of the tool on sequences with strong edges, the R-D performance on “BasketballDrill” sequence is shown below.

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| BasketballDrill | -0.4 | -0.5 | -0.4 | -0.6 | -0.6 | -0.5 |

Table-3 R-D performance of HM-3.1 Anchor vs. Proposed Intra Smoothing on “BasketballDrill” Sequence

# Conclusion

The simulation results show that proposed simplified Bilateral Intra smoothing filter in MDIS performs better than the original MDIS. On the sequence with strong edges, the proposed Intra smoothing tool outperforms the original MDIS much more than average.

# Reference

- J. Zhao, A. Segall, “On Intra Coding and MDIS”, JCT-VC Document JCTVC-E437, Geneva, March 2011.

- Y. Zheng, et al., “CE13: Mode Dependent Hybrid Intra Smoothing”, JCT-VC Document JCTVC-D282, Daegu, January 2011.

- C. Tomasi and R. Manduchi, “Bilateral filtering for gray and color images,” in Proc. 6th Int. Conf. Computer Vision, New Delhi, India, pp. 839–846, 1998.

# Patent rights declaration(s)

Santa Clara University and Hisilicon Technologies Co. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).