| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F529_r2 WG11 Number: m20960 |
| --- | --- |

| Title: | CE12: cross-check of SKT/SKKU deblocking filter (F258) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Kenneth Andersson, Andrey Norkin, Rickard Sjöberg Färögatan 6 16480 Stockholm | Tel: Email: | +46 10 7141538 kenneth r andersson @ ericsson com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This is a cross-verification of SKT/SKKU’s deblocking filter in JCTVC-F258. The software consists of several modifications compared to HM-3.0. The proposal has been verified successfully for the common conditions for BDR, PSNR, bit rate, and decoding time.

# Software

The software has been compiled successfully after a fix regarding unused variables (bAtCUBoundary, bAtCUHalf) with gcc 4.3.2 on Linux and Visual Studio for Windows.

Modifications compared to HM-3.0:

- The decision between the strong and weak filter is modified. An additional condition has been added which is identical to the current condition for deblocking filtering of the 8-pel block boundary but shifted one pixel away from the block boundary.

- Bs derivation for luma is modified. Luma is assigned Bs value 3 if both blocks are intra and have similar intra mode direction. Otherwise, if both blocks are intra they are assigned Bs value 4.

- The tc_offset is modified (value 3 is assigned for Bs = 4 and 1 for Bs = 3).

- The luma filter operation is modified.

- For luma component, a decision between filtering one or two pixels from the block boundary is done separately for two adjacent blocks. If only one pixel from the block boundary is filtered, the is is modified with the value (+/- delta/2), if two pixels are filtered, the first pixel is modified with the value (+/- delta).

- Chroma is assigned Bs value 3 if both blocks are intra and have same intra mode otherwise if both blocks are intra they are assigned Bs value 4.

- Chroma inter blocks are assigned Bs value 2 if at least one of the blocks contain coefficients. Chroma blocks with Bs value above 1 are filtered.

- Luma and chroma blocks with Bs value 4 have tc offset set to 3, and blocks with Bs value 3 have tc_offset 1.

There are three defines for luma, chroma and luma strong filter modifications (DF_SKKU_LUMA, DF_SKKU_CHROMA, DF_SKKU_STRONG). The combination with all three defines set to 1 has been tested.

# Results

## BDR performance

The software has been used for encoding and decoding according to common conditions [2] on a Linux cluster and the average luma BDR [3] compared to HM-3.0 is -1.1%.

Table I BDR performance HM-3.0 vs for SKKU deblocking

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -1,6962 | -0,5497 | -0,4735 | -1,0971 | -0,6267 | -0,5170 |
| Class B | -1,3074 | -0,5257 | -0,4657 | -1,0874 | -0,6987 | -0,5661 |
| Class C | -0,9791 | -0,7627 | -0,7368 | -0,9742 | -0,8516 | -0,8312 |
| Class D | -0,8694 | -0,6642 | -0,6905 | -0,8749 | -0,7315 | -0,6822 |
| Class E | -1,2861 | -0,0488 | -0,1192 | -1,0084 | -0,0152 | -0,0646 |
| Overall | -1,2287 | -0,5341 | -0,5145 | -1,0123 | -0,6189 | -0,5573 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -1,4863 | -0,3595 | -0,1473 | -0,8594 | 0,2404 | 0,5660 |
| Class B | -1,2815 | -0,8452 | -0,7860 | -0,9898 | -0,6849 | -0,5295 |
| Class C | -0,9543 | -0,6259 | -0,7124 | -0,8434 | -0,4116 | -0,6228 |
| Class D | -0,8033 | -0,7675 | -0,8321 | -0,6671 | -0,3995 | -0,4236 |
| Class E |  |  |  |  |  |  |
| Overall | -1,1402 | -0,6610 | -0,6293 | -0,8487 | -0,3358 | -0,2688 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -1,6280 | -1,1689 | -1,3843 | -0,9569 | -1,0591 | -0,7418 |
| Class C | -1,1417 | -0,5850 | -0,8445 | -0,7632 | 0,4410 | -0,2450 |
| Class D | -1,0048 | -0,2980 | -0,2904 | -0,7305 | -0,1242 | -0,2364 |
| Class E | -2,9760 | -2,7333 | -2,7910 | -1,6030 | -4,6599 | -3,7883 |
| Overall | -1,6034 | -1,0985 | -1,2396 | -0,9730 | -1,1255 | -1,0625 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

## Subjective inspection

The subjective inspection of a number of sequences has shown that the subjective quality is sometimes worse and sometimes better than that of the anchor. On average, the subjective quality was found to be similar to that of the anchor.

## Decoding time

| Configuration | Dec. time SKT/SKKU [s] | Dec. time HM3.0 [s] | Dec. SKT/SKKU [%] |
| --- | --- | --- | --- |
| Random access | 3246.29 | 3249.80 | 100% |
| Low delay | 2295.89 | 2320.95 | 99% |
| All intra | 7314.47 | 7286.33 | 100% |
| Random access loco | 2107.08 | 2083.71 | 101% |
| Low delay loco | 1502.35 | 1486.62 | 101% |
| All intra loco | 4151.23 | 4122.13 | 101% |
| Average |  |  | 100% |

# References

- J. Yang, K. Won, B. Jeon, ” CE12: SK Telecom/SKKU Deblocking Filter”, JCTVC-F258, Torino, July 2011.

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-E700, Geneva, March 2011

- G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.