| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F530_r2 WG11 Number: m20961 |
| --- | --- |

| Title: | CE12: Cross-verification of deblocking filter parameter adjustment on a slice level (F143) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a part of the core experiment on deblocking loop filtering (CE12) and it cross-verifies proposal F143 (Toshiba/Qualcomm) on deblocking parameter adjustment on the slice level. Three variants of parameter modifications have been proposed with the reported average luma BD-rate reduction of 0.1%, 0.4% and 0.3% for variants (a), (b) and (c) respectively on six standard configurations. The results are confirmed by the cross-verifier.

# Software and cross-verification

The software has been compiled successfully with gcc 4.3.2 on Linux and Visual Studio for Windows. The provided software has three different variants: (a)-(c) with different adjustment of the deblocking filter parameters, which are operated by the following macro definitions in TypeDef.h:

#define NEWDEBLOCKING_OFFSET    1

#if NEWDEBLOCKING_OFFSET

#define CHROMA_OFFSET           1

#define INTRA_TC_OFFSET         1

#endif

The following modifications of tc(QP) values have been done for intra and inter block boundaries for three proposed variants (a)-(c):

(a) Intra: (QP+4)-1, Inter: QP-1

in TypeDef.h,		#define NEWDEBLOCKING_OFFSET		1

#define CHROMA_OFFSET           	1

#define INTRA_TC_OFFSET		0

in config file,		LoopFilterNewTcOffset :		-1

(b) Intra: (QP+4)-2, Inter: QP-2

in TypeDef.h,		#define NEWDEBLOCKING_OFFSET		1

in TypeDef.h,		#define CHROMA_OFFSET           	1

in TypeDef.h,		#define INTRA_TC_OFFSET		0

in config file,		LoopFilterNewTcOffset :		-2

(c) Intra: (QP+3)-1, Inter: QP-1

in TypeDef.h,		#define NEWDEBLOCKING_OFFSET		1

in TypeDef.h,		#define CHROMA_OFFSET           	1

in TypeDef.h,		#define INTRA_TC_OFFSET		1

in config file,	LoopFilterNewTcOffset :			-1

Parameter tc in HM3.0 depends on the QP and is obtained from the table tc[QP]. For intra blocks, a value of tc[QP+tc_offset] is used, where tc_offset is equal to 4 in the current deblocking.

The software includes the following modifications. Macro-definition NEWDEBLOCKING_OFFSET introduces another new_tc_offset with the tc parameter for luma deblocking found as tc[QP+tc_offset+new_tc_offset], where the value for the new_tc_offset is sent in the slice header.

Another modifications of the source code is CHROMA_OFFSET, which applies the same computation of tc for chroma tc as for luma by using the same new_tc_offset parameter as for luma.

The modification INTRA_TC_OFFSET modifies the value of tc_offset (for intra block boundaries) from 4 to 3 for both luma and chroma components.

# Objective performance

The objective performance of the cross-verified method is shown in the table below. The decoding time on a single computer is reported for case (c) in the next section. The scheme has been compared against the HM3.0 anchor. The comparison results are reported in Table 1 - Table 3. The obtained results perfectly match the results provided by the proponents.

Table 1 Comparison of F143 vs HM3.0 as anchor, case (a)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.5 | -0.5 | -0.2 | -0.6 | -0.5 |
| Class B | -0.5 | -0.5 | -0.5 | -0.3 | -0.7 | -0.6 |
| Class C | -0.3 | -0.7 | -0.7 | -0.2 | -0.8 | -0.8 |
| Class D | -0.3 | -0.7 | -0.7 | -0.2 | -0.7 | -0.7 |
| Class E | -0.4 | 0.0 | -0.1 | -0.2 | 0.0 | -0.1 |
| Overall | -0.4 | -0.5 | -0.5 | -0.2 | -0.6 | -0.6 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -0.5 | -0.2 | -0.1 | -0.5 | -0.3 |
| Class B | -0.2 | -0.5 | -0.4 | -0.1 | -0.5 | -0.5 |
| Class C | -0.1 | -0.5 | -0.6 | -0.1 | -0.6 | -0.7 |
| Class D | -0.1 | -0.4 | -0.4 | 0.0 | -0.6 | -0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.1 | -0.5 | -0.4 | -0.1 | -0.5 | -0.5 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.1 | -0.2 | -0.2 |
| Class C | 0.0 | -0.2 | -0.3 | 0.0 | -0.1 | -0.2 |
| Class D | 0.0 | 0.3 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class E | 0.1 | -0.1 | 0.1 | 0.1 | -0.2 | -0.3 |
| Overall | 0.0 | 0.0 | -0.1 | 0.0 | -0.1 | -0.2 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.1 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.3 | 0.0 | -0.1 | -0.2 |
| Class D | 0.0 | -0.2 | 0.1 | 0.0 | -0.1 | 0.2 |
| Class E | 0.0 | 0.2 | -0.6 | 0.2 | 0.1 | -0.1 |
| Overall | 0.0 | 0.0 | -0.2 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

Table 2 Comparison of F143 vs HM3.0 as anchor, case (b)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9 | -0.5 | -0.5 | -0.6 | -0.6 | -0.5 |
| Class B | -0.8 | -0.5 | -0.5 | -0.6 | -0.7 | -0.6 |
| Class C | -0.5 | -0.7 | -0.7 | -0.5 | -0.9 | -0.8 |
| Class D | -0.5 | -0.7 | -0.7 | -0.4 | -0.7 | -0.7 |
| Class E | -0.7 | 0.0 | -0.1 | -0.5 | 0.0 | -0.1 |
| Overall | -0.7 | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.6 | -0.7 | -0.7 | -0.3 | -0.6 | -0.4 |
| Class B | -0.5 | -0.7 | -0.5 | -0.4 | -0.7 | -0.7 |
| Class C | -0.3 | -0.7 | -0.8 | -0.3 | -0.9 | -1.0 |
| Class D | -0.2 | -0.5 | -0.7 | -0.2 | -0.8 | -0.7 |
| Class E |  |  |  |  |  |  |
| Overall | -0.4 | -0.6 | -0.7 | -0.3 | -0.8 | -0.7 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | 0.0 | -0.2 | -0.3 | -0.4 | -0.7 |
| Class C | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.4 |
| Class D | -0.3 | 0.1 | 0.3 | -0.3 | -0.4 | -0.3 |
| Class E | -0.4 | -0.4 | 0.1 | -0.4 | -0.5 | -0.6 |
| Overall | -0.3 | -0.1 | 0.0 | -0.3 | -0.4 | -0.5 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.3 | -0.2 | -0.2 | -0.1 |
| Class C | -0.2 | -0.2 | -0.4 | -0.2 | -0.2 | -0.3 |
| Class D | -0.2 | -0.3 | -0.7 | -0.2 | -0.2 | -0.6 |
| Class E | -0.6 | 0.2 | -0.5 | -0.2 | -0.3 | -0.5 |
| Overall | -0.3 | -0.1 | -0.5 | -0.2 | -0.2 | -0.3 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

Table 3 Comparison of F143 vs HM3.0 as anchor, case (c)

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.9 | -0.5 | -0.5 | -0.6 | -0.6 | -0.5 |
| Class B | -0.8 | -0.5 | -0.5 | -0.6 | -0.7 | -0.6 |
| Class C | -0.5 | -0.7 | -0.7 | -0.5 | -0.9 | -0.8 |
| Class D | -0.5 | -0.7 | -0.7 | -0.5 | -0.7 | -0.7 |
| Class E | -0.7 | 0.0 | -0.1 | -0.5 | 0.0 | -0.1 |
| Overall | -0.7 | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | -0.5 | -0.5 | -0.6 | -0.2 | -0.7 | -0.5 |
| Class B | -0.4 | -0.7 | -0.6 | -0.3 | -0.7 | -0.7 |
| Class C | -0.2 | -0.7 | -0.7 | -0.2 | -0.9 | -1.0 |
| Class D | -0.2 | -0.6 | -0.7 | -0.2 | -0.8 | -0.7 |
| Class E |  |  |  |  |  |  |
| Overall | -0.3 | -0.6 | -0.6 | -0.2 | -0.8 | -0.7 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.1 | -0.1 | 0.0 | -0.3 | -0.4 |
| Class C | -0.1 | -0.2 | -0.3 | 0.0 | -0.2 | -0.5 |
| Class D | -0.1 | -0.3 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class E | 0.0 | -0.3 | -0.2 | -0.1 | 0.0 | -0.3 |
| Overall | -0.1 | -0.2 | -0.1 | 0.0 | -0.2 | -0.3 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.1 | -0.4 | 0.1 | -0.2 | 0.0 |
| Class C | -0.1 | -0.1 | -0.2 | 0.0 | -0.2 | -0.3 |
| Class D | -0.1 | -0.2 | -0.2 | 0.0 | -0.2 | -0.4 |
| Class E | -0.2 | 0.2 | -0.9 | 0.0 | 0.3 | -0.2 |
| Overall | -0.1 | -0.1 | -0.4 | 0.0 | -0.1 | -0.2 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Complexity considerations

In order to evaluate the complexity changes of the F143, the decoding time on a single machine has been measured for both the proposal case (c) and the anchor. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time.

Table 4 Decoding time of F143 vs HM3.0 decoding time, case (c)

| Configuration | Dec. time proposal [s] | Dec. time HM3.0 [s] | Dec. time proposal [%] |
| --- | --- | --- | --- |
| Random access | 3240.35 | 3249.80 | 100% |
| Low delay | 2318.14 | 2320.95 | 100% |
| All intra | 7274.56 | 7286.33 | 100% |
| Random access loco | 2091.51 | 2083.71 | 100% |
| Low delay loco | 1491.17 | 1486.62 | 100% |
| All intra loco | 4118.25 | 4122.13 | 100% |
| Average |  |  | 100% |

Table 4 shows that the cross-verified method has similar complexity as HM3.0 anchor.

# Subjective performance

A number of sequences from variant (c) have been compared subjectively to HM3.0 anchor. The subjective quality has been found similar to that of the anchor on average (often similar, sometimes slightly better, sometimes slightly worse).

# References

- T. Yamakage, S. Asaka, T. Chujoh, M. Karczewicz, I. S. Chong, " CE12: Deblocking filter parameter adjustment in slice level ", JCTVC-F143, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC Meeting, Geneva, March 2010.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.