| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F531 WG11 Number: m20962 |
| --- | --- |

| Title: | Cross-verification of MediaTek's line buffer reduction proposal (F053) by Ericsson |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-verification |  |  |
| Author(s) or Contact(s): | Andrey Norkin Färögatan 6 164 80 Stockholm Sweden | Tel: Email: | +46 10 714 84 02 andrey.norkin@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The contribution is a cross-verification of MediaTek’s proposal F053 on reduction of line buffer. Two methods of buffer reduction have been proposed. The first method has a BD-rate change 0.0% on average for six standard configurations while the second method results in a 0.2% BD-rate increase. The cross-verifier confirms the BDR and decoding time results.

# Software and cross-verification

The contribution F053 contains two methods for reducing the pixel line buffer and the software for both two methods has been cross-verified. The software method 1 is turned on by the following macro-definition in TComLoopFilter.h

#define DF_7_5 1

The software for method 2 is turned on by the macro-definition in TypeDef.h.

#define MTK_DISABLE_DF_LCU_BOUNDARY_3	1

The software has been compiled successfully with gcc 4.3.2 on Linux and Visual Studio for Windows.

The software has been studied and found to correspond to the description provided in F053. Both methods only modify filtering of horizontal LCU boundaries.

Method 1 uses pixels after deblocking of the vertical edges for the deblocking filtering decisions on the horizontal LCU boundaries. HM3.0 instead uses the non-deblocked pixels and needs therefore to support two line buffers for filtering the horizontal LCU boundaries.

Method 2 also modifies filtering of the horizontal LCU boundaries. HM3.0 uses the non-deblocked pixels from the buffer and needs therefore to support two line buffers for filtering of horizontal LCU boundaries. It is proposed to avoid modifying the pixels from the upper LCU when performing deblocking of the horizontal LCU boundaries. The deblocking decisions for the block boundary are based on the pixels of the lower block. In the deblocking filter operation, only the first pixel from the block boundary in the upper block is used. The values of the other pixels of the upper block are replaced by the value of the first pixel from the block boundary.

# Objective performance

The objective performance of the cross-verified method is shown in the table below. The decoding time on a single computer is reported in the next section. The scheme has been compared against the HM3.0 anchor. The comparison results are reported in Table 1 and Table 2. The obtained BDR results perfectly match the results provided by the proponents.

Table 1 Comparison of F053 vs HM3.0 as anchor, method 1

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | 0.1 |
| Class B | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.2 | 0.0 | 0.0 | -0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.2 | -0.2 |
| Class D | 0.0 | -0.2 | 0.5 | 0.0 | 0.2 | 0.2 |
| Class E | 0.1 | 0.5 | 0.1 | 0.1 | 0.0 | -0.2 |
| Overall | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | -0.1 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

Table 2 Comparison of F053 vs HM3.0 as anchor, method 2

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.3 | 0.3 | 0.2 | 0.4 | 0.4 |
| Class B | 0.2 | 0.5 | 0.6 | 0.3 | 0.5 | 0.6 |
| Class C | 0.1 | 0.4 | 0.4 | 0.1 | 0.4 | 0.5 |
| Class D | 0.1 | 0.3 | 0.3 | 0.1 | 0.4 | 0.4 |
| Class E | 0.3 | 0.6 | 0.7 | 0.3 | 0.8 | 0.8 |
| Overall | 0.2 | 0.4 | 0.5 | 0.2 | 0.5 | 0.5 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Random Access HE |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A | 0.2 | 0.4 | 0.3 | 0.3 | 0.3 | 0.5 |
| Class B | 0.3 | 0.6 | 0.6 | 0.3 | 0.6 | 0.6 |
| Class C | 0.2 | 0.5 | 0.5 | 0.2 | 0.4 | 0.4 |
| Class D | 0.1 | 0.3 | 0.3 | 0.1 | 0.3 | 0.3 |
| Class E |  |  |  |  |  |  |
| Overall | 0.2 | 0.5 | 0.4 | 0.2 | 0.4 | 0.5 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.4 | 0.3 | 0.4 | 0.1 | 0.0 |
| Class C | 0.2 | 0.2 | 0.3 | 0.2 | 0.2 | 0.3 |
| Class D | 0.1 | 0.2 | 0.4 | 0.1 | 0.1 | 0.1 |
| Class E | 0.9 | 0.6 | 0.9 | 0.7 | 0.5 | 0.2 |
| Overall | 0.3 | 0.3 | 0.4 | 0.3 | 0.2 | 0.1 |
| Enc Time[%] | N/A |  |  | N/A |  |  |
| Dec Time[%] | N/A |  |  | N/A |  |  |

# Complexity considerations

In order to evaluate the complexity changes of the F053, the decoding time on a single computer has been measured for both methods. The time has been obtained without writing decoded pictures on the disk in order to minimize the influence of the I/O time.

Table 3 Decoding time of F053 vs HM3.0 decoding time, method 1

| Configuration | Dec time proposal (geom.mean) [s] | Dec. time HM3.0 (geom. mean) [s] | Dec. proposal [%] |
| --- | --- | --- | --- |
| Random access | 25.40467445 | 25.41744958 | 100% |
| Low delay | 21.27852867 | 21.24478118 | 100% |
| All intra | 58.55175 | 58.4686 | 100% |
| Random access loco | 17.55841492 | 17.39727287 | 101% |
| Low delay loco | 14.71428581 | 14.57582397 | 101% |
| All intra loco | 34.36153664 | 34.21577 | 100% |
| Average |  |  | 100% |

Table 4 Decoding time of F053 vs HM3.0 decoding time, method 2

| Configuration | Dec time proposal (geom.mean) [s] | Dec. time HM3.0 (geom. mean) [s] | Dec. proposal [%] |
| --- | --- | --- | --- |
| Random access | 25.65010798 | 25.41744958 | 101% |
| Low delay | 21.58205197 | 21.24478118 | 102% |
| All intra | 59.09304 | 58.4686 | 101% |
| Random access loco | 17.69455041 | 17.39727287 | 102% |
| Low delay loco | 14.826281 | 14.57582397 | 102% |
| All intra loco | 34.70336217 | 34.21577 | 101% |
| Average |  |  | 101% |

The decoding time tables indicate that method 1 has a similar decoding time as the anchor while there is a slight increase in decoding time for method 2.

# Subjective performance

The subjective quality of both methods has been tested against the HM3.0 anchor.

The subjective quality of method 1 has been found similar to that of the anchor on the studied sequences.

Regarding method 2, subjective differences have been observed on some sequences in terms of distinct slightly more visible horizontal block boundaries and somewhat more trailing compared to the anchor.

# References

- C.-W. Hsu, J. An, X. Guo, J.-L. Lin, Y.-W. Huang, S. Lei, “Deblocking Filter with Reduced Pixel Line Buffers for LCU-based Processing”, JCTVC-F053, Torino, July 2011.

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-E700, JCT-VC Meeting, Geneva, March 2010.

- Gisle Bjontegaard, “Calculation of average PSNR differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.