| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F532 |
| --- | --- |

| Title: | CE6.b Test Summary and spec text of SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Report |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Yongbing Lin, HiSilicon Haitao Yang, Huawei Guichun Li, Santa Clara University Yunfei Wang, Tsinghua Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei Feng Wu, Microsoft | Email: | laichangcai@huawei.com jzxu@microsoft.com haoping.yu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei, Santa Clara University |  |  |

_____________________________

# Abstract

This proposal reports the summary of CE6.b test results, which including the results for each harmonization of SDIP and one of the tool in HM. And the result of combining all harmonization together did by the listed proponents is also reported. In addition, the spec text of related harmonization is provided based on the HM3.0-SDIP software, which can be easily referenced by each harmonization case.

# Tests summary

The work related to CE6.b was done based on Section 4.2 in the latest version of JCTVC-E706-v4[1], i.e. CE6.b: Short Distance Intra Prediction (SDIP) Harmonization. The goal of this subset of CE6 is to complete and provide a harmonization solution that unifies the adopted SDIP features together with the remaining and existing HM tools from the conclusion of the 5th JCTVC meeting, test and confirm the SDIP performance in the harmonization software, revise and verify and confirm the potential WD test document matches the resulting new HM-SDIP software.

The software integration of SDIP and HM-3.0-dev was completed on May 5 and the new software was uploaded into the HM-3.0-SDIP branch on the HM software repository server. The descriptions of the harmonization subjects were finalized through email discussions among the CE participants and the latest version of the CE document was uploaded to the JCTVC document site on May 19. The cross verification started on June 23, and below is a summary of the test results.

# Results summary

#### Test1: Harmonization of SDIP and RQT

Case1:

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.02 | 0.11 | 0.21 | -0.17 | -0.66 | -0.50 |
| Class B | -0.02 | 0.20 | 0.23 | -0.22 | -0.55 | -0.54 |
| Class C | -0.03 | 0.22 | 0.24 | -0.26 | -0.49 | -0.47 |
| Class D | -0.03 | 0.23 | 0.18 | -0.23 | -0.36 | -0.33 |
| Class E | -0.03 | 0.25 | 0.25 | -0.31 | -1.09 | -1.10 |
| All | -0.03 | 0.20 | 0.22 | -0.24 | -0.61 | -0.56 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |

Case 2:

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.03 | -0.02 | 0.12 | -0.16 | -0.62 | -0.46 |
| Class B | -0.21 | -0.22 | -0.18 | -0.57 | -0.66 | -0.63 |
| Class C | -0.06 | 0.12 | 0.12 | -0.32 | -0.51 | -0.50 |
| Class D | -0.09 | 0.14 | 0.12 | -0.31 | -0.37 | -0.34 |
| Class E | -0.47 | -0.46 | -0.42 | -0.86 | -1.33 | -1.14 |
| All | -0.16 | -0.08 | -0.04 | -0.43 | -0.66 | -0.59 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

#### Test2: Harmonization of SDIP and Mode-Dependent DCT/DST

Case1 (positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.67 | 0.19 | 0.19 | 0.71 | 0.36 | 0.44 |
| Class B | 0.66 | 0.33 | 0.28 | 0.85 | 0.54 | 0.47 |
| Class C | 0.81 | 0.58 | 0.60 | 1.32 | 0.77 | 0.67 |
| Class D | 0.88 | 0.76 | 0.77 | 1.37 | 0.78 | 0.74 |
| Class E | 0.99 | 0.58 | 0.80 | 1.33 | 0.50 | 0.67 |
| Overall | 0.79 | 0.47 | 0.50 | 1.09 | 0.59 | 0.59 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Case2(positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.56 | 0.06 | 0.05 | 0.49 | 0.26 | 0.28 |
| Class B | 0.52 | 0.13 | 0.02 | 0.54 | 0.30 | 0.22 |
| Class C | 0.55 | 0.29 | 0.20 | 0.80 | 0.38 | 0.30 |
| Class D | 0.58 | 0.36 | 0.33 | 0.86 | 0.38 | 0.39 |
| Class E | 0.78 | -0.02 | 0.21 | 0.85 | 0.01 | 0.18 |
| Overall | 0.59 | 0.17 | 0.15 | 0.69 | 0.28 | 0.28 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

Case3:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.02 | -0.19 | -0.19 | -0.09 | -0.14 | -0.20 |
| Class B | -0.02 | -0.12 | -0.17 | -0.06 | -0.11 | -0.16 |
| Class C | -0.02 | -0.20 | -0.19 | -0.08 | -0.17 | -0.20 |
| Class D | -0.02 | -0.13 | -0.16 | -0.03 | -0.11 | -0.13 |
| Class E | -0.09 | -0.42 | -0.51 | -0.22 | -0.60 | -0.56 |
| Overall | -0.03 | -0.20 | -0.23 | -0.09 | -0.20 | -0.23 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

#### Test3: Harmonization of SDIP and MDCS (positive means gain here)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.06 | 0.05 | 0.03 | 0.08 | 0.10 |
| Class B | 0.05 | 0.05 | 0.03 | 0.06 | 0.10 | 0.13 |
| Class C | 0.16 | 0.16 | 0.17 | 0.15 | 0.18 | 0.20 |
| Class D | 0.18 | 0.19 | 0.16 | 0.11 | 0.09 | 0.11 |
| Class E | 0.02 | -0.05 | -0.04 | 0.02 | 0.10 | 0.05 |
| All | 0.08 | 0.09 | 0.08 | 0.08 | 0.11 | 0.12 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

#### Test4: Harmonization of LM mode and the chroma prediction in SDIP

1.Microsoft test case ( positive means gain here):

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.01 | 0.34 | 0.38 | 0.03 | 1.02 | 0.54 |
| Class B | 0.03 | 0.52 | 0.52 | 0.07 | 0.76 | 0.68 |
| Class C | 0.13 | 1.03 | 0.99 | 0.21 | 1.27 | 1.28 |
| Class D | 0.09 | 0.64 | 0.54 | 0.13 | 0.85 | 0.69 |
| Class E | 0.02 | 0.50 | 0.49 | 0.06 | 0.93 | 1.07 |
| All | 0.06 | 0.61 | 0.59 | 0.10 | 0.96 | 0.83 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

2. LGE provided the results for another test case as shown in[xxx]

#### Test5: Harmonization of SDIP and MDIS

1. HiSilicon & Santa Clara University test cases

Case1(positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.40 | 0.38 | 0.70 | 0.65 | 0.45 | 0.75 |
| Class B | 0.19 | 0.27 | 0.25 | 0.33 | 0.19 | 0.18 |
| Class C | -0.13 | -0.04 | 0.00 | -0.34 | -0.13 | -0.18 |
| Class D | 0.06 | 0.10 | 0.16 | 0.10 | 0.10 | 0.08 |
| Class E | 0.31 | 0.51 | 0.52 | 0.59 | 0.46 | 0.43 |
| Overall | 0.16 | 0.23 | 0.31 | 0.25 | 0.20 | 0.24 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Case2(positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.37 | 0.22 | 0.51 | 0.53 | 0.25 | 0.47 |
| Class B | 0.20 | 0.27 | 0.23 | 0.29 | 0.21 | 0.25 |
| Class C | -0.04 | 0.09 | 0.11 | -0.06 | 0.10 | 0.04 |
| Class D | 0.06 | 0.19 | 0.12 | 0.12 | 0.10 | 0.08 |
| Class E | 0.36 | 0.42 | 0.30 | 0.59 | 0.39 | 0.38 |
| Overall | 0.18 | 0.23 | 0.25 | 0.28 | 0.20 | 0.24 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 97% |  |  |

Case3:

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.04 | -0.19 | -0.14 | -0.12 | -0.04 | -0.12 |
| Class B | 0.00 | 0.04 | 0.04 | -0.02 | 0.07 | 0.09 |
| Class C | 0.04 | 0.06 | 0.04 | 0.08 | 0.02 | 0.04 |
| Class D | -0.01 | 0.03 | -0.05 | -0.01 | 0.01 | -0.01 |
| Class E | 0.03 | -0.07 | 0.00 | 0.02 | 0.13 | 0.10 |
| Overall | 0.00 | -0.02 | -0.02 | -0.01 | 0.03 | 0.02 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

2.Qualcomm provided the results for another test case as shown in[xxx]

#### Test6: Harmonization of SDIP and PLANAR

Case1(positive means gain here):

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | -0.5 | -0.2 | 0.5 | -0.1 | 0.3 |
| Class B | 0.7 | 0.2 | 0.2 | 0.5 | 0.4 | 0.4 |
| Class C | 0.3 | 0.0 | 0.0 | 0.4 | 0.3 | 0.3 |
| Class D | 0.4 | 0.0 | 0.0 | 0.4 | 0.4 | 0.4 |
| Class E | 0.8 | 0.1 | -0.2 | 0.7 | 0.3 | 0.3 |
| All | 0.6 | 0.0 | 0.0 | 0.5 | 0.2 | 0.3 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Case2(positive means gain here):

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.6 | -0.4 | -0.2 | 0.5 | 0.1 | 0.5 |
| Class B | 0.8 | 0.3 | 0.3 | 0.5 | 0.6 | 0.6 |
| Class C | 0.5 | 0.1 | 0.0 | 0.4 | 0.4 | 0.5 |
| Class D | 0.4 | 0.1 | 0.0 | 0.4 | 0.5 | 0.5 |
| Class E | 1.1 | 0.1 | 0.1 | 0.7 | 0.8 | 0.8 |
| All | 0.6 | 0.0 | 0.1 | 0.5 | 0.5 | 0.6 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Case3:

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class B | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | -0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class E | -0.2 | -0.5 | -0.4 | -0.2 | -0.3 | -0.1 |
| All | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

#### Test7: Harmonization of SDIP and deblocking filter

1: Tsinghua, HiSilicon test case

Case 1:

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | 0.2 | 0.3 | 0.9 | 2.2 | 2.4 |
| Class B | -1.3 | 0.3 | 0.9 | 0.9 | 3.5 | 4.5 |
| Class C | -0.8 | 0.1 | 0.3 | 0.0 | 2.9 | 3.6 |
| Class D | -0.6 | 0.3 | 0.4 | -0.2 | 3.1 | 3.8 |
| Class E | -0.7 | 1.3 | 1.5 | 1.4 | 7.1 | 7.5 |
| All | -1.1 | 0.4 | 0.6 | 0.6 | 3.6 | 4.2 |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 86% |  |  |

Case2(positive means gain here):

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | 0.3 | 0.4 | 0.9 | 2.3 | 2.5 |
| Class B | -1.2 | 0.4 | 1.0 | 1.0 | 3.5 | 4.5 |
| Class C | -0.7 | 0.2 | 0.4 | 0.1 | 3.0 | 3.7 |
| Class D | -0.6 | 0.4 | 0.5 | 0.0 | 3.2 | 3.9 |
| Class E | -0.6 | 1.4 | 1.6 | 1.5 | 7.1 | 7.5 |
| All | -1.1 | 0.5 | 0.8 | 0.7 | 3.6 | 4.3 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 95% |  |  | 90% |  |  |

Case3:

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.08 | 0.01 | 0.00 | 0.09 | 0.01 | -0.01 |
| Class B | 0.15 | 0.05 | 0.05 | 0.17 | 0.03 | 0.04 |
| Class C | 0.05 | 0.01 | 0.01 | 0.04 | 0.00 | 0.02 |
| Class D | 0.07 | 0.00 | 0.00 | 0.11 | 0.00 | 0.00 |
| Class E | 0.26 | -0.04 | -0.05 | 0.20 | -0.15 | -0.12 |
| All | 0.12 | 0.01 | 0.01 | 0.12 | -0.01 | -0.01 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

2. Qualcomm provided the results for another test case as shown in[xxx]

#### Test8: Harmonization of SDIP and DC prediction filtering

This harmonization was implemented into HM3.0-SDIP software and tested by Qualcomm[xxx].

Case1(positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.10 | 0.04 | 0.14 | 0.15 | 0.21 | 0.28 |
| Class B | 0.13 | 0.06 | 0.02 | 0.17 | 0.15 | 0.15 |
| Class C | 0.11 | 0.11 | 0.13 | 0.14 | 0.19 | 0.11 |
| Class D | 0.08 | 0.08 | 0.10 | 0.12 | 0.19 | 0.14 |
| Class E | 0.03 | 0.03 | 0.07 | 0.11 | 0.03 | 0.03 |
| Overall | 0.09 | 0.06 | 0.09 | 0.14 | 0.16 | 0.15 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Case 2(positive means gain here):

|  | All Intra HE |  |  | All Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.12 | 0.07 | 0.22 | 0.18 | 0.23 | 0.36 |
| Class B | 0.18 | 0.08 | 0.08 | 0.21 | 0.18 | 0.20 |
| Class C | 0.19 | 0.14 | 0.15 | 0.21 | 0.20 | 0.19 |
| Class D | 0.13 | 0.13 | 0.10 | 0.17 | 0.13 | 0.16 |
| Class E | 0.08 | 0.02 | 0.05 | 0.17 | 0.11 | 0.10 |
| Overall | 0.15 | 0.09 | 0.12 | 0.19 | 0.17 | 0.21 |
| Enc Time[%] | 107% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

#### Additional test: combine all harmonization by proponents in this proposal

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.04 | -0.53 | -0.68 | -0.41 | -1.26 | -1.25 |
| Class B | -0.19 | -0.50 | -0.49 | -0.65 | -0.98 | -1.00 |
| Class C | -0.03 | -0.15 | -0.14 | -0.35 | -0.80 | -0.81 |
| Class D | -0.04 | -0.05 | -0.09 | -0.29 | -0.56 | -0.49 |
| Class E | -0.47 | -1.59 | -1.40 | -1.20 | -2.84 | -2.37 |
| All | -0.14 | -0.51 | -0.52 | -0.55 | -1.20 | -1.12 |
| Enc Time[%] | 101% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 103% |  |  |

# Spec text

The spec text of described all the harmonization did by proponents listed in this proposal based on the HM3.0-SDIP software, thus it can be easily referenced by each harmonization case.

It can be seen in the attached JCTVC-E603_d8_sdip_text_harmonization.doc file.

# References

- M. Budagavi, K. Chono, R. Joshi, et.al., “Core Experiment 6: Intra Prediction Improvement,” ITU-T JCTVC-E706, Geneva, CH, March 2011.

# Patent rights declaration(s)

Huawei, Microsfot, HiSilicon, Tsinghua, USTC and Santa Clara University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).