| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F533 |
| --- | --- |

| Title: | CE6.b Report on test1 Harmonization of SDIP and RQT |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Report |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Yongbing Lin, HiSilicon Haitao Yang, Huawei Yunfei Wang, Tsinghua Jizheng Xu, Microsoft Lingzhi Liu, HiSilicon Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei Feng Wu, Microsoft | Email: | laichangcai@huawei.com jzxu@microsoft.com haoping.yu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This proposal reports the harmonization and results of SDIP and RQT, a straightforward harmonization solution of SDIP and RQT is described in this contribution, which including syntax coding harmonization between RQT and SDIP, and two depth RQT support for 8x32/32x8 PU. Compared with HM3.0-SDIP default configuration without harmonization of RQT and SDIP, coding gains about 0.16% and 0.43% can be achieved for all test sequences under Intra-HE and Intra-LC test condition respectively without encoding or decoding time increase, it can also provide a harmonized spec description for SDIP with the current WD.

# Harmonization of SDIP and RQT

## Harmonization of SDIP and RQT syntax element coding

JCTVC-E278 and its software (dev-HM3.0-sdip) introduced two types of PUs which were named as 2NxhN and hNx2N for CUs whose size is smaller than 64x64, where h means half or 0.5. 2NxhN and hNx2N can also be called as SDIP PUs. 2NxhN includes 32x8, 16x4 and 8x2 PU, and similarly, hNx2N includes 8x32, 4x16 and 2x8 PU for 32x32 to 8x8 CU. 1x16 and 16x1 are the second depth of transform unit of 4x16 and 16x4 PU, respectively, as revealed by Figure.1. However, in JCTVC-E278, the split_transform_flag (named as SDIP_TU_split in JCTVC-E278) was coded in CU level syntax with u(1) or ae(v), and cbf flags of luma or chroma for each TU were coded in transform tree syntax separately with u(1) or ae(v).

In this CE6.b.1 report proposal, not only the source code of SDIP has been implemented into the RQT structure, but also the coding of split_transform_flag and cbf of each TU are harmonized with RQT. At the first depth of transform TU or the cbfs of luma TUs are not all equal to 0, the cbf and transform_split_flag are combining coded with cbp_and_split_transform syntax (vlc(n,v)) when the entropy coding method is LCEC. Since 1x16/16x1 are the second depth of TU at 4x16/16x4 PU, the max RQT depth in SDIP PU is 2 depth, and the chroma proponents always use square partitions and the traditional one depth RQT when luma components of current CU are using SDIP non-square PU, which is the same as JCTVC-E278.

## Two depth RQT extension in SDIP PU

In section 2.1, although 2 depth non-square RQT was supported, the 8x32/32x8 PU actually only use one depth RQT because 8x32 or 32x8 could not be further split in JCTVC-E278. In this section, similar with 4x16/16x4 split into 1x16/16x1, we extended the non-square RQT at 8x32/32x8 PU to support 2 depth non-square RQT. In this extension, the second depth transform unit of 8x32/32x8 PU can be 2x32/32x2 transform unit with the same RQT structure and syntax encoding method in section 2.1. The prediction, scanning and entropy coding are similar with other non square partitions. By this extension, the benefits of non-square RQT can also be explored in 8x32/32x8 PU at the 32x32 CU.

# Simulation results

The proposed harmonization is implemented in the released SDIP software with revision number 866, which can be downloaded from the SDIP branch in the SVN server. The SDIP anchor data is obtained using software without modification.

Intra-HE and Intra LC configurations specified in the common test condition [2] is used for the simulation according to the CE6 description [3].

Thanks xxx for performing cross-verification.

## Harmonization of SDIP and RQT syntax element coding

The purpose of this test is to observe the performance of the harmonization of SDIP and RQT syntax element coding for cbf and split_transform_flag. The anchor data is obtained using the SDIP branch software r866 with original syntax coding method in JCTVC-E278. The proposed data is obtained using the SDIP branch software r866 with harmonized syntax coding method described in section 2.1 and Table-1 is the result.

Table-1 Harmonization of SDIP and RQT syntax

## Two depth RQT extension in SDIP PU

The purpose of this test is to observe the performance of two depth RQT extension in SDIP PU as described in section 2.2. The test is based on the harmonization of SDIP and RQT syntax element coding for cbf and split_transform_flag, which means the harmonization described in section 2.1 is turned ON, and two depth RQT is used in 8x32/32x8 PU is used. The anchor data is obtained using the SDIP branch software r866 with original syntax coding method in JCTVC-E278 which is the same as that of 3.1. The proposed data is obtained using the SDIP branch software r866 with harmonized syntax coding method and two depth RQT is used in 8x32/32x8 PU as described in section 2.1 and section2.2. Table-2 is the result.

Table-2 Two depth RQT of SDIP compared with HM3.0-SDIP software

When using the proposed data in 3.1 as the anchor of 3.2, as Table-3 shows, 0.13%(AIHE) and 0.19%(AILC) can be achieved on top of the harmonization of SDIP and RQT syntax element coding described in section 2.1.

Table-3 Performance of two depth RQT of SDIP on top of RQT syntax harmonization

# Conclusion

A straightforward harmonization solution of SDIP and RQT is described in this contribution, which including syntax coding harmonization between RQT and SDIP, and two depth RQT support for 8x32/32x8 PU. Compared with HM3.0-SDIP default configuration without harmonization of RQT and SDIP, coding gains about 0.16% and 0.43% can be achieved for all test sequences under Intra-HE and Intra-LC test condition respectively without encoding or decoding time increase, it can also provide a harmonized spec description for SDIP with the existing spec. We suggest adopting the proposed harmonization solution of SDIP and RQT.

# References

- X. Cao, X. Peng, C. Lai (HiSilicon), et.al., “CE6.b1 Report on Short Distance Intra Prediction Method,” ITU-T JCTVC-E278, Geneva, CH, March 2011.

- F. Bossen, “Common test conditions and software reference configurations,” ITU-T JCTVC-E700, Geneva, CH, March 2011.

- M. Budagavi, K. Chono, R. Joshi, et.al., “Core Experiment 6: Intra Prediction Improvement,” ITU-T JCTVC-E706, Geneva, CH, March 2011.

# Patent rights declaration(s)

Huawei, Microsfot, HiSilicon, Tsinghua and USTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).