| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F535 |
| --- | --- |

| Title: | Crosscheck report for Samsung's harmonizing ROT and SDIP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Changcai Lai Lingzhi Liu Jianhua Zheng | Tel: Email: | +86-0755-28422002 laichangcai@huawei.com |
| Source: | HiSilicon |  |  |

_____________________________

# Abstract

This contribution document reports the cross verification results of Samsung's harmonizing ROT and SDIP (JCTVC-F294) evaluated on the HM3.0_SDIP branch. The source code was implemented based on HM3.0-SDIP branch software. The cross-check has been completed successfully and the results match those provided by proponent of JCTVC-F294.

# Test description and Results

The source code was implemented based on HM3.0-SDIP branch software, and the anchor is the HM3.0-SDIP branch software with default configurations. The encoder and decoder reconstructed YUV files match. The experimental results of the rate-distortion are shown in Error: Reference source not found for all intra case and Table 2 for random access case. These results match the BD-rate results provided by proponents of JCTVC- F294. The decoding time and encoding time change are very similar with those of proponent’s.

Table 1 Crosschecked results for All Intra

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.00 | 0.43 | 0.53 | -1.09 | 0.04 | -0.12 |
| Class B | -0.98 | 0.78 | 0.89 | -1.07 | 0.43 | 0.48 |
| Class C | -0.46 | 0.49 | 0.53 | -0.60 | 0.28 | 0.30 |
| Class D | -0.44 | 0.56 | 0.48 | -0.53 | 0.28 | 0.29 |
| Class E | -0.68 | 0.74 | 0.79 | -0.88 | 0.56 | 0.51 |
| All | -0.72 | 0.60 | 0.65 | -0.85 | 0.31 | 0.29 |
| Enc Time[%] | 119% |  |  | 136% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 2 Crosschecked results for random access

|  | Random access |  |  | Random access Loco |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.41 | 0.09 | 0.27 | -0.46 | -0.46 | -0.63 |
| Class B | -0.51 | 0.55 | 0.65 | -0.55 | 0.23 | 0.20 |
| Class C | -0.34 | 0.25 | 0.30 | -0.37 | 0.11 | 0.08 |
| Class D | -0.21 | 0.33 | 0.55 | -0.26 | -0.12 | 0.11 |
| Class E |  |  |  |  |  |  |
| All | -0.38 | 0.32 | 0.45 | -0.42 | -0.04 | -0.05 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 98% |  |  |