| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F536 |
| --- | --- |

| Title: | CE6.b Report on test7 Harmonization of SDIP and deblocking filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal, Report |  |  |
| Author(s) or Contact(s): | Xiaoran Cao, Tsinghua Xiulian Peng, USTC Changcai Lai, HiSilicon Jizheng Xu, Microsoft Jianhua Zheng, HiSilicon Yun He, Tsinghua Haoping Yu, Huawei Feng Wu, Microsoft | Email: | laichangcai@huawei.com jzxu@microsoft.com haoping.yu@huawei.com |
| Source: | Tsinghua University, University of Science and Technology of China, HiSilicon, Microsoft, Huawei |  |  |

_____________________________

# Abstract

This proposal reports the harmonization and results of SDIP and deblocking filter, a straightforward harmonization solution of SDIP and deblocking filter is described in this contribution, which is consistent with the WD.

# Harmonization of SDIP and deblocking filter

As described in WD, the deblocking filter process shall be applied to all prediction unit edges and transform unit edges of a picture, except edges at the boundary of the picture, any edges for which the deblocking filter process is disabled by disable_deblocking_filter_idc and any edges coinside with slice boundaries when loop_filter_across_slice_flag is equal to 0. For the transform units and prediction units with edges smaller than 8 samples in either vertical or horizontal direction, only the edges lying on the 8x8 sample grid are filtered.

When a CU is encoded with SDIP PU (2NxhN or hNx2N), only some PU edges are no smaller than 8 samples and the edges are lying on the 8x8 sample grid and thus need to be filtered. The only change was made on the step of derivation process of prediction unit boundary as follows:

If PartMode is equal to PART_2NxhN, horEdgeFlags[ k ][ 1 << ( log2CUSize – 2 ) ], horEdgeFlags[ k ][ (1 << ( log2CUSize – 2 )) * 2 ], horEdgeFlags[ k ][ (1 << ( log2CUSize – 2 )) * 3 ]  are set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

–	If PartMode is equal to PART_hNx2N, verEdgeFlags [ 1 << ( log2CUSize – 2 ) ][ k ], verEdgeFlags[ (1 << ( log2CUSize – 2 )) * 2 ] [ k ], verEdgeFlags[ (1 << ( log2CUSize – 2 )) * 3 ] [ k ]  are set equal to FilterInternalEdgesFlag for k = 0.. ( 1 << log2CUSize ) – 1.

The other steps of deblocking filtering are the same as HM3.0.

# Simulation results

The proposed harmonization is implemented in the released SDIP software with revision number 866, which can be downloaded from the SDIP branch in the SVN server. The SDIP anchor data is obtained using software without modification.

Intra-HE and Intra LC configurations specified in the common test condition is used for the simulation according to the CE6 description. Three cases were tested in this subtest:

Case1: SDIP=0, deblocking filter=0, SDIP_deblocking filter=0

The RD data of this test case is compared with HM3.0 anchor data to evaluate the performance of existing deblocking filter in HM3.0, anchor is HM3.0. Table-1 is the result of case1.

Table-1 Result of case1

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.1 | 0.2 | 0.3 | 0.9 | 2.2 | 2.4 |
| Class B | -1.3 | 0.3 | 0.9 | 0.9 | 3.5 | 4.5 |
| Class C | -0.8 | 0.1 | 0.3 | 0.0 | 2.9 | 3.6 |
| Class D | -0.6 | 0.3 | 0.4 | -0.2 | 3.1 | 3.8 |
| Class E | -0.7 | 1.3 | 1.5 | 1.4 | 7.1 | 7.5 |
| All | -1.1 | 0.4 | 0.6 | 0.6 | 3.6 | 4.2 |
| Enc Time[%] | 102% |  |  | 100% |  |  |
| Dec Time[%] | 97% |  |  | 86% |  |  |

Case2: SDIP=1, deblocking filter=0, SDIP_deblocking filter=0

The RD data of this test case is compared with SDIP anchor data to evaluate the performance of existing deblocking filter in the environment of SDIP software. Anchor is SDIP branch software and Table-2 is the result of this case.

Table-2 Result of case2

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | 0.3 | 0.4 | 0.9 | 2.3 | 2.5 |
| Class B | -1.2 | 0.4 | 1.0 | 1.0 | 3.5 | 4.5 |
| Class C | -0.7 | 0.2 | 0.4 | 0.1 | 3.0 | 3.7 |
| Class D | -0.6 | 0.4 | 0.5 | 0.0 | 3.2 | 3.9 |
| Class E | -0.6 | 1.4 | 1.6 | 1.5 | 7.1 | 7.5 |
| All | -1.1 | 0.5 | 0.8 | 0.7 | 3.6 | 4.3 |
| Enc Time[%] | 101% |  |  | 102% |  |  |
| Dec Time[%] | 95% |  |  | 90% |  |  |

Case3: SDIP=1, deblocking filter=1, SDIP_deblocking filter=1

The RD data of this test case is compared with SDIP anchor data to evaluate the harmonization of deblocking filter in SDIP PUs. Anchor is SDIP branch software and Table-2 is the result of this case.

Table-3 Result of case3

|  | Intra |  |  | Intra LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class B | 0.2 | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Class E | 0.3 | 0.0 | 0.0 | 0.2 | -0.2 | -0.1 |
| All | 0.1 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |

# Conclusion

A straightforward harmonization solution of SDIP and deblocking is described in this contribution, which only need a small change on the step of derivation process of prediction unit boundary.

# References

- M. Budagavi, K. Chono, R. Joshi, et.al., “Core Experiment 6: Intra Prediction Improvement,” ITU-T JCTVC-E706, Geneva, CH, March 2011.

# Patent rights declaration(s)

Huawei, Microsfot, HiSilicon, Tsinghua and USTC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).