| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F537r1 |
| --- | --- |

| Title: | On the motion compensation process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen 3240 Hillview Ave Palo Alto, CA 94304 USA | Tel: Email: | +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | DOCOMO USA Labs |  |  |

_____________________________

# Abstract

This contribution proposes two modifications to the motion compensation process to simplify the process by removing rounding operations and to ensure that all data after each of the vertical and horizontal filtering passes holds in 16-bit memory. With these modifications simplified and corrected reference software and text are proposed in which the proposed code is several thousands lines of code shorter than the current one, and the text is also shortened. The proposed modifications have a negligible impact on coding efficiency (all reported average BD-rate differences for luma are 0.0%) and on run times (98-103%). Commonalities exist with the proposals described in JCTVC-F480 and JCTVC-F585, which were independently developed. A confirmation of the performance reported in JCTVC-F480 is provided, as well as a comparative analysis.

# Introduction

This contribution addresses four issues:

- Following the adoption of JCTVC-E242 in Geneva, a clipping operation was removed in the motion compensation process. A side-effect of the removal of this clipping operation is that the resulting data may not hold within 16 bits as currently implemented in the software. In extreme cases values up to +33150 may be computed and they exceed the signed 16-bit range (max +32767).

- Cascaded rounding is applied when generating predictions for bi-prediction and averaging the predictions. Additionally the rounding process described in version 8 of the WD3 (JCTVC-D603) does not match the reference software implementation.

- For the case where input samples are wider than 8 bits (bitDepth > 8), the interpolated values after a first filtering pass (horizontal or vertical) exceed the 16-bit range. Consequently the second pass requires multiplications with operands wider than 16 bits, which is undesirable, for software implementations in particular.

- The implementation of the motion compensation process in the reference software currently consists of a similar function duplicated many times over, resulting in several thousand lines of code. It is desirable to have a much more generic implementation that facilitates experimentation with alternate filters.

# Proposed modifications

To address the four issues mentioned in the introduction, the following changes are proposed:

- Fix the software by adding a negative offset to the values during intermediate steps to make sure they always hold in the signed 16-bit range (this has no normative impact).

- Remove the variables offset1 and offset2 in sections 8.4.2.2.1 and 8.4.2.2.2 of the WD (set them to 0).

- Fix the order in which horizontal and vertical filtering is done for inputs wider than 8 bits. It is proposed to perform horizontal filtering first followed by vertical filtering. After horizontal filtering, interpolated values are scaled such as to fit within the 16-bit signed range. Note that this modification does not affect the ability to freely choose the ordering of filtering when the bit depth is 8. It should be noted that the order of processing is also fixed for transforms.

- Modify the software such as to have only a single generic filtering function. Additionally data structures are modified in the motion estimation such as to have separate buffers for the interpolated blocks for the various fractional-pel positions (as opposed to have them interleaved as now, which requires additional handling functions). This leads to a reduction of the code size of more than 2000 lines of source code. Note that some of this simplification is the result of the above modifications as the input and output data of the filtering functions are now always 16 bits wide.

Essentially the motion compensation process can be described as follows with the above modifications:

- Shift left reference samples by 14-bitDepth bits

- If applicable, apply horizontal filter and shift right by 6 bits (no rounding offset)

- If applicable, apply vertical filter and shift right by 6 bits (no rounding offset)

- If bipred, add predictions, shift right by 1 bit (no rounding offset)

- Shift right by 14-bitDepth bits (with rounding offset) and clip

In the proposed text attached to this contribution, the first 2 steps have been merged to minimize changes to the text. The text related to the last 2 steps is not modified.

The table below summarizes the changes:

| Case | Bit depth = 8 | Bit depth > 8 |
| --- | --- | --- |
| 1D filtering, unipred | No change | No change |
| 2D filtering, unipred | No change | Reduce bit depth after first filter |
| 1D filtering, bipred | No change | Remove cascaded rounding |
| 2D filtering, bipred | Remove cascaded rounding (after second filter) | Reduce bit depth after first filter Remove cascaded rounding (after second filter) |

# Results

Experiments were conducted on version 3.2 of the HM software and results are reported below. BD-rate numbers were rather consistent for all sequences, including Nebuta and BQTerrace.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | 0.1 | 0.0 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Class E |  |  |  |  |  |  |
| Overall | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class D | 0.1 | 0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class E | -0.2 | 0.1 | -1.0 | -0.2 | -0.4 | -0.4 |
| Overall | 0.0 | 0.1 | -0.3 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 101% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.5 | 0.3 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.7 | 0.0 | 0.0 | 0.0 |
| Overall | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Note on run times: all results (including anchor) were obtained using an auto-vectorizing compiler (in this case gcc 4.4.4 was used, with options –O3 –march=core2). A quick look at the assembly code generated by the compiler indicates that, even though the compiler was able to vectorize parts of the code (i.e., generate MMX/SSE instructions), the generated code is not as good as it could be. Using a better compiler (e.g., a more recent version of gcc or icc) may further reduce run times. Using a compiler without auto-vectorization capabilities may lead to different run time results. Essentially the code was written in a generic way that makes it look slow, but a good compiler can make it run quite fast.

Further note: attempts to have gcc 4.1.2 generate vectorized code for the filtering functions were unsuccessful.

# Source code extract

The following shows the source code for the one and only interpolation function. It is a template function where N defines the length of the filter (e.g., 8 for luma, 4 for chroma), isVertical defines whether to do horizontal of vertical filtering, and isFirst and isLast indicate whether the bit width of input and output data is the native bit width (e.g., 8 or 10 bits) or the extended bit width (i.e., 16 bits including 14 bits of data and 2 bits for overflow). In essence these parameters act as regular function parameters, except that separate code will be generated (and optimized) by the compiler for the used combinations of these parameters.

template<int N, bool isVertical, bool isFirst, bool isLast>

Void TComInterpolationFilter::filter(Short const *src, Int srcStride, Short *dst, Int dstStride, Int width, Int height, Short const *coeff)

{

Int row, col;

Short c[8];

c[0] = coeff[0];

c[1] = coeff[1];

if ( N >= 4 )

{

c[2] = coeff[2];

c[3] = coeff[3];

}

if ( N >= 6 )

{

c[4] = coeff[4];

c[5] = coeff[5];

}

if ( N == 8 )

{

c[6] = coeff[6];

c[7] = coeff[7];

}

Int cStride = ( isVertical ) ? srcStride : 1;

src -= ( N/2 - 1 ) * cStride;

Int offset;

Short maxVal;

Int headRoom = IF_INTERNAL_PREC - (g_uiBitDepth + g_uiBitIncrement);

Int shift = IF_FILTER_PREC;

if ( isLast )

{

shift += (isFirst) ? 0 : headRoom;

offset = 1 << (shift - 1);

offset += (isFirst) ? 0 : IF_INTERNAL_OFFS << IF_FILTER_PREC;

maxVal = g_uiIBDI_MAX;

}

else

{

shift -= (isFirst) ? headRoom : 0;

offset = (isFirst) ? -IF_INTERNAL_OFFS << shift : 0;

maxVal = 0;

}

for (row = 0; row < height; row++)

{

for (col = 0; col < width; col++)

{

Int sum;

sum  = src[ col + 0 * cStride] * c[0];

sum += src[ col + 1 * cStride] * c[1];

if ( N >= 4 )

{

sum += src[ col + 2 * cStride] * c[2];

sum += src[ col + 3 * cStride] * c[3];

}

if ( N >= 6 )

{

sum += src[ col + 4 * cStride] * c[4];

sum += src[ col + 5 * cStride] * c[5];

}

if ( N == 8 )

{

sum += src[ col + 6 * cStride] * c[6];

sum += src[ col + 7 * cStride] * c[7];

}

Short val = ( sum + offset ) >> shift;

if ( isLast )

{

val = ( val < 0 ) ? 0 : val;

val = ( val > maxVal ) ? maxVal : val;

}

dst[col] = val;

}

src += srcStride;

dst += dstStride;

}

}

Additionally filters are simply defined in arrays such as:

const Short TComInterpolationFilter::m_lumaFilter[4][NTAPS_LUMA] =

{

{  0, 0,   0, 64,  0,   0, 0,  0 },

{ -1, 4, -10, 57, 19,  -7, 3, -1 },

{ -1, 4, -11, 40, 40, -11, 4, -1 },

{ -1, 3,  -7, 19, 57, -10, 4, -1 }

};

const Short TComInterpolationFilter::m_chromaFilter[8][NTAPS_CHROMA] =

{

{  0, 64,  0,  0 },

{ -3, 60,  8, -1 },

{ -4, 54, 16, -2 },

{ -5, 46, 27, -4 },

{ -4, 36, 36, -4 },

{ -4, 27, 46, -5 },

{ -2, 16, 54, -4 },

{ -1,  8, 60, -3 }

};

# Relationship with JCTVC-F480

Document JCTVC-F480 proposes changes that are similar to the ones described above in Section 2. In particular harmonization between uni- and bi-prediction is sought. Motion compensation functions are unified such that the same filtering functions are used for uni- and bi-prediction. However a separate set of filtering functions is still used for fractional-pel motion estimation.

Whereas in Section 2 it is suggested to remove cascaded rounding for the bi-prediction case, the proposal F480 aligns the uni-prediction case with the bi-prediction one, thereby introducing cascaded rounding in the uni-prediction case (i.e., an additional rounding offset is added).

A summary of changes introduced by JCTVC-F480 is provided below (similar to the one provided in Section 2).

| Case | Bit depth = 8 | Bit depth > 8 |
| --- | --- | --- |
| 1D filtering, unipred | No change | Introduce cascaded rounding |
| 2D filtering, unipred | Introduce cascaded rounding (after second filter) | Introduce cascaded rounding (after second filter) |
| 1D filtering, bipred | No change | No change |
| 2D filtering, bipred | No change | No change |

The coding efficiency of JCTVC-F480 was verified using software provided by Samsung, which was also used for the above analysis. The results provided below are consisted with those reported in JCTVC-F480.

|  |  | Random access |  |  | Random access LC |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 92% |  |  | 94% |  |  |
| Dec Time[%] | 91% |  |  | 91% |  |  |
|  | Low delay |  |  | Low delay LC |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.2 | 0.3 | 0.0 | 0.1 | 0.2 |
| Class C | 0.0 | 0.0 | 0.1 | 0.0 | 0.2 | -0.1 |
| Class D | 0.0 | 0.3 | 0.4 | 0.0 | -0.2 | 0.5 |
| Class E | 0.0 | 0.5 | 1.7 | 0.0 | 0.1 | 0.2 |
| All | 0.0 | 0.2 | 0.5 | 0.0 | 0.0 | 0.2 |
| Enc Time[%] | 92% |  |  | 95% |  |  |
| Dec Time[%] | 92% |  |  | 91% |  |  |
|  | Low delay (P) |  |  | Low delay LC (P) |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.3 | 0.1 | 0.0 | 0.2 | 0.4 |
| Class C | 0.0 | 0.2 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class D | 0.0 | 0.4 | -0.5 | 0.0 | 0.0 | 0.1 |
| Class E | 0.1 | 1.2 | 0.6 | 0.0 | 0.1 | -0.2 |
| All | 0.0 | 0.5 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 93% |  |  | 96% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

Note on run times: the HM 3.0 reference was compiled using gcc 4.1.2 and the tested proposal using gcc 4.4.4. Differences in run times should thus be attributed to differences in compiler versions.

# Conclusions

A simplified implementation of the motion compensation process is proposed, which leads to a significant reduction of the number of lines of source code. In addition normative modifications related to rounding and filtering direction order are proposed in other to reduce the number of functions required to implemented the motion compensation process and enable the proposed simplified implementation.

It is recommended to integrate this simplified implementation into the HM software as it significantly improves code quality, readability and flexibility.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).