| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F542 |
| --- | --- |

| Title: | ALF with low latency and reduced complexity for HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Fuldseth, Cisco Systems G. Bjøntegaard, Cisco Systems | Tel: Email: Tel: Email | +47 92414082 arild.fuldseth@cisco.com gbjonteg@cisco.com |
| Source: | Cisco Systems |  |  |

_____________________________

# Abstract

The document describes an ALF technique suitable for low latency applications. One single set of ALF filter coefficients are computed, quantized and transmitted sequentially for each block (typically 64x64 or 128x128) using a single pass technique. BD-rate gains between 0.7 % and 2.0% are reported.

# Introduction

ALF as specified in HM 3.0 brings a coding gain of typically (4-6) % measured in BDR.  The operations are mainly performed on a frame basis.  This may lead to additional latency caused by the coding process.  This makes ALF less attractive for low (sub-frame) delay real time applications like video conferencing.  Low delay ALF is also addressed in [1]. Furthermore ALF represents a considerable implementation complexity.  The present document therefore describes some ideas and initial simulation tests on an ALF version that could be useful for low latency applications.

# Description

ALF is performed for luma only. In the following the main aspects of the tested method are briefly described.

Units for performing ALF

HM 3.0 perform ALF on a frame basis.  According to the present contribution ALF is performed on smaller units. In particular, 64x64 and 128x128 luma pixels have been used.  This will be referred to as an ALF-unit.

Pixel matrix

The 5x5 diamond shaped pixel matrix has been used.

Pixel classifications

No pixel classification based on variance classes has been used. With the smaller ALF-units classification is considered to be less important.

Calculating ALF coefficients

Calculation of covariance data and solving for optimal coefficients are performed in a similar way to HM 3.0.  One difference is that no dc adjustment is used.

Quantization of ALF coefficients

The ALF coefficients are quantized more coarsely than in HM 3.0.  This is mainly in order to save coding bits since coefficients may have to be sent for every ALF-unit.  For the 5x5 diamond shape structure 7 ALF coefficients are calculated.  With the coarse quantization, especially the DC response of the filter may suffer.  To overcome this only 6 out of 7 coefficients are quantized and transmitted.  The last coefficient – representing the center position of the filter is calculated to give a final DC response of 1.

Coding of ALF coefficients

The coefficients are encoded using one bit for the sign and one UVLC for the magnitude. No temporal prediction is used.

Overall procedure of the ALF process

Calculation and quantization of a set of filter coefficient is performed for an ALF-unit.  Then an RD test is made to decide if the filter shall be used.  1 bit is used to signal whether the ALF is enabled or disabled.  If the outcome of the RD test is positive, filtering is performed with dequantized filter coefficients.  No further switch between filters is performed.

A nice side-effect of the simplicity is that the number of code lines in the source code is reduced from ~ 10 000 to less than 500.

# Simulation results

Table 1 shows BD-rate results for enabling ALF with the low complexity configuration. ALF units of 64x64 and 128x128 are used. For reference, the results of the current HM3.0 ALF filter are included.

Table 1. BD-rate results for HM3.0-ALF and proposed, low delay ALF for 64x64 blocks and for 128x128 blocks.

|  | HM3.0-ALF |  |  | Proposed - 64x64 |  |  | Proposed - 128x128 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra LC |  |  | All Intra LC |  |  | All Intra LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -3,8 | -1,0 | -1,4 | -1,3 | 1,2 | 1,2 | -2,0 | 0,5 | 0,5 |
| Class B | -2,2 | -2,3 | -1,7 | -0,6 | 0,9 | 0,9 | -1,1 | 0,5 | 0,5 |
| Class C | -2,0 | -2,6 | -3,7 | -0,7 | 0,6 | 0,6 | -0,7 | 0,2 | 0,2 |
| Class D | -0,9 | -2,2 | -2,7 | -0,3 | 0,3 | 0,3 | -0,2 | 0,1 | 0,1 |
| Class E | -3,3 | -3,8 | -4,6 | -0,9 | 1,5 | 1,5 | -1,5 | 0,8 | 0,8 |
| Overall | -2,4 | -2,3 | -2,7 | -0,7 | 0,9 | 0,9 | -1,1 | 0,4 | 0,4 |
| Enc Time[%] | 174 % |  |  | 100 % |  |  | 102 % |  |  |
| Dec Time[%] | 142 % |  |  | 103 % |  |  | 105 % |  |  |
|  | Random Access LC |  |  | Random Access LC |  |  | Random Access LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -6,2 | -3,0 | -3,9 | -1,5 | 2,8 | 2,5 | -3,2 | 1,6 | 1,2 |
| Class B | -4,3 | -1,4 | 0,0 | -0,9 | 1,9 | 1,8 | -2,0 | 0,7 | 0,7 |
| Class C | -3,0 | 0,8 | -0,6 | -1,0 | 1,2 | 1,2 | -1,4 | 0,4 | 0,4 |
| Class D | -2,7 | 0,6 | 0,7 | -1,0 | 0,9 | 0,9 | -1,3 | 0,2 | 0,1 |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -4,1 | -0,8 | -0,9 | -1,1 | 1,7 | 1,6 | -2,0 | 0,7 | 0,6 |
| Enc Time[%] | 115 % |  |  | 101 % |  |  | 101 % |  |  |
| Dec Time[%] | 128 % |  |  | 107 % |  |  | 109 % |  |  |
|  | Low delay B LC |  |  | Low delay B LC |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |
| Class B | -4,8 | -1,1 | -0,4 | -1,1 | 2,0 | 1,7 | -2,3 | 1,2 | 0,9 |
| Class C | -3,8 | 0,7 | -1,0 | -1,8 | 1,2 | 1,1 | -1,9 | 0,5 | 0,3 |
| Class D | -2,5 | 0,4 | 0,6 | -0,9 | 0,9 | 0,7 | -1,0 | 0,2 | 0,1 |
| Class E | -6,0 | -6,6 | -1,0 | 0,0 | 5,0 | 5,3 | -3,1 | 1,8 | 1,3 |
| Overall | -4,2 | -1,3 | -0,4 | -1,0 | 2,1 | 2,0 | -2,0 | 0,9 | 0,6 |
| Enc Time[%] | 113 % |  |  | 99 % |  |  | 100 % |  |  |
| Dec Time[%] | 126 % |  |  | 104 % |  |  | 106 % |  |  |

# Conclusions

Preliminary results have been presented for an ALF version that could be useful for applications requiring low latency. BD-rate gains are between 0.7% and 2.0% depending on test configuration.

It is proposed that this work can be continued in an ALF core experiment.

# Patent rights declaration(s)

Cisco Systems may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

This number also includes SAO source code.