| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F544 |
| --- | --- |

| Title: | Improved arithmetic coding based on the probability aggregation-more results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongbo Zhu | Tel: Email: | 8613701663601 hurricanezhb@hotmail.com |
| Source: | Hongbo Zhu |  |  |

_____________________________

# Abstract

The PA-Coder is improved in this proposal in two aspects. The first one is the new probability states and new probability state transition table. The second one is to code the non-stop significance map/MV as low-sub-interval always regardless if it is MPS. The second one could accelerate the decoding speed.

# Introduction to the arithmetic coding based on the probability aggregation

The arithmetic coding based on the probability aggregation (PA-Coder) is introduced in [1] and improved in [2, 3, 4]. In this proposal, the PA-Coder is improved in two aspects. The first one is to eliminate the CABAC probability state and denote the symbol state with logprobMPS-1 directly. AslogprobMPS∈4,128, 7-bits are enough for the new state, logprobMPS-1. New probability transition tables are designed for the new probability state when LPS/MPS symbol is coded, the state jumps between new states. Same with the current design, the new state is stored in one byte with the MPS symbol. The MPS symbol is stored in the 0th bit, while the high 7bits is occupied by the state. When a LPS symbol is coded/decoded, the state is updated as:

nextstate=translpstable[currstate>>1]currstate∧1

When a MPS symbol is coded/decoded, the state is updated as follows:

nextstate=transmpstable[currstate>>1]currstate∧1

The trans_lps_table and the trans_mps_table are 128-entry tables. They are trained by the following 1080p sequences in table 1 at QP=20. As the training is not enough, the new transition table shows big loss on coding efficiency.

| crowd_run_1080p | ducks_take_off_1080p |
| --- | --- |
| in_to_tree_1080p | old_town_cross_1080p |
| park_joy_1080p | pedestrian_area |
| riverbed | rush_hour |
| station2 | tractor |

Table 1: 1080p sequences to train the state transition table

In the current CABAC design, all states are initialized when starting. As the new state is different from the CABAC state, a table is designed to map the CABAC state to the new state according to the corresponding probability.

The current interval is divided into two disjoint sub-interval in a binary arithmetic coder, each sub-interval is associated to MPS or LPS. In CABAC, the low-interval is always associated to the MPS. In the PA-Coder, the low-interval is associated to the symbol 0 regardless if it is MPS for the syntax elements significance map and MV. According to the fast algorithm of the PA-Coder[3], the low-interval decoding is very simple (only need a comparison and an addition). In a practical implementation, the logprob of the low-interval is always stored in memory as the state of the current probability, the decoding of the significance map/MV is to compare and add iterately to find a high-interval. The figure 1 illustrates the coding of the bin of the MV.

# Experiments results

The PA-Coder is integrated into the HM3.1 and run under the common test condition JCTVC-E700 [5]. The results are showed in figure 2. It is clear that the big loss on coding efficiency is due to the bad-designed probability estimation. The design of the state transition table must include more sequences on more QP. On the other hand, the PA-Coder is faster than the CABAC especially for the encoder. The encoder/decoder platform is listed in table 2.

Table 2: encoder/decoder platform configuration

|  | EXE | platform | CPU | cores | Harddisk | DRAM | BUS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| encoder | X86 | Win7x64 | amd phenom (tm) II x6 1075T @3.0GHz | 6 | WD 1TB | 6G | 1333MHz |
| decoder | X86 | Win7x64 | intel core 2 quad Q8300@2.5G | 4 | WD 500GB | 2G | 1333MHz |

Figure 2: BD_Bitrate results between PA-Coder and anchor

# References

- Hongbo Zhu, “Arithmetic coding based on probability-aggregation and delayed-subdivision”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 1st Meeting: Dresden, DE, 15-23 April, 2010, Document JCTVC-A027.

- Hongbo Zhu, “Some improvement of the arithmetic coding based on probability aggregation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 2st Meeting: Geneva, CH, 21-28 July, 2010, Document JCTVC-B091.

- Hongbo Zhu, “Decoding Improvement on the PA-Coder”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 3rd Meeting: Guangzhou, CN, 7-15 October, 2010.

- Hongbo Zhu, “More improvements and results of the arithmetic coding based on probability aggregation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC TC1/SC29/WG11 4rd Meeting: Daegu, KR, 20-28 January, 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-E700, Geneva, April 2011

# Patent rights declaration(s)

Hongbo Zhu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).