| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F546 |
| --- | --- |

| Title: | Sliding Window Improvement for Temporal Scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Chen, Peisong Chen, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | +1 858 845 6589 cheny@qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This is a follow-up proposal for JVCVC-E399. It was reported that the sliding window mechanism is not efficient when temporal scalability is used and it is proposed that constraints be include to disallow sliding window to affect the reference pictures with a lower temporal id than the current picture.

# Proposal

The following modification is needed in the specification text (only the red text is newly added).

#### 8.2.5.3 Sliding window decoded reference picture marking process

This process is invoked when adaptive_ref_pic_marking_mode_flag is equal to 0.

Depending on the current field, the following applies.

- If the current picture is a coded field that is the second field in decoding order of a complementary reference field pair, and the first field has been marked as “used for short-term reference”, the current picture is also marked as “used for short-term reference”.

- Otherwise, the following applies.

-	Let numShortTerm be the total number of reference frames, complementary reference field pairs and non-paired reference fields for which at least one field is marked as “used for short-term reference”. Let numLongTerm be the total number of reference frames, complementary reference field pairs and non-paired reference fields for which at least one field is marked as “used for long-term reference”.

-	Let temporal_id of the current picture be tId.

-	When numShortTerm + numLongTerm is equal to num_ref_frames, the condition that numShortTerm is greater than 0 shall be fulfilled, and the short-term reference frame, complementary reference field pair or non-paired reference field with a temporal_id equal to or higher than tID that has the smallest value of FrameNumWrap is marked as “unused for reference”. When it is a frame or a complementary field pair, both of its fields are also marked as “unused for reference”.

# Discussion

Currently, reference picture management is not implemented in the HM software, including sliding window and memory management control operation, both of which are related to frame_num which is not present in the software.

Therefore, the software implementation for this proposal is not ready yet. We volunteer to implement sliding window and memory management control operation (for short-term reference pictures). We will be able to provide the software implementation for this proposal after reference picture management is implemented.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).