| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F547 |
| --- | --- |

| Title: | Cross-check of Tiles (JCTVC-F335), experiment 1 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Rickard Sjöberg, Per Wennersten Ericsson Research 164 80 Stockholm Sweden | Tel: Email: | +46 761153563 rickard.sjoberg@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution presents results of a cross-check of "Tiles" proposed by Cisco, eBrisk, Sharp and TI in document JCTVC-F335. The "Experiment 1: MTU size matching" as described in F335 has been verified successfully for the common conditions for BDR, PSNR, bit rate and decoding time. The source code was well written and compiled without any problems. Decoding time was 100.7% of the anchor decoding time. It should be noted that separate Tile configuration parameters was supplied for each combination of resolution, QP, and test case for this experiment.

# Introduction

"Tiles" is a proposal that partitions a picture into rectangular segments. Each tile is processed in raster-scan order, and LCUs within each tile are also processed in raster-scan order. Tiles were first introduced in document JCTVC-E408. This document provides a cross-check of experiment 1 (MTU size matching) as described in JCTVC-F335.

# Experiment 1: MTU size matching

The source code provided by the proponents was successfully compiled with gcc 4.3.2 on Linux and Visual Studio for Windows without any problems. The software was used for encoding and decoding according to common conditions [1] on a Linux cluster to get compression efficiency numbers.

## BDR performance

We found a perfect match for all bitstreams between data provided by proponents and results generated by us, shown below. It can be noted that most gains is achieved for Class E sequences coded with all-Intra. Full results are available in the excel attachment.

Table I BDR performance for experiment 1

|  | Proponent JCTVC-F335 |  |  |  |  |  | Cross-checker |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AI HE |  |  | AI LC |  |  | AI HE |  |  | AI LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.1 | -0.4 | 0.2 | 0.0 | -0.4 | 0.0 | -0.1 | -0.4 | 0.2 | 0.0 | -0.4 | 0.0 |
| Class B | -0.7 | -1.0 | -0.7 | -0.7 | -1.3 | -0.9 | -0.7 | -1.0 | -0.7 | -0.7 | -1.3 | -0.9 |
| Class C | -0.5 | -0.6 | -0.7 | -0.6 | -0.9 | -0.8 | -0.5 | -0.6 | -0.7 | -0.6 | -0.9 | -0.8 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.2 |
| Class E | -2.3 | -3.0 | -3.2 | -2.5 | -4.8 | -3.9 | -2.3 | -3.0 | -3.2 | -2.5 | -4.8 | -3.9 |
| Overall | -0.7 | -1.0 | -0.8 | -0.7 | -1.3 | -1.0 | -0.7 | -1.0 | -0.8 | -0.7 | -1.3 | -1.0 |
| Enc Time[%] | 72% |  |  | 68% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 58% |  |  | 57% |  |  | 105% |  |  | 96% |  |  |

|  | Proponent JCTVC-F335 |  |  |  |  |  | Cross-checker |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RA HE |  |  | RA LC |  |  | RA HE |  |  | RA LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.2 | -1.0 | 0.2 | -0.4 | -0.7 | -0.5 | -0.2 | -1.0 | 0.2 | -0.4 | -0.7 | -0.5 |
| Class B | -0.6 | -1.0 | -0.6 | -0.7 | -1.2 | -0.9 | -0.6 | -1.0 | -0.6 | -0.7 | -1.2 | -0.9 |
| Class C | -0.3 | -0.2 | -0.3 | -0.3 | -0.4 | -0.4 | -0.3 | -0.2 | -0.3 | -0.3 | -0.4 | -0.4 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.3 | -0.6 | -0.2 | -0.4 | -0.6 | -0.5 | -0.3 | -0.6 | -0.2 | -0.4 | -0.6 | -0.5 |
| Enc Time[%] | 84% |  |  | 80% |  |  | 101% |  |  | 101% |  |  |
| Dec Time[%] | 69% |  |  | 63% |  |  | 107% |  |  | 105% |  |  |

|  | Proponent JCTVC-F335 |  |  |  |  |  | Cross-checker |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LD (B) HE |  |  | LD (B) LC |  |  | LD B HE |  |  | LD B LC |  |  |
|  | Y | U | V | Y | U | V | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.2 | -0.5 | -0.9 | -0.9 | -0.4 | -0.4 | -0.2 | -0.5 | -0.9 | -0.9 |
| Class C | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 | -0.3 | -0.2 | -0.2 | -0.3 | -0.3 | -0.3 | -0.3 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 | 0.0 |
| Overall | -0.1 | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 | -0.1 | -0.2 | -0.2 | -0.2 | -0.4 | -0.4 |
| Enc Time[%] | 96% |  |  | 94% |  |  | 101% |  |  | 100% |  |  |
| Dec Time[%] | 94% |  |  | 95% |  |  | 108% |  |  | 106% |  |  |

## Decoding times

Decoding times were measured on Windows on a single machine as an average of several runs. No YUV files were written. The proposal shows the following decoding time compared to the HM-3.0 anchor.

Table II Decoding time for experiment 1

|  | Proposal (s) | Anchor (s) | Dec. speed (%) |
| --- | --- | --- | --- |
| RA | 7269.525 | 7286.298 | 99.8 |
| RA LC | 4113.619 | 4113.285 | 100.0 |
| LD | 3295.852 | 3265.479 | 100.9 |
| LD LC | 2144.985 | 2102.68 | 102.0 |
| AI | 2340.618 | 2330.691 | 100.4 |
| AI LC | 1518.215 | 1498.715 | 101.3 |
| Average |  |  | 100,7 |

## Slice statistics

Some slice statistics was gathered for both anchor and test. Out of the 424 bitstreams, the Tile code had fewer slices than the Anchor in 223 cases while there were 54 cases where the anchor had fewer slices.

# Comments on source code

In general, we found the source code to be clearly written. Some small issues could be fixed, such as adding range-checks for configuration parameters and reducing the amount of tile objects allocated.

# General comments

Apart from setting TileBoundaryIndependenceIdr to 1, the only difference between Proposal and Anchor is the scan order of the LCUs. It should be noted that in order to achieve this, separate configuration parameters have been supplied for each combination of resolution, QP, and test case. The provided settings were designed to achieve square Tiles so the impact of TileBoundaryIndependenceIdr is expected to be small.

# References

- F. Bossen, “Common test conditions and software reference configurations” JCTVC-E600, Switzerland, March 2011