| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F552 |
| --- | --- |

| Title: | Parallel Processing of Residual Data in HE |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Joel Sole Rajan Joshi Marta Karczewicz 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-845-2429 Email: joels@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This document presents two techniques to facilitate the parallelization of the residual coding in HE. The first one extends the concept of parallel context processing of coefficient level to include the significance map coding pass. This provides localized data access for all the coding passes of the residual. The BD-rate of this technique for AI-HE, RA-HE and LB-HE configuration is 0.25℅, 0.21℅, and 0.05℅, respectively. A delayed state update in CABAC is also proposed to help parallelism. The BD-rate of both techniques combined for AI-HE, RA-HE and LB-HE configuration is 0.15℅, 0.13℅, and 0.00℅, respectively.

- Introduction

In JCTVC-E335/F288 [1] it is proposed that the same reverse scan is used for significance map coding as well as coefficient level coding. The scan starts with the last significant coefficient and proceeds to the DC coefficient in the reverse scanning order. The level coding is performed in 3 passes on subsets of 16 coefficients in the reverse scan order. In this contribution, we extend this idea to the significance map coding by interleaving significance map and coefficient level coding for each subset. We also propose the use of delayed state update in CABAC to enhance parallelism. Delaying the state update to the end of the block was proposed in [2]. We propose a shorter delay in the state update in order to improve the parallelism without incurring relevant performance penalty.

# Technical description

## Interleaving of significance map coding and coefficient level coding

In [1], the reverse scan is used for both significance map coding and coefficient level coding. For coefficient level coding, the coefficients are divided into subsets of 16 coefficients. The level coding is performed in the following 3 passes:

- Bin 1 of the absolute value of the coefficient level

- Remaining coefficient level

- Sign

In this contribution, we extend the concept of subsets to significance map coding. Thus for each subset, significance map coding and 3 passes of coefficient level coding are performed before proceeding to the next subset. Thus for each subset, the following 4 passes are performed:

- Significance

- Bin 1 of the absolute value of the coefficient level

- Remaining coefficient level

- Sign

The interleaving provides some benefits for parallelization. For example, while coefficient level coding is proceeding, the significance map contexts for some of the coefficients from the next subset can be pre-calculated. Similarly, when significance coding pass is proceeding, the context set to be used for coefficient level coding of the current subset can be pre-calculated.

It should be stressed that this part of the proposal represents a reordering of the coding operations proposed in JCTVC-E335/F288 Part 2. Since the scan order in both cases is the same and significance map contexts are different from coefficient level coding contexts, the BD-rate results are nearly identical. Note that the performance is not very sensitive to the precise sub-set size chosen. The sub-set size can be varied from 1 to the TU size with nearly identical performance.

## Delayed state update in CABAC

In HEVC, the speed of CABAC is affected by two factors. One is that the context for the next bin may depend on the current bin. The other one is that the state update for the current bin depends on the value of the current bin. We propose delaying the state update by k bins, where k > 0. This allows the arithmetic coder to process the next bin(s) without waiting for the state update to be completed. This is potentially useful when the context for the next bin(s) to be processed is already known. This can happen in significance map coding when multiple pixels on the same diagonal are being processed. Consider the case when k = 1. In this case, the only requirement is that the state update be completed by the time encoding (or decoding) of the next bin is complete. The delayed update may have a negative effect on performance for k = 1 only when two consecutive contexts are the same.

The delayed update method for k = 1 is implemented along with the 4 scan passes algorithm. The delayed update is only applied to the contexts used in these passes, since they constitute the largest amount of bins in HM.

- Results

The first part of this proposal harmonizes all the residual data coding passes. Dividing the data in groups of 16 coefficients has a BD-rate for AI-HE, RA-HE, and LB-HE configurations of 0.25℅, 0.21℅, and 0.05℅, respectively. Table 1 shows these results. The implementation is in HM3.0 and common test conditions are used.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.47 | -0.52 | -0.57 | -0.68 | -1.04 | -1.21 |  |  |  |
| Class B | -0.20 | -0.59 | -0.60 | -0.13 | -0.80 | -0.66 | 0.03 | -1.48 | -1.55 |
| Class C | -0.21 | -0.42 | -0.43 | 0.00 | -0.66 | -0.58 | 0.03 | -1.44 | -1.58 |
| Class D | -0.18 | -0.50 | -0.48 | -0.04 | -0.69 | -0.91 | -0.17 | -2.27 | -2.00 |
| Class E | -0.19 | -0.91 | -0.78 |  |  |  | -0.11 | -2.43 | -3.17 |
| All | -0.25 | -0.57 | -0.56 | -0.21 | -0.79 | -0.83 | -0.05 | -1.84 | -1.97 |
| Enc T[%] | 98% |  |  | 99% |  |  | 97% |  |  |
| Dec T[%] | 97% |  |  | 98% |  |  | 98% |  |  |

Table 1. BD-rate for residual data partitioning

When the delayed probability update with k = 1 is used in conjunction with the above method, the BD-rate is 0.15℅, 0.13℅, and 0.00℅ for the AI-HE, RA-HE, and LB-HE configurations, respectively. Table 2 shows these results.

| BD-rate | All Intra HE |  |  | Random access HE |  |  | Low delay B HE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Y | U | V |
| Class A | -0.32 | -0.34 | -0.39 | -0.60 | -1.19 | -1.41 |  |  |  |
| Class B | -0.09 | -0.49 | -0.45 | -0.05 | -0.72 | -0.49 | 0.08 | -1.24 | -1.37 |
| Class C | -0.13 | -0.34 | -0.34 | 0.09 | -0.60 | -0.62 | 0.08 | -1.46 | -1.32 |
| Class D | -0.11 | -0.43 | -0.37 | 0.01 | -0.76 | -0.81 | -0.05 | -1.75 | -1.54 |
| Class E | -0.09 | -0.80 | -0.64 |  |  |  | -0.17 | -2.14 | -2.31 |
| All | -0.15 | -0.46 | -0.43 | -0.13 | -0.81 | -0.81 | 0.00 | -1.59 | -1.58 |
| Enc T[%] | 97% |  |  | 96% |  |  | 98% |  |  |
| Dec T[%] | 98% |  |  | 98% |  |  | 98% |  |  |

Table 2. BD-rate for residual data partitioning + delayed state update

- References

- J. Sole, R. Joshi, M. Karczewicz, “Unified scans for the significance map and coefficient level coding in high coding efficiency”, JCTVC-E335, 5th JCT-VC Meeting, Geneva, CH, March 2011.

- H. Sasai, T. Nishi, “Parallel context processing for significance map using block-based context updates”, JCTVC-E226, 5th JCT-VC Meeting, Geneva, CH, March 2011.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex: Text for Working Draft

Table 3 shows the proposed sub-set scanning for the residual coding syntax elements. The rest remains as described in JCTVC-F288.

|  | Descriptor |
| --- | --- |
| numLastSubset = last_significant_coeff_scan_position >> 4 |  |
| for( i = numLastSubset; i >= 0; i−− ) { |  |
| offset = i << 4 |  |
| for( n = 15; n >= 0 && n + offset != last_significant_coeff_scan_position ; n−− ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| significant_coeff_flag[ xC ][ yC ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n−− ) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) |  |
| coeff_abs_level_greater1_flag[ n ] | ae(v) |
| } |  |
| for( n = 15; n >= 0; n−− ) { |  |
| if( coeff_abs_level_greater1_flag[ n ] ) { |  |
| coeff_abs_level_greater2_flag[ n ] | ae(v) |
| if( coeff_abs_level_greater2_flag[ n ] ) |  |
| coeff_abs_level_minus3[ n ] | ae(v) |
| } |  |
| } |  |
| for( n = 15; n >= 0; n−−) { |  |
| xC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 0 ] |  |
| yC = ScanOrder[ log2TrafoSize − 2 ][ scanIdx ][ n + offset ][ 1 ] |  |
| if( significant_coeff_flag[ xC ][ yC ] ) { |  |
| coeff_sign_flag[ n ] | ae(v) |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = ( coeff_abs_level_minus3[ n ] + 3 ) * ( 1 − 2 * coeff_sign_flag[ n ] ) |  |
| } else |  |
| transCoeffLevel[ x0 ][ y0 ][ trafoDepth ][ cIdx ][ n + offset ] = 0 |  |
| } |  |
| } |  |

Table 3. Sub-set scanning of residual coding syntax elements

###### Delayed state transition process (for 9.3.3.2.1.1 in WD3)

The DecodeDecision process is split into two processes called DecodeDecision and StateTransition (Flowchart 1 and 2).

Inputs to DecodeDecision  process are the decoded value binVal and valMPS values of the context variable associated with ctxIdxTable and ctxIdx.

Output of this process is MPScurr of the context variable associated with ctxIdx.

Inputs to the StateTransition process are the pStateIdx of the previous decoded bin and MPScurr and MPSprev indicating whether the current and previous bin was MPS and valMPS value of the context variable associated with ctxIdxTable and ctxIdx.

Outputs of this process are the updated pStateIdx and  valMPS of the context variable associated with previous ctxIdx and MPSprev indicating the whether bin was MPS or not.

Depending on the decoded value MPSprev, the update of the two variables pStateIdx and valMPS associated with previous ctxIdx is derived as specified by the following pseudo-code:

if(MPSprev) 
	pStateIdx = transIdxMPS( pStateIdx )
else {				(6-1)
	if( pStateIdx  = =  0 )
		valMPS = 1  valMPS
	pStateIdx = transIdxLPS( pStateIdx )
}

Flowchart 1. Decoding a decision of delayed state update

Flowchart 2. Transition  a decision of delayed state update