Change of Woking Darft for proposal

JCTVC-F554: On secondary transforms for intra prediction residual

The document is based on JCTVC-E603_d8.doc and change is colored yellow.

## Scaling, transformation and array construction process prior to deblocking filter process

### Scaling and transformation process

### Inverse scanning process for transform coefficientsI

### Scaling process for transform coefficients

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nS specifying the size of the current transform unit,

–	a (nS)x(nS) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a (nS)x(nS) array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nS is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

[Ed.: (WJ) wait decision on transform precision extension]

[Ed: (WJ) derivation of trafoPrecisionExt is duplicated to that of the scaling process. Better to remove one of two.]

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, nS is equal to 4 or 8 or 16 or 32, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-1 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-1 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 8.1.4.1 to  according to the size of the transform unit nS, with the (nS)x(nS) array d and the transform type variable horizTrType as the inputs and the output is the (nS)x(nS) array e.

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 8.1.4.1 to  according to the size of the transform unit nS, with the (nS)x(nS) array e and the transform type variable vertTrType as the inputs and the output is the (nS)x(nS) array f.

- If nS is equal to 4 or 8, the residual sample value rij is derived by

rij = ( fij + 25+trafoPrecisionExt ) >> ( 6 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-1)

- Otherwise (nS is equal to 16 or 32), the residual sample value rij is derived by

rij = ( fij + 29+trafoPrecisionExt ) >> ( 10 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-2)

[Ed.: (WJ) dependent on transformation process – should be modified again after transformation process is fixed]

#### Transformation process for 4 samples

Inputs of this process are an array of 4 samples of the scaled transform coefficients x with elements xi, with i = 0..3 and the transform type variable trType.

Output of this process is an array of 4 samples of the residual samples y with elements yi, with i = 0..3.

Depending on trType, the following applies:

–	If tyType is equal to 1, the following ordered steps apply:

- A set of intermediate values c0, c1, c2 and c3 is calculated as follows:

c0 = x0 + x2		(8-3)
c1 = x2 + x3		(8-4)
c2 = x0 – x3		(8-5)
c3 = 74 * x1		(8-6)

- The output values yi with i = 0..3 are then specified as follows:

y0 = ( 29*c0 + 55*c1 + c3 + 64 ) >> 7		(8-7)
y1 = ( 55*c2 – 29*c1 + c3 + 64 ) >> 7		(8-8)
y2 = ( 74*( x0 - x2 + x3 ) + 64 ) >> 7		(8-9)
y3 = ( 55*c0 + 29 * c2 – c3 + 64 ) >> 7		(8-10)

[Ed. (WJ): proponent’s comment - to share the quantization scaling value, transform precision (7-bit in this case) should be synchronized with DCT]

–	Otherwise, the following ordered steps apply: [Ed. (WJ): TBD – 4-point DCT]

#### Transformation process for 8 samples

Inputs of this process are an array of 8 samples of the scaled transform coefficients x with elements xi, with i = 0..7 and the transform type variable trType.

Output of this process is an array of 8 samples of the residual samples y with elements yi, with i = 0..7.

[Ed: (WJ) TBD]

8.5.4.2.1

Depending on trType, the following applies:

–	If tyType is equal to 1,

and if secTrSize is 4, the following ordered steps apply:

1. apply inverse DST4x4

2. apply forwardDCT4x4

else if secTrSize is 8, the following ordered steps apply:

- A set of intermediate values t0, to t7  from inut x0, to x7 is calculated as follows:

ti = xi for i=0..7		(8-11)

- Next values xi, i = 0..7 are modified as follows:

x0 = ( 122*t0 + 34*t1 - 15*t2 + 8*t3 -5* t4  + 3 t5 + 2*t6 + t7 + 64 ) >> 7	(8-12)
x1 = ( – 37*t0 + 118*t1 -27 *t2 + 13 *t3 -7*t4 + 4*t5 + 2*t6 + t7 + 64 ) >> 7	(8-13)
x2 = ( – 8*t0 – 33*t1 - 122*t2 + 19*t3 -9*t4 + 5*t5 + 3*t6 + t7 + 64 ) >> 7	(8-14)
x3 = ( – 3*t0 – 10*t1 + 24*t2 + 124*t3 -14*t4 + 6*t5 + 3*t6 + t7 + 64 ) >> 7	(8-15)

x4 = ( -2*t0 – 5*t1 +  8*t2 -16*t3 - 126* t4  + 10 *t5 +     4*t6 + 2*t7 + 64 ) >> 7	(8-16)
x5 = ( – t0     -3*t1 +  4*t2 -   6*t3 +  11*t4 + 127*t5 +     6*t6 + 2*t7 + 64 ) >> 7	(8-17)
x6 = ( – t0 –       t1 +  2*t2  -  3*t3 +    4*t4  -     7*t5 + 128*t6 + 4*t7 + 64 ) >> 7	(8-18)
x7 = (       – 10*t1 +      t2   -     t3  +    2*t4 -      2*t5  -     4*t6 + 128*t7 + 64 ) >> 7	(8-19)

inverseDCT8x8 is then invoked.

#### Transformation process for 16 samples

Inputs of this process are an array of 16 samples of the scaled transform coefficients x with elements xi, with i = 0..15 and the transform type variable trType.

Output of this process is an array of 16 samples of the residual samples y with elements yi, with i = 0..15.

[Ed: (WJ) TBD]

8.5.4.3.1

Depending on trType, the following applies:

–	If tyType is equal to 1,

and if secTrSize is 4, the following ordered steps apply:

1. apply inverse DST4x4

2. apply forwardDCT4x4

else if secTrSize is 8, the following ordered steps apply:

- A set of intermediate values t0, to t7  from inut x0, to x7 is calculated as follows:

ti = xi for i=0..7		(8-20)

- Next values xi, i = 0..7 are modified as follows:

x0 = ( 122*t0 + 34*t1 - 15*t2 + 8*t3 -5* t4  + 3 t5 + 2*t6 + t7 + 64 ) >> 7	(8-21)
x1 = ( – 37*t0 + 118*t1 -27 *t2 + 13 *t3 -7*t4 + 4*t5 + 2*t6 + t7 + 64 ) >> 7	(8-22)
x2 = ( – 8*t0 – 33*t1 - 122*t2 + 19*t3 -9*t4 + 5*t5 + 3*t6 + t7 + 64 ) >> 7	(8-23)
x3 = ( – 3*t0 – 10*t1 + 24*t2 + 124*t3 -14*t4 + 6*t5 + 3*t6 + t7 + 64 ) >> 7	(8-24)

x4 = ( -2*t0 – 5*t1 +  8*t2 -16*t3 - 126* t4  + 10 *t5 +     4*t6 + 2*t7 + 64 ) >> 7	(8-25)
x5 = ( – t0     -3*t1 +  4*t2 -   6*t3 +  11*t4 + 127*t5 +     6*t6 + 2*t7 + 64 ) >> 7	(8-26)
x6 = ( – t0 –       t1 +  2*t2  -  3*t3 +    4*t4  -     7*t5 + 128*t6 + 4*t7 + 64 ) >> 7	(8-27)
x7 = (       – 10*t1 +      t2   -     t3  +    2*t4 -      2*t5  -     4*t6 + 128*t7 + 64 ) >> 7	(8-28)

inverseDCT16x16 is then invoked.

#### Transformation process for 32 samples

Inputs of this process are an array of 32 samples of the scaled transform coefficients x with elements xi, with i=0..31 and the transform type variable trType.

Output of this process is an array of 32 samples of the residual samples y with elements yi, with i=0..31.

[Ed: (WJ) TBD]

8.5.4.4.1

Depending on trType, the following applies:

–	If tyType is equal to 1,

and if secTrSize is 4, the following ordered steps apply:

1. apply inverse DST4x4

2. apply forwardDCT4x4

else if secTrSize is 8, the following ordered steps apply:

- A set of intermediate values t0, to t7  from inut x0, to x7 is calculated as follows:

ti = xi for i=0..7		(8-29)

- Next values xi, i = 0..7 are modified as follows:

x0 = ( 122*t0 + 34*t1 - 15*t2 + 8*t3 -5* t4  + 3 t5 + 2*t6 + t7 + 64 ) >> 7	(8-30)
x1 = ( – 37*t0 + 118*t1 -27 *t2 + 13 *t3 -7*t4 + 4*t5 + 2*t6 + t7 + 64 ) >> 7	(8-31)
x2 = ( – 8*t0 – 33*t1 - 122*t2 + 19*t3 -9*t4 + 5*t5 + 3*t6 + t7 + 64 ) >> 7	(8-32)
x3 = ( – 3*t0 – 10*t1 + 24*t2 + 124*t3 -14*t4 + 6*t5 + 3*t6 + t7 + 64 ) >> 7	(8-33)

x4 = ( -2*t0 – 5*t1 +  8*t2 -16*t3 - 126* t4  + 10 *t5 +     4*t6 + 2*t7 + 64 ) >> 7	(8-34)
x5 = ( – t0     -3*t1 +  4*t2 -   6*t3 +  11*t4 + 127*t5 +     6*t6 + 2*t7 + 64 ) >> 7	(8-35)
x6 = ( – t0 –       t1 +  2*t2  -  3*t3 +    4*t4  -     7*t5 + 128*t6 + 4*t7 + 64 ) >> 7	(8-36)
x7 = (       – 10*t1 +      t2   -     t3  +    2*t4 -      2*t5  -     4*t6 + 128*t7 + 64 ) >> 7	(8-37)

DCT32x32 is then invoked.