| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F554 WG 11Number :m20987 |
| --- | --- |

| Title: | On secondary transforms for intra prediction residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

It was previously shown by Han, Saxena & Rose in ICASSP 2010, that following intra prediction, the optimal transform is not DCT, but DST Type-7 with performance close to KLT, along the direction of prediction, for the horizontal and vertical modes. The 4x4 DST from JCTVC-E125 was adopted in the HEVC Geneva meeting in March 2011. Shibahara & Nishi had proposed a mode-dependent 2-step transform in JCTVC-D151. In this contribution, various mode-dependent secondary transforms (MD-ST) are presented which are used after DCT based on the intra prediction mode. The proposed DCT/MD-ST transform scheme is applied at block sizes 8x8 and higher. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. The conventional quantization tables for HM 3.0 are retained and no changes have been made to the scanning order. Experimental results are provided with HM 3.0 as anchor for the test conditions as stipulated in Core Experiment 7. Average BD Rate gains of 0.6 %, 0.5 %, 0.3 % and 0.3% (respectively 0.4%, 0.3%, 0.3 % and 0.2 %) are obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low complexity settings respectively for the 8x8 MD-ST (respectively 4x4 MD-ST).

# Introduction

In HM 3.0, mode-dependent DCT/DST scheme from JCTVC-E125 [1] is used at block size 4x4. The mode-dependent DCT/DST scheme was published in literature in [2]. The 8x8 mode-dependent DCT/DST [1], and various other secondary transforms such as the Rotational Transform [3], and Mode-Dependent 2-step transform [4] are currently being investigated in Core Experiment 7 [5]. In this contribution, we propose various mode-dependent secondary transforms (MD-ST), which are used after DCT based on the intra prediction direction in HM 3.0. The proposed DCT/MD-ST scheme does not require a R-D search, or any additional signaling information.

Simulation results are provided for the proposed DCT/MD-ST scheme with HM 3.0 as anchor.

# Mode-Dependent Secondary Transform

## Mode-Mapping for using MD-ST

We retain the same mapping table from various prediction modes to the horizontal and vertical transform for the DCT/MD-ST scheme at block sizes 8x8 and higher, which is already present in HM 3.0 for mapping the intra prediction modes to either DCT or DST at size 4x4.

For example, for the horizontal unified intra direction prediction mode from [6] in HM 3.0, for the 4x4 block (DST, DCT) transform pair is used as the (horizontal,vertical) transform. In a similar vein, for the DCT/MD-ST (at block sizes 8x8 or larger) scheme, MD-ST is used in the horizontal direction after DCT, while for the vertical direction only DCT is used.

## Mode-Dependent Secondary Transform

### 8x8 MD-ST

The following 8x8 matrix A is used as the secondary matrix. A operates only on the 8x8 low-frequency coefficients of the 8x8 (or larger) DCT output:

A =           [ 122   -37    -8    -3    -2    -1    -1     0

34   118   -33   -10    -5    -3    -1    -1

-15   -27  -122    24     8     4     2     1

8    13    19   124   -16    -6    -3    -1

-5    -7    -9   -14  -126    11     4     2

3     4     5     6    10   127    -7    -2

2     2     3     3     4     6   128    -4

1     1     1     1     2     2     4   128 ]

The matrix A is derived by using the modified 8x8 covariance matrix in [4], wherein the slope for increment for the elements is reduced to 50% for the covariance matrix presented in [1].

### 4x4 MD-ST

We also present a 4x4 MD-ST (which operates only on the 4x4 low-frequency coefficients of DCT) T that is derived from the existing transforms in HM 3.0. Specifically,

T4 = C4TS4

where C4 and S4 are the conventional DCT and DST Type 7 [1,2] at size 4 already in HM3.0. To motivate the use of a secondary transform after DCT, let us consider the existing transform scheme in HM 3.0. While a combination of DCT C4 and DST S4 in Fig. 1 below has been shown to be the optimal transform [1, 2], an equivalent mathematical way of representation can be described by Fig. 3:

Fig. 1: Application of an additional transform as an alternate primary transform

Fig. 2: Application of an additional transform as a secondary transform after DCT.

Let us define T = C4-1 . S4 = C4T. S4, where I as an Identity matrix. The above application of Mode-Dependent DCT/DST in Fig.1 can then equivalently be expressed as an operation of DCT/MD-ST scheme as in Fig. 2.

Next, in order to improve the compression efficiency, primary alternate transforms other than conventional DCT can be applied at block sizes 8x8, 16x16 and 32x32. However, these primary transforms will have the same size as the block size. In general, these alternate transforms at higher block sizes such as 32x32 may have marginal gains that may not justify the enormous cost of supporting additional 32x32 transform in the hardware.

Also, since most of the energy in the DCT coefficients is concentrated in the low-frequency coefficients, it should be adequate to perform operations only on the first few coefficients (typically 4 or 8) of DCT output. These operations can be performed by simply using a MD-ST of size 4x4 or 8x8 such as presented above. Furthermore, the same secondary transform derived for a block size (such as 8x8) can be applied at higher block sizes (such as 16x16 or 32x32), while this is not true for alternate primary transform. This re-utilization of MD-ST at higher block sizes is one of the most important advantages of secondary transform.

Shibahara & Nishi in [4] have proposed a MD-ST at size 4x4. Here, we derive our MD-ST from the 4x4 DCT and DST, already existing in HM 3.0.

# Discussion on Fast Implementation

Both the DCT and DST in HM 3.0 have fast implementations. The DCT in HM 3.0 is implemented using 48 multiplications, 96 additions and 32 shifts. For the DST in HM3.0, the number of mults, adds and shifts are respectively 64, 120 and 32. A 4x4 MD-ST implemented using these two will therefore require 112 mults, 216 adds and 64 shifts. Note that this number can be further reduced by finding faster implementations for DCT and DST, as being investigated in Core Experiments 10 and 7 on Core and Additional Transforms respectively.

For a SIMD implementation, where a matrix multiplication is preferred, the following matrix can be used as the 4x4 MD-ST:

T4 = C4TS4

= round(C4T*S4/128)

=  [ 121    37     18      8

-41    117     31    10

-8     -37    121     18

-3      -6    -21    126 ]

This implementation would require 128 multiplications, 128 additions and 32 shifts.

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [5] were followed for conducting Experiments 1 ansd 2. Here the anchor was HM 3.0 with the conventional DCT at block sizes 8x8 and larger. The performance of the various proposed mode-dependent DCT/MD-ST transform schemes was evaluated for the following settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

Experiment 1:  8x8 MD-ST with HM 3.0 as Anchor

Table 1 shows the simulation results when the proposed DCT/ 8x8 MD-ST scheme is applied at size 8x8 and higher. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_8x8.xls. We also implemented this by clipping the transform output at the end of each stage within 16 bits, and the results are identical, and provided in JCTVC-F554_MD-ST_8x8_Clipped.xls. We would like to thank Panasonic for performing the cross-check for this in JCTVC-F680.

Table 1: Simulation Results for the proposed mode-dependent DCT/ 8x8 MD-ST with HM 3.0 as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.7 | -0.5 | -0.7 | -0.5 | -0.9 | -1.2 |  |
|  | Class B | -0.7 | -0.8 | -0.8 | -0.4 | -1.0 | -1.0 |  |
|  | Class C | -0.4 | -0.6 | -0.6 | -0.4 | -0.7 | -0.7 |  |
|  | Class D | -0.2 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |  |
|  | Class E | -0.9 | -1.2 | -1.2 | -0.7 | -1.1 | -1.3 |  |
|  | All | -0.6 | -0.7 | -0.7 | -0.5 | -0.8 | -0.9 |  |
|  | Enc Time[%] | 102% |  |  | 103% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 103% |  |  |  |
|  |  |  | Random access HE |  |  | Random access LC |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.5 | -0.2 | -0.5 | -0.3 | -0.3 | -0.6 |  |
|  | Class B | -0.5 | -0.5 | -0.2 | -0.3 | -0.5 | -0.6 |  |
|  | Class C | -0.3 | -0.1 | -0.4 | -0.3 | -0.5 | -0.5 |  |
|  | Class D | -0.1 | 0.0 | -0.3 | -0.2 | -0.3 | -0.2 |  |
|  | Class E |  |  |  |  |  |  |  |
|  | All | -0.3 | -0.2 | -0.4 | -0.3 | -0.4 | -0.5 |  |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 99% |  |  | 100% |  |  |  |

From the above experimental results, average BD rate gains of 0.6%, 0.5 %, 0.3 % and 0.3 % are obtained for Intra High Efficiency, Intra Low complexity, Random Access High Efficiency and Random Access Low Complexity settings. The additional encoding/decoding time due to the introduction of 8x8 MD-ST is almost negligible.

Experiment 2:  4x4 MD-ST (using existing transforms in HM 3.0) with HM 3.0 as Anchor

Table 2 shows the simulation results when the proposed DCT/ 4x4 MD-ST scheme is applied at size 8x8 and higher. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_4x4.xls.We also implemented this by clipping the transform output at the end of each stage within 16 bits, and the results are identical, and provided in JCTVC-F554_MD-ST_4x4_Clipped.xls. Also, to avoid the clipping at first stages, we shifted the 2 stages of transform by 8 and 6 bits respectively. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_4x4_Clipped_2.xls. We would like to thank Panasonic and Sony for performing the cross-check for this in JCTVC-F680 and JCTVC-F716.

Table 2: Simulation Results for the proposed mode-dependent DCT/ 4x4 MD-ST with HM 3.0 as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.4 | -0.2 | -0.2 | -0.3 | -0.4 | -0.5 |  |
|  | Class B | -0.4 | -0.3 | -0.3 | -0.2 | -0.5 | -0.5 |  |
|  | Class C | -0.2 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |  |
|  | Class D | -0.1 | -0.3 | -0.3 | -0.1 | -0.3 | -0.3 |  |
|  | Class E | -0.6 | -0.8 | -0.8 | -0.5 | -0.8 | -0.9 |  |
|  | All | -0.4 | -0.4 | -0.4 | -0.3 | -0.5 | -0.5 |  |
|  | Enc Time[%] | 101% |  |  | 101% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 102% |  |  |  |
|  |  |  | Random access HE |  |  | Random access LC |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.4 | 0.0 | -0.6 | -0.2 | -0.3 | -0.1 |  |
|  | Class B | -0.3 | -0.2 | -0.1 | -0.2 | -0.2 | -0.3 |  |
|  | Class C | -0.2 | -0.1 | -0.4 | -0.2 | -0.3 | -0.3 |  |
|  | Class D | -0.2 | -0.3 | -0.4 | -0.2 | -0.4 | -0.4 |  |
|  | Class E |  |  |  |  |  |  |  |
|  | All | -0.3 | -0.2 | -0.4 | -0.2 | -0.3 | -0.3 |  |
|  | Enc Time[%] | 100% |  |  | 100% |  |  |  |
|  | Dec Time[%] | 100% |  |  | 101% |  |  |  |

From the above experimental results, average BD rate gains of 0.4%, 0.3 %, 0.3 % and 0.2 % are obtained for Intra High Efficiency, Intra Low complexity, Random Access High Efficiency and Random Access Low Complexity settings. The additional encoding/decoding time due to the introduction of 4x4 MD-ST is almost negligible. Further note that here no additional transforms are introduced in HM 3.0, and the existing transforms in HM3.0 are used to obtain the 4x4 MD-ST.

Experiment 3:  8x8 MD-ST with SDIP+DST4x4 for rectangular blocks as Anchor

Table 3 shows the simulation results when the proposed DCT/ 8x8 MD-ST scheme is applied to square blocks of size 8x8 and higher. The anchor for these set of simulations is based on software implementation “hm-3.0-dev-sdip” in the SVN server with revision number r866. Also, in the anchor the 4x4 DCT/DST scheme from [1] was harmonized with SDIP. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_8x8_SDIP.xls. We would like to thank Panasonic for performing the cross-check for this in JCTVC-F680.

Table 3: Simulation Results for the proposed mode-dependent DCT/ 8x8 MD-ST (only on square blocks) with SDIP+4x4 DST Harmonized as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.7 | -0.2 | -0.3 | -0.5 | -0.7 | -0.9 |  |
|  | Class B | -0.7 | -0.5 | -0.3 | -0.5 | -0.8 | -0.8 |  |
|  | Class C | -0.4 | -0.3 | -0.3 | -0.4 | -0.5 | -0.5 |  |
|  | Class D | -0.2 | -0.2 | -0.3 | -0.3 | -0.4 | -0.3 |  |
|  | Class E | -0.9 | -0.2 | -0.3 | -0.8 | -0.4 | -0.6 |  |
|  | Overall | -0.6 | -0.3 | -0.3 | -0.5 | -0.6 | -0.6 |  |
|  | Enc Time[%] | 101% |  |  | 103% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 103% |  |  |  |

From the above experimental results, average BD rate gains of 0.6% and 0.5 % are obtained for Intra High Efficiency and Intra Low complexity settings. This shows that even on top of SDIP, a secondary transform provides similar gains as for HM 3.0 in Experiment 1.

Experiment 4:  8x8 MD-ST for square and rectangular blocks with SDIP+DST4x4 for rectangular blocks as Anchor

Table 4 shows the simulation results when the proposed DCT/ 8x8 MD-ST scheme is applied to square blocks of sizes 8x8, 16x16 and 32x32, as well as to rectangular blocks of size 16x1, 8x2, 16x4 and 32x8 in SDIP. The anchor for these set of simulations is based on software implementation “hm-3.0-dev-sdip” in the SVN server with revision number r866. Also, in the anchor the 4x4 DCT/DST scheme from [1] was harmonized with SDIP. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_8x8_SDIP_Rect.xls. We would like to thank Panasonic for performing the cross-check for this in JCTVC-F680.

Table 4: Simulation Results for the proposed mode-dependent DCT/ 8x8 MD-ST (on square and rectangular blocks) with SDIP+4x4 DST Harmonized as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.8 | -0.4 | -0.6 | -0.6 | -0.9 | -1.1 |  |
|  | Class B | -0.9 | -1.0 | -0.8 | -0.7 | -1.0 | -1.0 |  |
|  | Class C | -0.5 | -0.6 | -0.6 | -0.5 | -0.6 | -0.6 |  |
|  | Class D | -0.3 | -0.4 | -0.4 | -0.4 | -0.5 | -0.4 |  |
|  | Class E | -1.2 | -0.9 | -0.9 | -1.1 | -0.8 | -0.9 |  |
|  | Overall | -0.7 | -0.7 | -0.7 | -0.7 | -0.7 | -0.8 |  |
|  | Enc Time[%] | 101% |  |  | 103% |  |  |  |
|  | Dec Time[%] | 103% |  |  | 103% |  |  |  |

From the above experimental results, average BD rate gains of 0.7% and 0.7 % are obtained for Intra High Efficiency and Intra Low complexity settings. This shows that compression efficiency for the rectangular blocks of SDIP, similar to square blocks in Experiment 3, is also improved by applying the proposed DCT/MD-ST technique.

Experiment 5:  4x4 MD-ST for square and rectangular blocks with SDIP+DST4x4 for rectangular blocks as Anchor

Table 5 shows the simulation results when the proposed DCT/ 4x4 MD-ST scheme is applied to square blocks of sizes 8x8, 16x16 and 32x32, as well as to rectangular blocks of size 16x1, 8x2, 16x4 and 32x8 in SDIP. The anchor for these set of simulations is based on software implementation “hm-3.0-dev-sdip” in the SVN server with revision number r866. Also, in the anchor the 4x4 DCT/DST scheme from [1] was harmonized with SDIP. Detailed results are available in the attached Excel sheet JCTVC-F554_MD-ST_4x4_SDIP_Rect.xls. We would like to thank Panasonic and Sony for performing the cross-check for this in JCTVC-F680 and JCTVC-F716.

Table 5: Simulation Results for the proposed mode-dependent DCT/ 4x4 MD-ST (on square and rectangular blocks) with SDIP+4x4 DST Harmonized as anchor

|  |  | All Intra HE |  |  | All Intra LC |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | Y | U | V |  |
|  | Class A | -0.4 | -0.2 | -0.2 | -0.4 | -0.4 | -0.5 |  |
|  | Class B | -0.6 | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 |  |
|  | Class C | -0.3 | -0.4 | -0.4 | -0.4 | -0.4 | -0.4 |  |
|  | Class D | -0.2 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |  |
|  | Class E | -0.9 | -0.7 | -0.7 | -0.8 | -0.5 | -0.7 |  |
|  | Overall | -0.5 | -0.4 | -0.4 | -0.4 | -0.4 | -0.5 |  |
|  | Enc Time[%] | 101% |  |  | 103% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 102% |  |  |  |

From the above experimental results, average BD rate gains of 0.5% and 0.4 % are obtained for Intra High Efficiency and Intra Low complexity settings for the mode-dependent DCT/4x4 MD-ST scheme. This shows that even on top of SDIP, a secondary transform provides additional gains for HM 3.0 in Experiment 2 for the mode-dependent DCT/4x4 MD-ST scheme.

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed 8x8 and 4x4 MD-ST’s were derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 3.0.

Transform Matrices: For the 8x8 MD-ST, we only require the storage of one additional 8x8 matrix corresponding to 8x8 MD-ST.

For the 4x4 MD-ST, we do not require any additional transform matrix.

Bit Shift at Intermediate Levels: In our implementation, we shift the output of 8x8 MD-ST and 4x4 MD-ST by 7 bits after vertical or horizontal transform.

The bit-precision for all the transforms is 16 bits, similar to in HM 3.0.

Scanning: In this proposal we have retained the conventional HM 3.0 scans

Operation Counts: We next present the operation count Table for the proposed 8x8 MD-ST and 4x4 M-ST transforms. For ease of comparison with the other transforms being evaluated in CE 7, we copy  Table 4 for the operation counts of all the other transforms, and add the numbers for our proposed 4x4 MD-ST and 8x8 MD-ST. Finally, in the top row, we also add comments whether any new additional transforms in HM 3,0 are introduced in the various proposals.

Also, note that for the 8x8 MD-ST, 11 matrix elements are 1’s/0’s/-1’s, implying that the number of mults can be further reduced from 64x8x2 = 1024 to (64-11)x8x2 = 53x16 = 832. Further note that 19 elements are just powers of 2’s. So these 19 multiplications can be reduced to simple shift operators. Hence, 19x8x2 = 304 mults can simply be replaced by shifts. Similarly the number of adds reduce to (64-1)x8x2 = 1008, because of the presence of 1 zero in 8x8 MD-ST. Thus the total number of mults, shifts and adds can go down from 1024 mults, 1024 adds and 128 shifts for a 8x8 secondary matrix multiplication to 832-304 =528 mults, 1024 adds, and 128+304=432 shifts.

For the 4x4 MD-ST, we simply add the numbers of existing DCT and DST in HM 3.0, and also mention that this number can be further reduced by various related proposals as being investigated in Core Experiments 10 and 7 on Core and Additional Transforms respectively. For the 4x4 MD-ST, we also need to take 6 swap operations twice while taking the transpose for the horizontal transform of the top 4x4 blocks. If the operations of clipping are also included, then for 8x8 MD-ST, for worst case, we require additional 128 clips (equivalent to 256 additions), while for 4x4 MD-ST, we require additional 64 clips (128 additions) or 32 clips (or 64 additions) depending on implementation (if clips are applied to both, or only second stage respectively).

Similarly for Tool 3, additional number of operations will be 64 or 32 clips (implying 128 or 64 adds) depending on implementation.

Table 6. Inverse transform operations count summary for all tools in CE 7 (obtained from CE 7 Summary Report [7]) and adding clipping operations for Tool 3, and adding operation count tables for the proposed 8x8 and 4x4 MD-ST

|  |  |  |  | Operation Counts |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tool Trans-form Size |  | HM2 | HM3 (DCT) |  | HM3 (DST) | Tool 1 (F138) 8x8 MD DCT/ KLT | Tool 2 (F282) 8x8 MD DCT/ DST | Tool 3 (F224) MD secondary 4x4 | Tool 4 (F294) RD secondary ROT | Tool 5 (F229) RD or fast 2D Intra/Inter DST-II | Tool 6 (F283) Reduced-complex. 4x4 DST | F-554 MD secondary 8x8/4x4 |
| Comments on Introducing New Transform Core |  | NA | NA |  | NA | Yes | Yes | Yes | Yes | Yes¦Yes | No(Simplifying existing HM 3.0 DST) | Yes¦No |
| 4x4 | Mults | 0 | 48 |  | 64 |  |  |  |  | +0 ¦ 48 | 40 |  |
| 8x8 | Adds | 80 | 96 |  | 120 |  |  |  |  | +8 ¦ 96 | 120 |  |
|  | Shifts | 32 | 32 |  | 32 |  |  |  |  | +0 ¦ 32 | 56 |  |
|  | Mults | 0 | 352 |  |  | 1024 | 1024 | +128 | +240 | +0 ¦ 352 |  | +528¦+112 |
| 16x16 | Adds | 640 | 576 |  |  | 1024 | 1024 | +256 (or 192) | +288 | +32 ¦ 576 |  | 1264¦+344(or 280) |
|  | Shifts | 352 | 128 |  |  | 128 | 128 | +32 | +336 | +0 ¦ 128 |  | 432¦+64 |
|  | Mults | 1408 | 3752 |  |  |  |  | +128 | +240 | +0 ¦ 3752 |  | +528¦+112 |
| 32x32 | Adds | 2624 | 3712 |  |  |  |  | +256 (or 192) | +288 | +128 ¦ 3712 |  | 1136¦+344(or 280) |
|  | Shifts | 1600 | 512 |  |  |  |  | +32 | +336 | +0 ¦ 512 |  | 432¦+64 |
|  | Mults | 7424 | 21888 |  |  |  |  | +128 | +240 | +0 ¦ 21888 |  | +528¦+112 |
|  | Adds | 13440 | 25856 |  |  |  |  | +256 (or 192) | +288 | +512 ¦ 25856 |  | 1136¦+344(or 280) |
|  | Shifts | 7296 | 2048 |  |  |  |  | +32 | +336 | +0 ¦ 2048 |  | 432¦+64 |

Finally note that the 112 mults, 344 (or 280) adds, and 64 shifts for the proposed 4x4 MD-ST will reduce to (112-16) = 96 mults, 344-8 = 336 (or 280-8=272) adds, and 64 shifts if the fast implementation of JCTVC-F153 is adopted with 6 mults, and 10 adds (instead of current 8 mults and 11 adds in HM 3.0 is used for implementation. This will not change any Rate or PSNR values).

## Discussion on Complexity for MD-ST’s

While complexity is multi-dimensional, an attractive property of secondary transforms is less number of operations (mults, adds and shifts) as compared to primary alternate transforms at sizes 8x8 or higher. Further the proposed 4x4 MD-ST does not require any additional transform core. Also 4x4 MD-ST can be mathematically applied as a secondary transform for all blocks via the matrix presented in Section 3. Or it can be implemented using the already existing fast algorithms for DCT and DST in HM 3.0.

# Conclusions

In this contribution, we have presented a mode-dependent DCT/MD-ST scheme for intra-prediction residual coding instead of the conventional DCT at block sizes 8x8 and higher. The proposed DCT/8x8MD-ST scheme requires the storage of only 1 additional 8x8 MD-ST matrix, is based on the intra prediction mode, and does not require any explicit signaling. Further, we also present a low-complexity 4x4 MD-ST, which is derived from the DCT and DST-Type 7 matrices already existing in HM3.0 at size 4. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show significant gains in compression performance as compared to HM 3.0 anchors. We recommend adopting this proposal into HM.

# References

1. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” JCTVC-E125, Geneva, Switzerland, March 2011.

2. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

3. E. Alshina, A. Alshin, F. Fernandes, A. Saxena, V. Seregin, Z. Ma, W. J. Han, “CE7: Experimental results of ROT by Samsung,” JCTVC-E380, Geneva, Switzerland, March 2011.

4. Y. Shibahara and T. Nishi, “Mode Dependent 2-step transform for Intra coding,” JCTVC-D151, Daegu, Korea, January 2011.

5. R. Cohen et al., “Description of Core Experiment 7 (CE7): Additional Transforms,” JCTVC-E707, Geneva, Switzerland, March 2011.

6. Min et al., “Unification of the Directional Intra Prediction Methods in TMuC,” ITU-T JCTVC-B100_revision02, Geneva, Switzerland, July 2010.

7. R. Cohen et al., “CE7: Summary report of Core Experiment on additional transforms,” JCTVC-F027, Torino, Italy, July 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Two implementations were described for Tool 5. In the first, the IDST is implemented as a secondary transform by performing the HM3 IDCT together with coefficients switching and sign alternations. In the second, the IDST is implemented as a replacement transform but with the same number of operations as HM3 IDCT [footnote from [7]].