| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, IT, 14-22 July, 2011 | Document: JCTVC-F558 |
| --- | --- |

| Title: | CE7: Crosscheck of tool 3 – Panasonic's mode dependent secondary transform (JCTVC-F224) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | R. Joshi 5775 Morehouse Drive, San Diego, CA 92121-1714 USA | Tel: +1-858-658-4511 Email: rajanj@qualcomm.com |  |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution provides a summary of a cross-check performed by Qualcomm of Panasonics' mode dependent secondary transform proposal (tool 3) in CE7 (JCTVC-F224). The results of the crosscheck matched those reported by Panasonic. The encoding and decoding times also matched closely.

- Introduction

Panasonic proposes a mode-dependent secondary transform that operates on the low-frequency coefficients output by the HM core transform. The secondary transforms is a 4x4 pseudo-DST and operates for 8x8 and larger blocks.

# Results

Results for 4 configurations, AI-HE, AI-LC, RA-HE, and RA-LC, were crosschecked, as specified in CE7 test conditions. Table 1 shows a summary of the BD-rate results. More detailed coding results can be found in the Excel attachment. No YUV files were written when the decoding times were measured. Encoding as well as decoding was done on a LINUX cluster (64 bit) by specifying only one type of CPU.

Table 1: BD-rate results for the proposed mode dependent secondary transform

# Conclusions

We have verified results reported in Panasonics' proposed mode dependent secondary transform. For all-intra configuration, BD-rate of -0.4% and -0.3% is achieved for HE and LC, respectively. For random access configuration, BD-rate of -0.3% and -0.2% is achieved for HE and LC, respectively. The encoding and the decoding times were within margin of error of those reported by Panasonic.