| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, Italy, 14-22 July 2011 | Document: JCTVC-F562 |
| --- | --- |

| Title: | Updated Video Test Material Submission for "Screen Content" Coding Experiments: Scrolling Text, Overlays, Editing, Window Switching, and On-line Gaming |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gregory Cook (1) Wen Gao (1) Jiantong Zhou (2) Haoping Yu (1) (1). Huawei Technologies (USA) 2330 Central Expressway Santa Clara, CA 95050 (2). Huawei Technologies Co., LTD Section B, Bantian, Longgang District Huawei Base, Shenzhen, China | Tel: +1 408 330 4816 Email: gregory.cook@huawei.com wen.gao@huawei.com zhoujiantong@huawei.com haoping.yu@huawei.com |  |
| Source: | Huawei Technologies (USA) |  |  |

_____________________________

# Abstract

This contribution consists of a submission of twelve test sequences in response to W11867, “Request for Video Test Material for ‘Screen Content’ Coding Experiments”; nine of the sequences are updates to previously submitted material and three are new sequences.   The sequences consist of camera-view content with text overlays and scrolling text, and camera-view content with document editing, scrolling and window switching, high contrast text and gradient backgrounds and on-line gaming.  The sequences were updated based on comments from the break out group, and follow the general guidelines given in W11867.  Simulations based on the test conditions agreed to by the Screen Content Coding Ad Hoc Group were made.

# Introduction

This contribution is an update to [1], and submitted in further response to [2], which is a request for video test material for “Screen Content” Coding (SCC) experiments.

During the 5th JCT-VC meeting in Geneva, comments on the original submitted sequences were obtained during a break-out group session.  These comments were then used as the basis to update the sequences.  The changes to the sequence are described in Section 2.  Also in Section 2 are described two new sequences which feature text and graphics in high contrast and gradient backgrounds in typical document sharing applications.  In addition, a new sequence is submitted which pertains to on-line gaming content, and is also described in Section 2.

# Sequence Description

## Camera-view content with text overlays and scrolling text

The sequences BasketballDrillText_832x480_50.yuv, BQMallText_832x480_60_yuv, PartySceneText_832x480_50.yuv, RaceHorsesText_832x480_30.yuv, BasketballDriveText_1920x1080_50.yuv, BQTerraceText_1920x1080_60.yuv, CactusText_1920x1080_50.yuv were created using text overlays and scrolling text over camera-view content.   The procedure for creating these sequences is essentially unchanged as described in [1].   In order to provide a more challenging sequence, it was suggested that the scrolling text at the bottom should be moved up so that there was a gap between the bottom of the picture and the bottom of the scrolling text.  In addition, it was recommended that the size and placement of the scrolling text be such that it was not an integer multiple of 16 or 64 so that it was likely to fall across multiple CU boundaries.   The scrolling text/graphic has been moved up so that there is a gap of 12 pixels from the bottom and is 38 pixels wide for the 832x480 case.    For the 1920x1080 case, there is a gap at the bottom of 28 pixels, and the graphic is now 74 pixels wide.   Also the overlay has been increased in size to match the size of the picture, 200x100 pixels for the 832x480 case and 400x200 in the 1920x1080 case, but no other modifications were made.  Copyright permission to use the sequence for video compression research purposes has been obtained.

## Camera-view content with document editing, scrolling and window switching

The sequences ScreenCaptureDocEditing_1280x720_60.yuv and ScreenCaptureWinSwitching_1280x720_60.yuv were created by mixing camera-view content with a screen capture of typical graphical user interface actions.   The first sequence has primarily document editing actions such as scrolling, insertion, and deletion, while the second sequence has primarily document window resizing, moving, and switching effects.    The consensus of the break-out group was that the sequences could be made more challenging by including fonts of different type and size.     Consequently, each sequence has a variety of fonts and the pace of the changes is increased.  In addition, the document editor and slide editor are copyright free software.  Copyright permission to use the sequence for video compression research purposes has been obtained.

## Document sharing content with moving windows and slide editing

The sequences ScreenCaptureSlideEditing_1280x720_30.yuv  and ScreenCaptureWinMoving_1280x720_30.yuv  (each 300 frames in length) were created using the method described in [1].   These sequences were created for the purpose of improving compression of text and graphics in high contrast and gradient backgrounds.  The first sequence consists of several windows containing English text of several different font types and sizes, and also Chinese text of several different font sizes.   Some of the text is strictly black letters on a white background, while other fonts have a chroma component at the edges of the letters.   The windows are then brought to the foreground, moved, changed in size, and finally closed; these actions are typical for a document sharing application.   The second sequence has a black-on-white editing application with a slide presentation application that has a gradient background for the slides.  This results in areas of both high and low contrast, and also extensive areas of gradient color.   The actions in this sequence are also typical for a document sharing application; in this case the slide is edited with text copied from the document.

As shown in the attached spreadsheet, the SDIP technique is very effective when used for screen content material as described in Section 1.2 and Section 1.3.   The average Y BD-rate for the four sequences for an all Intra HE case is –13.5%, as shown in Table 1.

## Gaming graphics

A new sequence of length 500 frames and a frame rate of 30 frames/second, called chinaspeed_1024x768_30.yuv, has been obtained.   This sequence is typical of “3D action oriented” gaming graphics, which is one of the categories requested in [1].  On-line gaming is a new category of video games where the content is no longer created and rendered on a local gaming console, but on a remotely located server.  The local computer or settop box is required to only decompress the video stream sent via the Internet.   As a consequence, highly efficient compression and low-latency are clear requirements for this challenging application.

The sequence consists of a rear-view of a car rendered in 3D perspective, with moving backgrounds of a road surface and the sky.   Both smooth gradient and more “random” textures are used in the rendering.  The sequence also has stationary text and overlays which indicate the status of the game.    Copyright permission to use the sequence for video compression research purposes has been obtained.

As shown in the attached spreadsheet, the SDIP technique is quite effective when used for gaming material.  The average Y BD-rate for screen capture sequences for an all Intra case is -13.5%, and -6.3% for the on-line gaming sequence, as given in Table 1.

|  | Intra |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Screen Capture | -13.5 | -13.5 | -13.4 | -13.8 | -11.8 | -12.1 |  |
| Gaming | -6.3 | -6.8 | -5.8 | -6.5 | -5.6 | -4.4 |  |
| All | -12.0 | -12.2 | -11.9 | -12.4 | -10.6 | -10.5 |  |
|  | Random access |  |  | Random access LoCo |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Screen Capture | -10.3 | -10.2 | -10.1 | -10.7 | -9.3 | -9.6 |  |
| Gaming | -2.8 | -3.2 | -2.5 | -3.3 | -2.4 | -1.2 |  |
| All | -8.8 | -8.8 | -8.6 | -9.2 | -7.9 | -7.9 |  |
|  | Low delay |  |  | Low delay LoCo |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Screen Capture | -5.9 | -6.3 | -5.9 | -6.6 | -5.7 | -6.5 |  |
| Gaming | -1.3 | -1.5 | -1.4 | -1.8 | -1.2 | -1.0 |  |
| All | -5.0 | -5.3 | -5.0 | -5.6 | -4.8 | -5.4 |  |
|  | Low delay P |  |  | Low delay P LoCo |  |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Screen Capture | -5.8 | -6.5 | -6.5 | -6.5 | -5.6 | -6.1 |  |
| Gaming | -1.2 | -1.5 | -1.4 | -1.6 | -1.5 | -0.8 |  |
| All | -4.9 | -5.5 | -5.5 | -5.5 | -4.8 | -5.0 |  |

Table 1: Summary of results for Screen Capture and Gaming sequences comparing HM-3.0 and HM-3.0-dev-sdip

# Patent rights declaration(s)

Huawei Technologies (USA) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

The following copyright statement accompanies each of the following submitted sequences: BasketballDrillText_832x480_50.yuv, BQMallText_832x480_60_yuv, PartySceneText_832x480_50.yuv, RaceHorsesText_832x480_30.yuv, BasketballDriveText_1920x1080_50.yuv, BQTerraceText_1920x1080_60.yuv, ScreenCaptureDocEditing_1280x720_60.yuv and ScreenCaptureWinSwitching_1280x720_60.yuv.

Other than the copyrighted sequences used as a basis for the creation of these sequences, these sequences and all intellectual property rights therein remain the property of Huawei Technologies (USA).    Copyright as claimed by the original owners, in this case NTT DOCOMO, INC, remains with the owners.  These sequences may only be used for the purpose of developing, testing and promulgating  technology standards.   Huawei Technologies (USA) makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.

The following copyright statement accompanies the following submitted sequence: CactusText_1920x1080_60.yuv

Other than the copyrighted sequences used as a basis for the creation of these sequences, these sequences and all intellectual property rights therein remain the property of Huawei Technologies (USA).    Copyright as claimed by the original owners, in this case RAI, remains with the owners.  These sequences may only be used for the purpose of developing, testing and promulgating  technology standards.   Huawei Technologies (USA) makes no warranties with respect to the sequences and expressly disclaims any warranties regarding their fitness for any purpose.

The following copyright statement accompanies the following submitted sequences: ScreenCaptureWinMoving_1280x720_30.yuv and ScreenCaptureSlideEditing_1280x720_30.yuv.

This sequence and all intellectual property rights therein remain the property of Huawei Technologies (USA).  This sequence may only be used for the purpose of developing, testing and promulgating technology standards. Huawei Technologies (USA) makes no warranties with respect to the sequence and expressly disclaims any warranties regarding its fitness for any purpose.

The following copyright statement accompanies the following submitted sequence: Chinaspeed_1024x768_30.

This sequence and all intellectual property rights therein remain the property of Shanghai Shulong Computer Technology Co., Ltd. This material shall be used, copied, modified, or distributed within JCTVC only for developing the High-Performance Video Coding (HEVC) standards and for testing and promoting such standards. This Copyright and permission notice shall be duplicated whenever the data are copied or distributed. This material shall not be distributed with charge. For distributing of altered or processed versions of the data, any user must clearly indicate that the data has been altered and thus cannot be relied upon as representing the original material. Shanda Interactive Entertainment Limited makes no warranties with respect to the material and expressly disclaims any warranties regarding its fitness for any purpose. Unless the above conditions are agreed to by the recipient, no permission is granted for any use, copying, modification and distribution of the accompanying data.

# References

[1]  G. Cook, D. Wang, W. Gao, J. Zhou, and H. Yu, “Video Test Material Submission for "Screen Content" Coding Experiments: Scrolling Text, Overlays, Editing, Window Switching,” JCTVC-E305, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

[2] “Request for Video Test Material for "Screen Content" Coding Experiments”, ITU-T Q6/16 Visual Coding and ISO/IEC JTC1/SC29/WG11 Coding of Moving Pictures and Audio, W11867, January 2011.