| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 6th Meeting: Torino, 14-22 July, 2011 | Document: JCTVC-F563 |
| --- | --- |

| Title: | Non-Square Transform for 2NxN and Nx2N Motion Partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo, Joel Sole Rajan Joshi Peisong Chen Xianglin Wang Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121 USA | Tel: Email: | 1-858-651-8125 liweig@qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

The current HM software only employs square-shaped transforms. In this contribution, for inter non-square PUs, i.e., 2NxN PU and Nx2N PU, additional non-square transforms 2NxN and Nx2N are integrated in the RQT structure. Non-square transforms achieve an average BD-rate reduction of 0.2% for the RA HE case, 0.6% for LB HE, and 0.7% for LP HE. For the low complexity, the average BD-rate reductions are 0.1% for RA LC, 0.6% for LB LC and 0.6% for LP LC.

# Introduction

In current HM software, there are only square-shaped transforms. For non-square shape PU (2NxN and Nx2N), the transform either goes across motion boundary (2Nx2N transform) or covers only part of a motion partition (NxN or smaller transforms). In this contribution, we introduce 2NxN transform and Nx2N transform for 2NxN and Nx2N PUs respectively. These non-square transforms are integrated in the current RQT structure as additional transform choices.

# Non-square transform for 2NxN and Nx2N PU

Non-square transforms are introduced for the luma residue blocks of the following PU sizes: 32x16, 16x32, 16x8, 8x16, 8x4, and 4x8. For these PUs, the non-square transform size is the same as the PU size. No non-square transform is used for luma block of PU of other sizes, and no non-square transform is used for chroma block.

The proposed non-square transform reuses the transform core in the current HM software and no new transform core is introduced. For example, 2NxN transform is implemented using the transform core of NxN transform and the transform core of 2Nx2N transform. The quantization/scaling process is the same as square-shaped transform except different quantization matrix (g_auiQ_NS) and inverse quantization matrix (g_auiIQ_NS) are used due to different norms. Both matrices have 6 elements. The quantized coefficients are vectorized using Zig-Zag scan. In CABAC, coefficients of the non-square transform are mapped to locations in the square-shaped transform to share the contexts, so no new contexts are introduced. In CAVLC, coefficients of non-square transform are encoded using existing tables.

Non-square transforms are integrated in the RQT of non-square PUs. We use Nx2N PU as an example to describe the integration. For an Nx2N PU, both Nx2N transform and 2Nx2N transform are placed at the root level (level 0) of the RQT. The transforms at other levels are not changed (i.e., level 1- NxN; level 2- N/2xN/2). The encoder compares Nx2N and 2Nx2N transforms to select one as the root-level transform. The result of the selected root level transform is compared to the results of further split. Fig.1 shows the transforms used for Nx2N PU and the corresponding RQT level/depth.

Two syntax elements are introduced for non-square transforms. At root level of RQT, if transform split flag is 0 (i.e. root level transform is selected) and there is non zero luma coefficients, a flag non-square transform selection NST need to be transmitted. NST = 0 means the selected root level transform is square-shaped transform 2Nx2N and NST =1 means the selected transform is non-square transform Nx2N. If NST =1, an additional cbf for the second Nx2N block, CbfY1, is used. In CABAC, CbfY1 shares the context of cbfY of square transform block. For NST flag, new contexts are used and these are the only contexts introduced in this contribution. In CAVLC, the coding of NST and CbfY1 is combined with the coding of split flag and other cbf flags.

Figure 1 RQT for Nx2N PU (a) 2Nx2N transform (level 0), (b) Nx2N transform (level 0), (c) NxN transform (level 1); (d) N/2xN/2 transform (level 2) for the right-top subblock

# Experimental results

The proposed non-square transform is implemented on top of HM 3.0. The coding efficiency is evaluated under common conditions specified in [1]. The coding efficiency is measured by using 4-point BD-Rate. The encoding time is measured on clusters with same CPU, and the decoding time is measured by decoding all sequences on a single PC.

For the high efficiency, the gains are -0.2% for RA HE, -0.6% for LB HE and -0.7% for LP HE. For the low complexity, the improvement are -0.1% for RA LC, -0.6% for LB LC and -0.6% for LP LC. In terms of complexity, we did not observe increase in encoding time or decoding time.  provides the BD rate and the running time of non-square transforms.

|  | Random Access HE |  |  | Random Access LC |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V |
| Class A | -0.2 | -0.5 | -0.1 | -0.1 | 0.1 | 0.2 |
| Class B | -0.2 | 0.1 | 0.1 | -0.2 | 0.3 | 0.2 |
| Class C | -0.2 | 0.3 | 0.2 | -0.1 | 0.4 | 0.4 |
| Class D | -0.3 | 0.2 | 0.1 | -0.1 | 0.3 | 0.4 |
| Class E |  |  |  |  |  |  |
| Overall | -0.2 | 0.0 | 0.1 | -0.1 | 0.3 | 0.3 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |
|  | Low delay B HE |  |  | Low delay B LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | 0.4 | 0.3 | -0.5 | 0.8 | 0.5 |
| Class C | -0.5 | 0.4 | 0.3 | -0.5 | 0.9 | 0.6 |
| Class D | -0.5 | 0.3 | 0.5 | -0.5 | 0.5 | 0.5 |
| Class E | -0.6 | 0.5 | -0.1 | -1.1 | 0.2 | 0.5 |
| Overall | -0.6 | 0.4 | 0.3 | -0.6 | 0.6 | 0.5 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  | Low delay P HE |  |  | Low delay P LC |  |  |
|  | Y | U | V | Y | U | V |
| Class A |  |  |  |  |  |  |
| Class B | -0.6 | 0.1 | 0.0 | -0.5 | 1.0 | 0.9 |
| Class C | -0.6 | 0.5 | 0.2 | -0.5 | 0.9 | 0.9 |
| Class D | -0.7 | 0.4 | 0.0 | -0.5 | 0.5 | 0.6 |
| Class E | -0.9 | 0.1 | 0.1 | -1.3 | 0.7 | 0.6 |
| Overall | -0.7 | 0.3 | 0.1 | -0.6 | 0.8 | 0.8 |
| Enc Time[%] | 99% |  |  | 102% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Table 1 BD rate reduction of non-square transforms

# Conclusion

In this contribution, non-square transforms are introduced for 2NxN and Nx2N PU. It is observed the additional choice of non-square transform achieves bit rate reduction of 0.2% for RAHE, 0.6% for LB HE, 0.7% for LP HE, 0.1% for RALC, 0.6% for LB LC and 0.6% for LP LC. No increase in encoding time or decoding time is observed.

# Draft Text

The modifications to Spec are highlighted in yellow.

| transform_tree( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| if( entropy_coding_mode_flag && trafoDepth = = 0 && IntraSplitFlag = = 0) { |  |
| if( PredMode != MODE_INTRA ) |  |
| no_residual_data_flag | u(1) ¦ ae(v) |
| residualDataPresentFlag = !no_residual_data_flag |  |
| } else { |  |
| residualDataPresentFlag = TRUE |  |
| } |  |
| if( residualDataPresentFlag) { |  |
| intraSplitFlag = ( IntraSplitFlag && trafoDepth = = 0 ? 1 : 0 ) |  |
| maxDepth = ( PredMode = = MODE_INTRA ? max_transform_hierarchy_depth_intra + IntraSplitFlag : max_transform_hierarchy_depth_inter ) |  |
| if( !entropy_coding_mode_flag ) { |  |
| xB = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yB = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| if( trafoDepth != 0 && !cbf_luma[ xB ][ yB ][ trafoDepth − 1 ] ) { |  |
| if( cbf_cb[ xB ][ yB ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cb[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if( cbf_cr[ xB ][ yB ][ trafoDepth − 1 ] && log2TrafoSize > Log2MinTrafoSize ) |  |
| cbf_cr[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| if(log2TrafoSize <= Log2MaxTrafoSize && !intraSplitFlag && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth ) |  |
| split_transform_flag[ x0 ][ y0 ] [ trafoDepth ] | u(1) |
| } else |  |
| cbp_and_split_transform | vlc(n,v) |
| if(flagPattern == 16 && (Luma¦ ¦Luma_2 ) && Split == 0) |  |
| nonsquare_transform_flag |  |
| } |  |
| if( log2TrafoSize <= Log2MaxTrafoSize && log2TrafoSize > Log2MinTrafoSize && trafoDepth < maxDepth && !intraSplitFlag && entropy_coding_mode_flag ) |  |
| split_transform_flag [ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| If (trafoDepth == 0 && split_transform_flag[ x0 ][ y0 ][ trafoDepth ] == 0 |  |
| && (PartMode == PART_Nx2N ¦¦ PartMode == PART_2NxN ) ) |  |
| nonsquare_transform_flag | u(1) ¦ ae(v) |
| if( PredMode != MODE_INTRA && log2TrafoSize <= Log2MaxTrafoSize && entropy_coding_mode_flag ) { |  |
| firstChromaCbf = ( log2TrafoSize = = Log2MaxTrafoSize ¦ ¦ trafoDepth = = 0 ? 1 : 0 ) |  |
| if( firstChromaCbf ¦ ¦ log2TrafoSize > Log2MinTrafoSize ) { |  |
| xBase = x0 − ( x0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| yBase = y0 − ( y0 & ( ( 1 << log2TrafoSize ) − 1 ) ) |  |
| if( firstChromaCbf ¦ ¦ cbf_cb[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| if( firstChromaCbf ¦ ¦ cbf_cr[ xBase ][ yBase ][ trafoDepth − 1 ] ) |  |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| if( split_transform_flag[ x0 ][ y0 ][ trafoDepth ] ) { |  |
| x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 ) |  |
| transform_tree( x0, y0, log2TrafoSize − 1, trafoDepth + 1, 0 ) |  |
| transform_tree( x1, y0, log2TrafoSize − 1, trafoDepth + 1, 1 ) |  |
| transform_tree( x0, y1, log2TrafoSize − 1, trafoDepth + 1, 2 ) |  |
| transform_tree( x1, y1, log2TrafoSize − 1, trafoDepth + 1, 3 ) |  |
| } else if( entropy_coding_mode_flag ){ |  |
| if( PredMode = = MODE_INTRA ¦ ¦ trafoDepth != 0 ¦ ¦ cbf_cb[ x0 ][ y0 ][ trafoDepth ] ¦ ¦ cbf_cr[ x0 ][ y0 ][ trafoDepth ] ) |  |
| cbf_luma[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| If (nonsquare_transform_flag) |  |
| cbf_luma_second[ x_2 ][ y_2][ trafoDepth ] | u(1) ¦ ae(v) |
| if( PredMode = = MODE_INTRA ) |  |
| if( log2TrafoSize > Log2MinTrafoSize ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth ] | u(1) ¦ ae(v) |
| } else if( blkIdx = = 0 ) { |  |
| cbf_cb[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| cbf_cr[ x0 ][ y0 ][ trafoDepth − 1 ] | u(1) ¦ ae(v) |
| } |  |
| } |  |
| } |  |
| } |  |

nonsquare_transform_flag specifies whether non-square transform is applied to a coding unit (at RQT root level). nonsquare_transform_flag equal to 0,  2Nx2N transform is applied to the whole coding unit.  nonsquare_transform_flag equal to 1, 2 2NxN (or Nx2N depending on CU partition) are applied to the coding unit.

cbf_luma_second[ x0 ][ y0 ][ trafoDepth ] equal to 1 specifies that the second 2NxN or Nx2N luma transform block contains one or more transform coefficient levels not equal to 0. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered transform block relative to the top-left luma sample of the picture. The array index trafoDepth specifies the current subdivision level of a coding unit into blocks for the purpose of transform coding, which is always 0.

cbp_and_split_transform contains a number of coded block flags and/or a split_transform_flag. Depending on a 4-bit value flagPattern defined as follows, the value of cbp_and_split_transform is defined according to Table 7-2.

codeLuma = trafoDepth  = =  0  | |  cbf_luma[ xB ][ yB ][ trafoDepth − 1 ]
codeCb = 		trafoDepth  = =  0  | | (cbf_cb[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize )
codeCr = 		trafoDepth  = =  0  | | (cbf_cr[ xB ][ yB ][ trafoDepth − 1 ]  &&  
				log2TrafoSize  >  Log2MinTrafoSize ) 	(7-1)
codeSplitTrans  =  log2TrafoSize  <=  Log2MaxTrafoSize  &&  
					log2TrafoSize  >  Log2MinTrafoSize  &&  
					trafoDepth  <  maxDepth  &&  
					!intraSplitFlag

flagPattern = (codeLuma << 3) + (codeCb << 2) + (codeCr << 1) + codeSplitTrans

The values of codeLuma, codeCb, codeCr and codeSplitTrans specify respectivley if an associated flag of cbf_luma, cbf_cb, cbf_cr, and split_transform_flag is included in the syntax element cbp_and_split_transform.

In Table 7-2, Luma, Cb, Cr and Split denote the values of cbf_luma[ x0 ][ y0 ][ trafoDepth ], cbf_cb[ x0 ][ y0 ][ trafoDepth ], cbf_cr[ x0 ][ y0 ][ trafoDepth ] and split_transform_flag[ x0 ][ y0 ][ trafoDepth ] respectively. Luma1, Luma2 and Luma3 denote respectively the values of cbf_luma[ x1 ][ y0 ][ trafoDepth ], cbf_luma[ x0 ][ y1 ][ trafoDepth ] and cbf_luma[ x1 ][ y1 ][ trafoDepth ]. The values of x1 and y1 are derived as

x1 = x0 + ( ( 1 << log2TrafoSize ) >> 1 )		(7-2)

y1 = y0 + ( ( 1 << log2TrafoSize ) >> 1 )		(7-3)

Luma_2 denotes the value of cbf_luma_second[ x_2 ][ y_2][ trafoDepth ].

If PartMode == PART_Nx2N,

x_2 = x1

y_2 = y0

If PartMode == PART_2NxN,

x_2 = x0

y_2 = y1

If (Luma&&Luma_2) == 0 and Split ==0 and nonsquare_transform_flag == 1, one extra bit is transmitted to indicate which one of Luma and Luma_2 is 1

Table 7-2 – Specification of cbp_and_split_transform

| - flagPattern | - cbp_and_split_transform |
| --- | --- |
| - 8 | - (Luma<<3) + (Luma1<<2) + (Luma2<<1) + Luma3 - 1 |
| - 9 | - (Luma<<1) + Split |
| - 10 | - (Luma<<1) + Cr |
| - 11 | - (Luma<<2) + (Cr<<1) + Split |
| - 12 | - (Luma<<1) + Cb |
| - 13 | - (Luma<<2) + (Cb<<1) + Split |
| - 14 | - (Luma<<2) + (Cb<<1) + Cr |
| - 15 | - (Luma<<2) + ((Cb¦Cr)<<1) + Split |
| - 16 | - (Luma&&Luma_2<<2) + ((Cb¦Cr)<<1) + Split |

### Scaling process for transform coefficients

Inputs of this process are:

–	variable TWidth and THeight specifying the width and height of the current transform unit,

–	a TWidth x THeight array c of transform coefficients with elements cij,

–	a variable cIdx specifying the chroma component of the current block,

–	a variable qP specifying the quantization parameter.

Output of this process is scaled transform coefficients as a TWidth x THeight array of d with elements dij.

The variable trafoPrecisionExt is derived as follows:

–	If TWidth is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 2.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 2.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

The scaled transform coefficient array dij is derived as follows.

dij = ( cij * LevelScale(nS)x(nS)[ qP%6 ]) << ( qP/6 + trafoPrecisionExt ), with i, j = 0..nS-1	(8-4)

LevelScale = {40,45,51,57,64,72} if TWidth == THeight

LevelScale = { 28, 32, 36, 40, 45, 51 }if TWidth != THeight

### Transformation process for scaled transform coefficients

Inputs of this process are:

–	variable TWidth and THeight specifying the width and height of the current transform unit,

–	a TWidth x THeight array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

Output of this process is residual samples as a TWidth x THeight array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nS is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set equal to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

[Ed.: (WJ) wait decision on transform precision extension]

[Ed: (WJ) derivation of trafoPrecisionExt is duplicated to that of the scaling process. Better to remove one of two.]

Depending on PredMode and IntraPredMode, the following applies:

–	If PredMode is equal to MODE_INTRA, nS is equal to 4, and cIdx is equal to 0, the variables horizTrType and vertTrType are specified as Table 8-3 with IntraPredMode as input. [Ed. (WJ): DST is applied only for luma 4x4 block]

–	Otherwise, the variables horizTrType and vertTrType are set equal to 0.

Table 8-3 – Specification of horizTrType and vertTrType

| IntraPredMode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 0 | 1 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 |

| IntraPredMode | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| horizTrType | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
| vertTrType | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i = 0. TWidth-1, j=0.. THeight-1) is transformed to eij (i = 0. TWidth-1, j=0.. THeight-1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found to Error: Reference source not found according to the size of the transform unit TWidth and  THeight), with the TWidth x THeight array d and the transform type variable horizTrType as the inputs and the output is the TWidth x THeight array e.

- Each (vertical) column of the resulting matrix eij(i = 0. TWidth-1, j=0.. THeight-1) is transformed to fij (i = 0. TWidth-1, j=0.. THeight-1) by invoking the one-dimensional transformation process as specified in subclause Error: Reference source not found to Error: Reference source not found according to the size of the transform unit TWidth x THeight, with the TWidth x THeight array e and the transform type variable vertTrType as the inputs and the output is the TWidth x THeight array f.

- If nS is equal to 4 or 8, the residual sample value rij is derived by

rij = ( fij + 25+trafoPrecisionExt ) >> ( 6 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-5)

- Otherwise (nS is equal to 16 or 32), the residual sample value rij is derived by

rij = ( fij + 29+trafoPrecisionExt ) >> ( 10 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-6)

# References

[1]F.Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

Qualcomm may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).